3GPP TSG-RAN WG2 Meeting #131												R2-2505657

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.6.2

Source: 	Sony

Title:	LTM CG Resource consumption for the target cells

Document for:	Discussion and Decision

# Introduction

In RAN#102, a new WI of mobility enhancements for NR has been agreed which contains the following bullet point [1]:

- Specify support for inter-CU Layer 2 Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured.

  - As secondary priority, support the case when NR-DC is configured, and CU is acting as SN and MCG is unchanged.

  - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released.

    - Note: The case that LTM is configured in both MCG and SCG is excluded

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling.

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support.

In RAN2#126, the following agreements were reached:

Agreements on inter-CU LTM:

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In RAN2#127, the following agreements were reached:

Agreements on inter-CU LTM

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

In this contribution, we discuss how to reduce the configured grant resource consumption for the candidate target cells for both intra-CU and inter-CU scenarios.

## 2. LTM CG Resource consumption for the target cells

In Rel-19 LTM, configured grant resources for the target candidate cells are allocated in the RRC reconfiguration message. As the resources are pre-allocated for multiple candidate cells even if they are not used, there is an issue of resource consumption (i.e., allocated but not used at the end). Hence, how to minimize the resource consumption is an issue to be solved for both intra-CU and inter-CU deployment scenarios.

In the current LTM procedure as shown on Figure 1 below, the CG resources for the target cell are allocated in the RRC reconfiguration message (in Step 8). In step 14, a UE receives the LTM Cell Switch Command to the target cell, and in step 16, the UE applies CG resources to transmit the first message (e.g., data) to indicate its arrival at the target cell.

Figure 1. Rel-19 LTM procedure (Figure 9.2.3.5.2-1. Signalling procedure for LTM) [2]

In order to minimize the CG resource consumption, the CG resources that are allocated in step 8 can be initially deactivated by default, and then when the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM Cell Switch Command to the UE for a target cell (for example target cell index 1), the CG resources are activated only for that target cell (index 1). This means that activation message should be sent from the serving cell to the target cell only.

Proposal 1: RAN2 to discuss how minimize the configured grant resource consumption for the candidate target cells.

Proposal 2: CG resources of the target cell should be activated only after the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

As the activation message is sent from the serving cell to the target cell, some signalling impact is expected in RAN3, but we think implicit signalling would be enough by using existing message of CELL SWITCH NOTIFICATION in step 15 of Figure 1, that is: when target cell receives this message, it should activate the CG resources of that target cell only.

Proposal 3: Target cell should activate CG resources after receiving CELL SWITCH NOTIFICATION message from the source cell.

Based on proposal 3, it should be noted that in case of Inter-CU scenario, there will be noticeable delay for the target cell to activate the CG resources, in that case, an implementation can handle this issue whereby the serving cell slightly delays sending LTM CSC MAC CE to the UE as latency is better than interruption [3]. This may need slight changes to the current order of step 14 and step 15. We think step 15 should come earlier than step 14 by swapping them, or the order could be up to network implementation and a note can be added in the spec.

Proposal 4: Add a note in the spec that it is up to network implementation the order of step 14 and 15 where step 15 can be earlier than step 14.

## Conclusions

In this contribution, we have discussed how to reduce the configured grant resource consumption for the candidate target cells for both intra-CU and inter-CU scenarios. We have the following proposals:

Proposal 1: RAN2 to discuss how minimize the configured grant resource consumption for the candidate target cells.

Proposal 2: CG resources of the target cell should be activated only after the serving cell receives L1 measurement report from the UE and the serving cell makes the decision to issue the LTM command to the UE for the target cell. The activation message should be sent from the serving cell to the target cell only.

Proposal 3: Target cell should activate CG resources after receiving CELL SWITCH NOTIFICATION message from the source cell.

Proposal 4: Add a note in the spec that it is up to network implementation the order of step 14 and 15 where step 15 can be earlier than step 14.

## References

[1] RP-234036, “New WID: NR mobility enhancements Phase 4,” RAN#102.

[2] R2-2503732, “Introduction of NR mobility enhancements Phase 4 in TS 38.300”, draftCR	Rel-19	38.300.

[3] R2- 2408259, “Further discussion on Inter-CU LTM”, 3GPP TSG-RAN WG2 Meeting #127bis, MediaTek.