3GPP TSG-RAN WG2 Meeting #131				   		        R2-2505675

Bengaluru, India, 25 – 29 August, 2025

Title: 	Remaining issues of NW-sided data collection

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3

Document for:	Discussion/Decision

# Introduction

In [1], RRC open issues have been discussed, and some of them still need more RAN2 discussions. In addition, [2] is for discussion of UE capability related issues, and some of them are controversial.

This paper analyzes the network-sided data collection topic further, with a focus on the FFS points mentioned above.

# Discussion

## Time related content of logged data [RRC-21]

In the last meeting, RAN2 agreed that an indication of the ‘gap’ is needed.

1 Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

In our understanding the gap is just a by-product of the event-based logging, not something that we want to use to train models, so the knowledge about the amount of gap is not indispensable for the model training purposes. On the other hand, some signalling needs to be added to the logged results anyway for the network to know that the gap exists, so it may be straightforward to signal the amount of gap actually. This could be achieved by adding a time stamp to a first sample of each “block” of samples.

Proposal 1: (RRC-21) When the gap occurs in the logged measurements, the length of the gap should be indicated to the network by adding a time stamp to a first sample of each “block” of samples.

## UAI indications [RRC-19]

For the open issue RRC-19: Reporting assistance information related to logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated?

There is also an FFS on the encoding of data available in the last meeting.

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Based on the agreements in the last meeting, the separate indicators are considered, i.e., data available indication for full buffer, data available indication for buffer threshold, and low power indication. As for how the NW controls the indication reporting, we believe there is no need for a too granular configuration. In our view full buffer indication can be assumed to be enabled by default when the UE is configured with loggedDataCollectionAssistanceConfig. For the UE to be able to use data threshold, this threshold needs to be configured by the network, so its presence in the configuration is used to determine whether data availability based on threshold is enabled. Since there is no prohibit timer for this UAI information while the low power state indication is up to UE implementation, this should also be separately configurable, so that the network can disable the feature for the misbehaving UEs.

Proposal 2: (RRC-19) The indications reported by the UE as part of UAI for data collection purpose are controlled by the network in the following way:

- Full buffer indication is enabled by default when the UE is configured with loggedDataCollectionAssistanceConfig

- Data threshold is enabled by including data threshold in the configuration

- Low power state indication is configurable with a dedicated parameter to enable it

## Granularity of data request/report [RRC-39]

Open issue RRC-39 was initially discussing the naming of the request/report variables and parameters, in particular whether they should be use case specific. However, there were many comments raised in both open issues document and during RRC review about the granularity of the collected data request/report. Eventually, the following proposals were put forwards by the rapporteur:

- (RRC-39) The UE stores logged data for BM in a variable specific to L1 CSI related measurements. (RRC-39) RAN2 to discuss whether the name of the fields/IEs to request logged data from the UE and to report logged data or the availability of logged data to the NW are use case specific.

Firstly, we agree that the request and report should refer to a specific use case.

Proposal 3: The names of the fields/IEs to request logged data from the UE and to report logged data or the availability of logged data to the NW should be use case specific.

However, we do not think such granularity is sufficient. It was agreed previously that the network can provide the UE with multiple sets of configurations, each corresponding to different scenario or set of resources, even for a single use case. These configurations or scenarios may have different data collection requirements. After the UE performs data collection based on the provided configuration and logs the collected data locally, when the stored data volume meets the specified requirements, the UE sends a data availability indication. Since the speed of data collection may vary for each configuration or scenario, we believe that the data availability indication triggering threshold should be configured and indicated separately for each data collection configuration. Furthermore, after receiving the data availability indication from the UE, the network should be able request the data collected by the UE for the specific configuration or scenario.

Proposal 4: (RRC-39) Data availability triggering threshold should be configurable per data collection configuration.

Proposal 5: (RRC-39) The UE should include the corresponding configuration identifier (i.e. CSI-LoggedMeasurementConfigId) in the data available indication to allow the network to request specific data collected by the UE.

Proposal 6: (RRC-39) The network should be able to request the UE to send collected data for a specific data collection configuration, i.e.  UEInformationRequest for NW-side data collection should include one or more CSI-LoggedMeasurementConfigId(s).

## Low power state indication [RRC-29]

Open issue RRC-29 on the other hand discusses whether data availability indication should be sent when the UE sends low power state indication. In our understanding, when the UE indicates low power state, it may have collected some data already, but this fact might not have been indicated to the network before in case neither the data threshold nor full buffer has been reached. In this situation, data availability indication can also be included with another cause (current CR captures only ‘full buffer’ and ‘data threshold’ causes). Without this addition, the network will release the configuration from the UE and the data collected so far will be lost.

Proposal 7: (RRC-29) When the UE sends low power state indication to the network and it has already collected some data, it should also include data availability indication, e.g. with ‘low power’ or ‘other’ cause.

## Configuration details [RRC-25, RRC-30]

Regarding whether dynamic activation/deactivation of data collection should be supported. Data collection for model training is normally a continuous process and it is unclear what the purpose would be of activating and deactivating it dynamically. If the network no longer requires more data from the UE for a certain data collection configuration, the network may simply de-configure it using RRC signaling. Another potential reason to have dynamic activation/deactivation would be to allow the network to rely on semi-persistent CSI resources for data collection. Related to this, RAN2 made the following agreement during RAN2#128:

2. Measurements on aperiodic CSI resources are not reported for NW sided data collection.

Similarly, we do not see a reason to rely on semi-persistent resources and believe periodic CSI resources fit data collection purpose best. Based on the above consideration, we propose the following:

Proposal 8: (RRC-25) (RRC-30) Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

## SRB for data reporting [RRC-26]

In the last meeting, it was agreed that new SRB can be configured for NW-side data collection.

New SRB can be configured for NW-side data collection (with lower priority)

In the open issue list, there is an issue about whether legacy logged data can be carried over a new SRB in case the UE needs to report both legacy and AIML data to the network:

Open issue RRC-26: Multiplexing legacy logged data and AIML logged data in new SRB Issue description: RAN2#129bis agreed that 2. New SRB can be configured for NW-side data collection (with lower priority). Given the agreement on the new SRB for the transmission of the UEInformationResponse, Rapporteur’s understanding is that the new SRB may be used also for the transmission of the legacy SON/MDT reports (e.g. logged MDT measurements, RLF-Report, RA-reports, successful HO reports, etc.), whenever the UEInformationResponse carries both a legacy SON/MDT report and AIML logged data. However, this would impact how legacy SON/MDT reporting is performed which can only be at the moment on SRB1 or SRB2.

We believe it can be up to UE implementation whether to put both legacy SON/MDT and new data collection reports in the same response message or use separate ones for this purpose. If both the legacy SON/MDT report and AIML logged data are included in a single UEInformationResponse, the UE should select a new SRB when AIML logged data is present.

Proposal 9: (RRC-26) If both the legacy SON/MDT report and AIML logged data are included in a single UEInformationResponse, the UE should select a new SRB to transmit such message.

However, even if multiplexing is not supported, this can easily be captured in the procedure (or even using a NOTE) by reusing the current message, there is no need to introduce a new one as proposed in the latest version of the running RRC CR currently. We can simply capture that if collected data is to be included in the message, then no other information should be included and then this message is forwarded on SRBx.

Proposal 10: (RRC-26) In case RAN2 decides not to support multiplexing of the legacy SON/MDT report and AIML logged data, such limitation should be captured as part of the UE Information procedure description, not by introducing a new message dedicated to AIML data reporting.

## Collected data handling during HO/CHO/LTM [RRC-52]

So far RAN2 focused only on legacy handover procedure, but it may also happen that the UE executed CHO or LTM HO while configured with data collection. Unlike legacy HO scenario, during CHO/LTM, there is no possibility to send an indication to the UE to retain or discard logged data as the HO execution is performed by the UE automatically (for CHO) or via a switching command sent via MAC CE (LTM). On the other hand, the source gNB is aware of which cells can be subject to CHO/LTM, so the data retain/discard indication can be pre-configured together with CHO/LTM configuration for each candidate cell.

Proposal 11: (RRC-18) For CHO/LTM, a collected data retain/discard indication can be pre-configured by the gNB for each CHO/LTM candidate cell, e.g. in CondReconfigToAddMod and LTM-Candidate IEs.

## Use case applicability of NW-side data collection [RRC-34]

Regarding RRC-34 open issue on whether enhancements for NW-side data collection are per use case or common for all AI-related use cases, we do not think it needs to be extended to other use cases at the moment. For air interface AI, other use cases either belong to the UE-side model or a two-sided model, which means NW-side data collection does not need to be considered for them. As for the AIML for mobility, it is still under study and whether how to use the existing framework can be discussed in Rel-20 during the AIML for mobility WI phase. It is hard to consider this aspect at the moment because many aspects remain undetermined for AIML for mobility.

Proposal 12: (RRC-34) RAN2 should focus on NW-side data collection for beam management use case in Rel-19 and how to extend the framework to other use cases (e.g. AIML for mobility) can be discussed in Rel-20 when these use cases will be specified.

## UE capability related issues [UECap-1, UECap-2]

After RAN2#130 meeting, an e-mail discussion took place on the UE capabilities related to AIML for air interface support. After the discussion, some open issues related to NW-side data collection still remain to be solved, as summarized in [4], namely:

- Open issue UECap-1: whether UE can support other memory sizes and indicate to network via optional capability signaling

- Open issue UECap-2: How to define UE logged measurement of data collection for NW-side model

Additional UE memory sizes

As can be seen in [4], companies who argue against specifying a possibility for UE to signal additional memory sizes for NW-side data collection mention that this can be left up to UE implementation. The companies further give an example of logged MDT where the UE can allocate additional memory but does not signal this to the network. However, it should be noted that the situation in logged MDT is completely different as the UE collects the data in RRC IDLE/INACTIVE states and indicates data availability when establishing a connection with the network. For AIML, the data is collected in RRC CONNECTED state and the UE sends data availability indication either upon reaching full buffer or based on the threshold. The goal of introducing a data threshold was to avoid a situation where the UE stops data collection due to reaching full buffer as data collection continuity is important for the quality of AIML model training. Hence, the network will set the value of the threshold based on the memory size supported by the UE. Due to this, unlike in logged MDT, for the AIML data collection, the additional memory supported by the UE should be known to the network, otherwise the threshold will have to be always set by the network to a value lower than 64 kBytes, leading to more signalling overhead, even in case the UE is able to allocate more memory for data collection.

Observation 1: Unlike in logged MDT, in NW side data collection for AIML, the network can set data volume threshold in order to fetch the data before the UE runs out of memory. This way data collection continuity can be ensured.

Observation 2: In case additional memory size is not known to the network, data volume threshold will have to be always set to a value lower than 64 kBytes, leading to more signalling overhead, even in case the UE is able to allocate more memory for data collection.

Based on these observations we propose the following:

Proposal 13: (UECap-1) As an optional capability, the UE should be allowed to signal that it supports memory sizes larger than the minimum memory required for NW-side data collection, e.g. 128 kB, 256 kB, 512 kB.

Exact definition of NW-side data collection and its components

As a result of the e-mail discussion, the following proposal was put forward:

Proposal 3: Introduce an optional UE capability to indicate supporting logged measurements of data collection for NW-side model, which includes the minimum AS layer memory size. FFS whether the following components are part of this capability or separate capabilities, and how: - periodic logging - event-based logging - measurement quantities and resources (Set A/B, maximum resources of Set A/B, etc) - provide assistance information (e.g., data is available, reason for trigger, low power indication) for NW-side data collection

The companies could not reach consensus on which components should be part of the generic “NW-side data collection capability” and which should be signaled in addition to that, as optional capabilities. Here are our views on each these components.

| AS layer minimum memory | It seems straightforward and agreeable by the companies that this needs to be part of the generic capability. |
| --- | --- |
| Periodic vs event-based logging | At least one of periodic or event-based logging: it is obvious that the UE must support at least one of these. Most companies indicated that periodic logging should be the baseline and we agree with that view. This is because event-based logging was aimed at overhead optimization in case the network only requires data from a certain cell area to complement the data gathered earlier. However, beam management needs to be applied throughout the cell so it is natural to first collect the data without the need of pre-filtering at the UE side. Furthermore, event-based logging also uses periodic logging as the baseline (once the event is met) so it is hard to imagine the UE which supports event-based logging but not periodic logging. Event-based logging also brings additional complexity to UE implementation, so it may delay an adoption of the whole feature, if being mandatory. Observation 3: Event-based logging is only an optimization for the case where already collected data needs to be complemented with the data from a certain cell area. Observation 4: Event-based logging also uses periodic logging as the baseline (once the event is met) so it very unlikely for a UE to support event-based logging but not periodic logging. Observation 5: Event-based logging brings additional complexity to UE implementation on top of periodic logging. |
| UE assistance information | There are three different components of UAI for NW side data collection: - full buffer indication - indication based on data threshold - low power state indication When it comes to data availability indication due to full buffer and data volume threshold, we think they both should be supported as a baseline data collection capability. Without these indications, the network would have to blindly try to fetch the data which is inefficient from the signalling overhead perspective. Observation 6: Without data availability indications, the network would have to blindly try to fetch the data which is inefficient from the signalling overhead perspective. When it comes to low power state indication, it could be a separate capability, but we do not think it is needed. It should be noted that according to RAN2 agreements and as captured in the RRC running CR, how to determine the low power state is up to UE implementation. Even if the network configures the UE to report low power state, the UE is not required to do this under any specified conditions, so this feature is not testable anyway. Observation 7: Low power state determination is fully up to UE implementation and is not a testable feature. Based on this, even if the network enables low power state indication at the UE, the UE is not required to send an indication, if it does not support it. Hence, UAI can be treated as whole and be part of the generic NW-side data collection capability. |
| Measurement quantities and resources (Set A/B, maximum resources of Set A/B, etc) | This part is related to the following comment made during the UE capabilities e-mail discussion: We find the proposal beneficial as the aim is to develop a general framework allowing for the NW to collect data for various AIML use cases, e.g. in Rel-20 it is likely to be reused/adapted for data collection related to UE mobility scenarios. Observation 8: The NW-side data collection framework developed in Rel-19 is intended to be reused for other use cases in future, e.g. for mobility in Rel-20. What is more, a similar approach was taken for QoE where capabilities related to general behavior were decoupled from capabilities related to specific applications, e.g. qoe-IdleInactiveMeasReport-r18 Indicates whether the UE supports NR QoE Measurement Collection in RRC_IDLE and RRC_INACTIVE states for the services indicated with qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.; UE; No; No; No. qoe-MTSI-MeasReport-r17 Indicates whether the UE supports NR QoE Measurement Collection for MTSI services, see TS 26.114 [30].; UE; No; No; No. One thing that needs to be decided though is the granularity of the capability, e.g. whether it is use case specific or measurement specific. At the moment, we propose to focus on beam management use case and specify a single capability for this. |

Based on the analysis and observations captured in the table above, we have the following proposals:

Proposal 14: (UECap-2) Periodic logging should be a part of the generic NW-side data collection capability.

Proposal 15: (UECap-2) Event-based logging should be a separate optional capability.

Proposal 16: (UECap-2) UAI related to data collection is part of the generic NW-side data collection capability

Proposal 17: (UECap-2) The UE capabilities related to an overall NW-side data collection procedure (e.g. periodic logging, event-based logging) should be decoupled and signaled separately from use case specific capabilities (e.g. data collection for beam management, data collection for mobility).

# Conclusion

Based on the discussion in the Tdoc, we provide the following observations and proposals:

Time related content of logged data [RRC-21]

Proposal 1: (RRC-21) When the gap occurs in the logged measurements, the length of the gap should be indicated to the network by adding a time stamp to a first sample of each “block” of samples.

UAI indications [RRC-19]

Proposal 2: (RRC-19) The indications reported by the UE as part of UAI for data collection purpose are controlled by the network in the following way:

- Full buffer indication is enabled by default when the UE is configured with loggedDataCollectionAssistanceConfig

- Data threshold is enabled by including data threshold in the configuration

- Low power state indication is configurable with a dedicated parameter to enable it

Granularity of data request/report [RRC-39]

Proposal 3: The names of the fields/IEs to request logged data from the UE and to report logged data or the availability of logged data to the NW should be use case specific.

Proposal 4: (RRC-39) Data availability triggering threshold should be configurable per data collection configuration.

Proposal 5: (RRC-39) The UE should include the corresponding configuration identifier (i.e. CSI-LoggedMeasurementConfigId) in the data available indication to allow the network to request specific data collected by the UE.

Proposal 6: (RRC-39) The network should be able to request the UE to send collected data for a specific data collection configuration, i.e.  UEInformationRequest for NW-side data collection should include one or more CSI-LoggedMeasurementConfigId(s).

Low power state indication [RRC-29]

Proposal 7: (RRC-29) When the UE sends low power state indication to the network and it has already collected some data, it should also include data availability indication, e.g. with ‘low power’ or ‘other’ cause.

Configuration details [RRC-25, RRC-30]

Proposal 8: (RRC-25) (RRC-30) Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

SRB for data reporting [RRC-26]

Proposal 9: (RRC-26) If both the legacy SON/MDT report and AIML logged data are included in a single UEInformationResponse, the UE should select a new SRB to transmit such message.

Proposal 10: (RRC-26) In case RAN2 decides not to support multiplexing of the legacy SON/MDT report and AIML logged data, such limitation should be captured as part of the UE Information procedure description, not by introducing a new message dedicated to AIML data reporting.

Collected data handling during HO/CHO/LTM [RRC-52]

Proposal 11: (RRC-18) For CHO/LTM, a collected data retain/discard indication can be pre-configured by the gNB for each CHO/LTM candidate cell, e.g. in CondReconfigToAddMod and LTM-Candidate IEs.

Use case applicability of NW-side data collection [RRC-34]

Proposal 12: (RRC-34) RAN2 should focus on NW-side data collection for beam management use case in Rel-19 and how to extend the framework to other use cases (e.g. AIML for mobility) can be discussed in Rel-20 when these use cases will be specified.

UE capability open issues (UECap-1, UECap-2)

Observation 1: Unlike in logged MDT, in NW side data collection for AIML, the network can set data volume threshold in order to fetch the data before the UE runs out of memory. This way data collection continuity can be ensured.

Observation 2: In case additional memory size is not known to the network, data volume threshold will have to be always set to a value lower than 64 kBytes, leading to more signalling overhead, even in case the UE is able to allocate more memory for data collection.

Observation 3: Event-based logging is only an optimization for the case where already collected data needs to be complemented with the data from a certain cell area.

Observation 4: Event-based logging also uses periodic logging as the baseline (once the event is met) so it very unlikely for a UE to support event-based logging but not periodic logging.

Observation 5: Event-based logging brings additional complexity to UE implementation on top of periodic logging.

Observation 6: Without data availability indications, the network would have to blindly try to fetch the data which is inefficient from the signalling overhead perspective.

Observation 7: Low power state determination is fully up to UE implementation and is not a testable feature.

Observation 8: The NW-side data collection framework developed in Rel-19 is intended to be reused for other use cases in future, e.g. for mobility in Rel-20.

Proposal 13: (UECap-1) As an optional capability, the UE should be allowed to signal that it supports memory sizes larger than the minimum memory required for NW-side data collection, e.g. 128 kB, 256 kB, 512 kB.

Proposal 14: (UECap-2) Periodic logging should be a part of the generic NW-side data collection capability.

Proposal 15: (UECap-2) Event-based logging should be a separate optional capability.

Proposal 16: (UECap-2) UAI related to data collection is part of the generic NW-side data collection capability

Proposal 17: (UECap-2) The UE capabilities related to an overall NW-side data collection procedure (e.g. periodic logging, event-based logging) should be decoupled and signaled separately from use case specific capabilities (e.g. data collection for beam management, data collection for mobility).

# References

- [1] [POST130][026][AI PHY] 38.331 CR (Ericsson)

- [2] R2-2505298 Report of [POST130][038][AI PHY] UE capabilities (Xiaomi), Xiaomi