3GPP TSG-RAN WG2 Meeting #131	R2-2505677

Bengaluru, India, Aug 25th - 29th, 2025

Agenda item:		8.7.5

Source:		Samsung

Title:			Discussion on open issues for RLC enhancements

Document for:	Discussion & Decision

# 1 Introduction

In this paper we discuss the following open issues for RLC enhancements:

[RLC-10] Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. [RLC-11] FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” [RLC-12] FFS how to handle PDCP SN gap report handling during UE mobility [RLC-13] Whether the delay-reporting or non-delay-reporting information is visible or not to RLC? [RLC-14] If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. [UE cap-04] Whether a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

# 2 Discussion

- [RLC-11] FFS how to handle the case “if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”

- Whether/If the UE needs to continue to run t-PollRetransmit when it already knows the status of all RLC SDUs (either positively/negatively acknowledged, or discarded) up to POLL_SN.

RAN2 agreement in #130 meeting:

Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

We understand window stalling is not an issue as the Rx would be running t-RxDiscard and upon its expiry, will trigger a status report. Moreover, the scenario is considering that there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there seems no situation for a window stalling.

At the same time, we think the ongoing polling procedure is not necessary when the indicated RLC SDU for discard has sequence number equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded. Therefore, if the t-PollRetransmit is running, it should stop and reset t-PollRetransmit. Otherwise, there would be an ambiguity for the polling procotol as there is no suitable SDU to continue polling operation.

[RLC-11] Proposal 1: When the indicated RLC SDU for discard has a sequence number equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded, stop and reset t-PollRetransmit, if running. (Adopt TP 1)

- [RLC-12] FFS how to handle PDCP SN gap report handling during UE mobility

Previosuly RAN2 discussed an issue raised by RAN3 LS. Issue concerns to the PDCP SN gap report during mobility. In this scenario, UE may discard some PDCP SDUs before handover and informs the source gNB through a PDCP SN gap report. Now, the source gNB during the handover operation forwards the buffered packets along with the SNs for the PDCP packets that are not received in the SN STATUS TRANSFER message to the target gNB. However, target gNB can not recover those PDCP SDUs through retransmissions of not received packets which are already discarded by the UE. Also target gNB is not aware about the discard of those PDCP SDUs and this will lead to excesive delay due t-reordering operations.

In May 2025, RAN3 has addressed this issue and introduced a solution (R3-253905) where the source gNB forwards the SN gap report to the target gNB. To implement this, new IEs are included in the SN STATUS TRANSFER message to indicate the packets discarded before handover.

[RLC-12] Proposal 2: RAN2 to decide if RAN2 solution is needed in context of Rel-19 AM RLC enhancements while RAN3 has already addressed the SN STATUS TRANSFER to carry the discarded SUDs via Discard Bitmap IE.

In case RAN2 solution is needed:

Some companies think that the issue can be solved by allowing a re-transmisison or a re-submission of the PDCP SN gap report after handover to the target gNB that was earlier sent to the source gNB during mobility. However, such approach breaks the basic tenet of radio protocols wherein a Control PDU is neither stored nor retransmitted.

Instead, we think it is possible to consider a new trigger for PDCP SN gap report after handover that allows the reporting of the discard information including the previous set of discarded PDCP SDUs. More precisely, for AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility (e.g. prior to PDCP entity re-establishment) but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity should trigger a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any. Further, a separate configuration “sn-GapReportMobility” can be used for R19 AM DRBs for this purpose, to distinguish from the legacy configuration.

[RLC-12] Proposal 3: If RAN2 solution is needed as in proposal 2, then RAN2 agree to the following approach in order to address PDCP SN gap reporting for AM DRBs during mobility (Adopt TP 2):

- For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity triggers a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any.

- A separate configuration “sn-GapReportMobility” is used for R19 AM DRBs for this purpose to distinguish from the legacy configuration.

- [RLC-13] Whether the delay-reporting or non-delay-reporting information is visible or not to RLC?

For the PDCP data PDU that has already been submitted to lower layers, delay reporting indication is provided from PDCP to RLC for the delay-reporting PDCP SDU. It is straightforward to extend this delay reporting indication to also consider non-delay-reporting PDCP SDU (subject to dsr-ReportNonDelayCriticalData configuration).

Given this indication, it seems unnecessary and redundant to have RLC also define Delay-reporting RLC SDU and Non-delay-reporting RLC SDU and evaluate the corresponding data for delay reporting RLC data volume. This also makes the procedural description complex and prone to implementation error. We prefer to keep it simple and described only in the PDCP spec.

[RLC-13] Proposal 4: RAN2 agree that delay-reporting or non-delay-reporting information is not visible to RLC.

- Delay reporting indication provided from PDCP to RLC is extended to also consider non-delay-reporting PDCP SDU (subject to dsr-ReportNonDelayCriticalData configuration).

- Delay reporting indication is utilized by the RLC for data volume calculation.

- [RLC-14] If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, whether the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers.

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

As described above, as per the legacy specification, Tx RLC entity does not discard the corresponding RLC SDU or RLC SDU segment(s) if the RLC SDU or the RLC SDU segment(s) thereof has been submitted to lower layers. With R19 AM RLC enhancements (stopReTxDiscardedSDU is configured), Tx AM RLC entity does not consider theses RLC SDU or segments for transmission or retransmission, while actual discard is on the basis of the (fake) positive ACK received. We see no discrepancy in the legacy specification and Rel-19 AM RLC behavior when stopReTxDiscardedSDU is configured.

[RLC-14] Proposal 5: If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or the RLC SDU segment(s) only on basis of (fake) positive ACK received. There is no spec impact.

- [UE capability-04] Whether a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

- [RLC-10] Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard.

In our view, the unnecessary retransmission avoidance is based on a combined solution of Tx stopping the retransmission of discarded RLC SDU and Rx determining the discard of the SDU and abandoning it. It is therefore quite straightforward that for realizing this solution the UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19. This is also relevant in terms of association for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard, as network can configure the UE together with stopReTxObsoleteSDU and DL t-RxDiscard.

[UE-cap-04] Proposal 6: RAN2 agree that a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

[RLC-11] Proposal 1: When the indicated RLC SDU for discard has a sequence number equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded, stop and reset t-PollRetransmit, if running. (Adopt TP 1)

[RLC-12] Proposal 2: RAN2 to decide if RAN2 solution is needed in context of Rel-19 AM RLC enhancements while RAN3 has already addressed the SN STATUS TRANSFER to carry the discarded SUDs via Discard Bitmap IE.

[RLC-12] Proposal 3: If RAN2 solution is needed as in proposal 2, then RAN2 agree to the following approach in order to address PDCP SN gap reporting for AM DRBs during mobility (Adopt TP 2):

- For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink, when a PDCP SN gap report was previously transmitted during mobility but its successful delivery has not been confirmed by lower layers, the transmitting PDCP entity triggers a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any.

- A separate configuration “sn-GapReportMobility” is used for R19 AM DRBs for this purpose to distinguish from the legacy configuration.

[RLC-13] Proposal 4: RAN2 agree that delay-reporting or non-delay-reporting information is not visible to RLC.

- Delay reporting indication provided from PDCP to RLC is extended to also consider non-delay-reporting PDCP SDU (subject to dsr-ReportNonDelayCriticalData configuration).

- Delay reporting indication is utilized by the RLC for data volume calculation.

[RLC-14] Proposal 5: If stopReTxDiscardedSDU is configured, when indicated from upper layer to discard a particular RLC SDU, the transmitting side of an AM RLC entity shall discard the corresponding RLC SDU or the RLC SDU segment(s) only on basis of (fake) positive ACK received. There is no spec impact.

[UE-cap-04] Proposal 6: RAN2 agree that a UE supporting txRLC-StopReTxDiscardedSDU-r19 shall also indicate support of rxRLC-Discard-r19.

# 4 Annex: Text Proposals

TP 1

TS 38.322 When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity shall: - if the indicated RLC SDU for discard has a sequence number = X equal to POLL_SN and all other RLC SDU(s) with sequence number < POLL_SN is already acknowledged or discarded: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit.

TP 2

TS 38.323 ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. For AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReportMobility in TS 38.331 [3]), when a PDCP SN gap report was previously transmitted during mobility but its succesful delivery has not been confirmed by lower layers, the transmitting PDCP entity shall trigger a PDCP SN gap report that includes the same set of previously discarded PDCP SDUs, apart from newly discarded PDCP SDUs, if any. If a PDCP SN gap report is triggered for these discarded PDCP SDU(s), the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with these discarded PDCP SDU(s); - if more than one PDCP SDU is discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first of these discarded PDCP SDU(s) up to and including the last of these discarded PDCP SDU(s), rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '1' for these discarded PDCP SDU(s); - setting in the discard bitmap field as '0' for all other PDCP SDU(s). - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.