3GPP TSG-RAN WG2 Meeting #131	R2-2505679

Bangalore, India, August. 25th – 29th, 2025

Agenda item:		8.2.2

Source:		Lenovo

Title:	Remaining Issues of paging for Ambient IoT

Document for:		Discussion and Decision

- Introduction

In RAN#107 meeting, the WID for Rel-19 Ambient IoT in NR [1] was revised and approved, the objectives related to RAN2 aspects of the work item are given as below:

RAN2 Scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included. - IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included.

This contribution aims to present our views on the remaining issues of paging functionality for Ambient IoT devices based on the above WID objectives, MAC running CR and open issue list.

- Discussion

2.1 Transaction ID

In RAN2#129 meeting [2], transaction ID is introduced to avoid duplicate responses for the same service request from device side, with following agreements have been reached:

- The ’transaction ID’ can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID. - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size.

Considering the motivation to introduce such transaction ID is to distinguish the different service requests from CN in AS layer, while the service request is associated to a correlation ID in core network. Then the baseline principle can be assumed that: different CN correlation IDs are associated to different transaction IDs. Regarding to how does the reader generate the transaction ID based on the CN correlation ID, for example, the reader may use the least significant bit (LSB) X bits of correlation ID as the transaction ID, and the details can be left to reader’s implementation.

Another FFS issue is about the transaction ID size, since the transaction ID is generated based on the correlation ID, the detailed information on the structure and length of the correlation ID are designed by SA2. From AS layer’s perspective, if the principle can be agreed that the different transaction IDs are associated with different service requests, we suggest 3-4 bits which may be a reasonable size to ensure the uniqueness and avoid wraparound issue as much as possible.

Proposal 1 (Issue 1-2): It is left to reader’s implementation to generate the transaction ID, the size of the transaction ID can be 3-4 bits.

2.2 Paging content for CFA

In last RAN2#130 meeting [3], it has been agreed that for CFA, as the baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasion are absent. FFS on the need of transaction ID. Since during online discussions, companies proposed that there may have the case that both CFA and CBRA procedure target to the same device may have the same transaction ID, if transaction ID in CFA paging is included, then when CFA is ongoing, there comes a CBRA paging including the same transaction ID, then device determines the transaction ID is the same and skips to response to it. Otherwise, device responds unnecessarily. In our view, based on previous RAN2 agreements on parallel paging, the reader will not initiate both CBRA and CFA procedure for the same service request in parallel, it is may be not a typical use case. Even this case exists, the procedure is workable based on the principle that the device always responds to the new paging from the reader. Although in some extent, the device may increase the response delay since the access delay for CBRA may be higher than CFA, but latency is not a critical issue for A-IoT device. Furthermore, due to different paging message content/format for CBRA and CFA paging, device anyway cannot have unified behaviors for those two paging, there is unnecessary to define the unified behaviors for the device always to check transaction ID in paging. Therefore, it is proposed that for CFRA paging, the field related to transaction ID is not needed.

Proposal 2 (Issue 1-5): For CFA paging, the field related to transaction ID is not needed.

2.3 Visibility of paging identifier in MAC layer

In RAN2#129 meeting [2], RAN2 has assumed that the paging identifier is transparent to the A-IoT MAC layer and carried by upper layer. However, some companies raised concern that Paging ID needs to be visible in MAC for filter/grouping design purpose. Then RAN2 has kept it open to further discuss if there is really a need for visibility on paging identifier in the MAC layer.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer.

In our views, we have not seen the necessity to make the paging ID visible in the MAC layer for the grouping purpose. The paging ID is generated by the core network e.g. AIoTF based on the device ID or other temporary ID defined in CN. It is not suitable to further process the id e.g. filter or re-group in AS layer, which may cause additional security problem. Additionally, during the A-IoT paging email discussion [POST129][035][AIoT], some companies further proposed that the paging ID needs to be visible in the reader MAC layer since the reader needs to know the association between paging ID and AS ID for inventory and command procedure. We understood that the issue can be generalized as how the reader to know the command received from CN is targeted for which device, there may have two different solutions to solve this issue from our point of view: option1 is that the paging ID (may also be temp ID or device ID) is generated by the CN is visible to the MAC layer as proposed by some companies; and option2 is that the reader may identify that the received command is for which device based on the NGAP-like id between the reader and CN, this option may fall into RAN3’ scope. According to RAN3 LS R3-252481, the reader will allocate NGAP device ID for each device and maintain the per-session per-device context. Therefore, above issue can be resolved, it is suggested RAN2 to confirm the working assumption that the paging identifier is transparent to the AIoT MAC layer and can be carried by upper layer.

Proposal 3 (Issue 1-7): RAN2 to confirm the working assumption that the paging identifier is transparent to the A-IoT MAC layer and can be carried by upper layer.

- Conclusion

In this paper, we discussed the issues related to A-IoT paging procedure, and have following proposals:

Proposal 1 (Issue 1-2): It is left to reader’s implementation to generate the transaction ID, the size of the transaction ID can be 3-4 bits.

Proposal 2 (Issue 1-5): For CFA paging, the field related to transaction ID is not needed.

Proposal 3 (Issue 1-7): RAN2 to confirm the working assumption that the paging identifier is transparent to the A-IoT MAC layer and can be carried by upper layer.

- References

3GPP RAN#107, RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.

3GPP RAN2#129 meeting, Chair notes.

3GPP RAN2#130 meeting, Chair notes.