3GPP TSG-RAN WG2 Meeting#131	R2-2505682

Bangalore, India, Aug. 25th-29th, 2025

Agenda item:		8.4.2

Source:		Lenovo

Title:		 Open issues on LP-WUS in RRC Idle/ Inactive mode

Document for:		Discussion and Decision

- Introduction

In RAN#103 meeting, WID of Rel-19 Low-power wake-up signal and receiver for NR (LP-WUS/ WUR) [1] was discussed and confirmed. Currently, the UE’s Main Radio (MR) needs to periodically monitor PDCCH once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it needs to monitor PDCCH [2].

Furthermore, the following agreements were made during previous RAN2 meetings:

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. From RAN2 perspective, no new procedure is introduced for SI reception/updates. If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements. FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec. Send LS to RAN1 and RAN4 to inform the agreements. For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - - UE_ID is related to 5G-S-TMSI, - - N is the number of total paging frames in one DRX cycle, - - Ns is the number of the PO for a PF, - - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. LP-WUS is supported with eDRX, FFS on exact impact if any Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID = 5G-S-TMSI mod X. X is based on 32 subgrouping number. Details can be discussed in the running CR. Send LS to RAN3 (CC SA2/SA3) to inform our agreements on UE ID based subgrouping. Confirm the principle for determining CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping. Details will be discussed in the running CR. All the LP-WUS related configurations except for measurement configurations are provided in SIB1. FFS the details on measurement configurations. Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes. Use existing Srxlev/Squal for all MR measurement based entry/exit condition evaluation. Use measured value for all LR measurement based entry/exit condition evaluation. The entry/exit condition for LP-WUS monitoring is mandatory in LP-WUS configuration, if the LP-WUS configuration is provided by the NW. Full coverage for LP-WUS is not precluded, e.g., if there needs to be is a threshold value so that the condition is always fulfilled for all LPWUS UEs. It is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement-based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types. RAN2 aim at supporting enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, if the solution can be concluded in the August meeting.

This contribution looks at the remaining open issues related to LP-WUS in RRC Idle/ Inactive mode based on the open issue list.

- Discussion

Open Issue (RRC-12): Whether/how to enable/disable LP-WUS e.g., by RRC/NAS signaling

In RAN2#130 meeting [3], RAN2 has discussed the issue on whether to support enabling/disabling of LP-WUS in RRC_INACTIVE and RRC_IDLE state, the agreement has been reached that RAN2 aims at supporting enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, if the solution can be concluded in the August meeting. Regarding to how to support enabling/disabling LP-WUS in RRC_Inactive/Idle state, serval solutions have been proposed during the previous discussions, as follows:

•	NAS signalling

•	RRC signalling

•	No NAS/RRC spec changes

In our view, we do not think the motivation to support dedicated enabling/ disabling of LP-WUS for Idle/ Inactive modes at the end of the release is well-justified.  Firstly, the false alarm is not a critical issue since LP-WUS has been designed with 31 subgroups.  And latency also seems not critical since LP-WUS in Idle/ Inactive mode is mainly used for paging and that is generally not a latency-sensitive procedure. With respect to frequent transitions between MR and LR, we think that having different entry/exit conditions on different radios already address this issue. Additionally, RAN2 currently does not define any additional enable/ disable signaling for features such as PEI, so we think that LP-WUS feature can follow the same principle. Since during the email discussion [Post130][222][LPWUS], potential solution to support enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, seems most of companies aim to support such dedicated enable/disable mechanism,  we suggest to use legacy signaling related to subgroup ID to achieve the dedicated enabling/ disabling of LP-WUS per UE or just based on UE implementation.  RAN2 does not additionally specify any signaling for enabling/ disabling of LP-WUS feature in Idle/ Inactive mode.

- Proposal 1: (RRC-12) RAN2 does not additionally specify any signaling for enabling/ disabling of LP-WUS feature in Idle/ Inactive mode.

Open Issue (38304-8): Whether LP-WUS is only used in the last used cell or in any cell

Since it was previously agreed to use PEI as a baseline for LP-WUS subgrouping, we think that the same principle can be applied with respect to consider whether LP-WUS is only used in the last used cell or in any cells. In the case of lastUsedCellOnly is configured in system information of a cell, the UE monitors LP-WUS in this cell only if the UE most recently received RRCRelease without noLastCellUpdate in this cell, then a LP-WUS UE needs to store its last used cell information. Otherwise (i.e., if lastUsedCellOnly is not configured in system information of a cell), the UE monitors LP-WUS in the camped cell.

- Proposal 2: (38304-8) LP-WUS is only used in the last used cell when lastUsedCellOnly is configured in SIB.

Open Issue (38304-10): FFS whether/how LP-WUS with SDT is supported

Another open issue raised by the companies during the last meeting is whether/how to support LP-WUS with SDT[4]. In our understanding, there has no limitation for a LP-WUS UE to support legacy SDT procedures including both MO-SDT and MT-SDT.  For MO-SDT, which is initiated by the UE only if less than or equal to a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, if the UE is monitoring LP-WUS and there has UL data for transmission, UE may wake up the MR to initiate the UL data transmission based on implementation; While for MT-SDT,  which is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for radio bearers configured for SDT. In this case, according to current procedure, NW needs to transmit LP-WUS to wake up the UE which is monitoring LP-WUS before transmitting the paging message includes the SDT indication, then UE is awaked to perform the SDT procedure as legacy. Therefore, we have not seen additional specification impacts to support LP-WUS UE to perform SDT procedures if configured.

- Proposal 3: (38304-10) LP-WUS can support SDT procedure if configured with no additional specification impacts.

- Conclusion

This document discusses some procedures for a LR capable UE in the RRC_IDLE/ RRC_INACTIVE Mode. Following are the proposals and observations made in this document:

- (RRC-3) RAN2 does not additionally specify any signaling for enabling/ disabling of LP-WUS feature in Idle/ Inactive mode.

- (38304-8) LP-WUS is only used in the last used cell when lastUsedCellOnly is configured in SIB.

- (38304-10) LP-WUS can support SDT procedure if configured with no additional specification impacts.

- References

[1] 3GPP RAN#103, RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] 3GPP TR 38.869 v18.0.0: Study on low-power wake up signal and receiver for NR (Release 18)

[3] 3GPP RAN2 #130 meeting, chair notes.

[4] R2-2504264, LP-WUS in IDLE and INACTIVE, Nokia.