3GPP TSG-RAN WG2 Meeting #131	R2-2505701

Bangalore, India  Aug 25th – 29th , 2025

Agenda item:		8.17

Work Item:			IoT_NTN_TDD

Source:	Nordic Semiconductor ASA, Iridium, Thales

Title:					On early implementation of the IoT-NTN TDD mode

Document for:		Discussion and Decision

# Introduction

This paper discusses the early implementation of the IoT-NTN TDD mode [1] and documenting it in the Annex G of TS36.331.

# Early implementation of IoT-NTN TDD Mode

The IoT-NTN TDD mode can be seen and implemented as distinct feature on a basis of Rel-17 NB-IOT IoT-NTN. Its UE and network capability is implicitly associated with the used TDD band (currently only B249) that a non-“IoT-NTN TDD mode” capable UE won’t ever attempt to search for nor camp on a cell on that band. Therefore, there is no need for the UE to explicitly signal its capability and the network to explicitly signal support for the IoT-NTN TDD mode [2].

Observation-1: The UE capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Observation-2: The network capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Although new bands are introduced in a specific 3GPP release, it is typical for UEs to add support for bands as soon as they become available even if the UE ASN.1 baseline is still on the earlier releases. The same practice would apply in the IoT-NTN TDD mode case. Furthermore, there is no UE capability ambiguity on the network side even if an IoT-NTN TDD mode compliant UE with advertised Rel-17 ASN.1 baseline signals support for Rel-19 an IoT-NTN TDD mode specific band (B249 in this case), since the entire feature is associated to a specific band [2].

Observation-3: There is no UE IoT-NTN TDD mode capability or band support ambiguity from the network point of view, since supporting UEs are only being served on specific IoT-NTN TDD mode bands.

The baseline functionality of the IoT-NTN TDD mode has no Rel-18 features or other Rel-19 features that a compliant UE or a network would mandatorily or even optionally need to implement. The IoT-NTN TDD mode can be implemented directly on top of Rel-17 with an assumption the UE only needs to parse the IoT-NTN Mode related Rel-19 ASN.1.

Observation-4: The Rel-19 IoT-NTN TDD mode can be implemented on top of Rel-17.

Proposal-1: Document in 3GPP TS36.331 Annex G that Rel-19 IoT-NTN TDD mode is early implementable feature starting from Rel-17 for a UE supporting a IoT-NTN TDD mode band.

# Conclusions

In this paper, we discussed the early implementation of the IoT-NTN TDD mode. Our observations and proposals are listed below:

Early implementation of the IoT-NTN TDD mode:

Observation-1: The UE capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Observation-2: The network capability of the IoT-NTN TDD mode is implicitly associated with the used TDD band.

Observation-3: There is no UE IoT-NTN TDD mode capability or band support ambiguity from the network point of view, since supporting UEs are only being served on specific IoT-NTN TDD mode bands.

Observation-4: The Rel-19 IoT-NTN TDD mode can be implemented on top of Rel-17.

Proposal-1: Document in 3GPP TS36.331 Annex G that Rel-19 IoT-NTN TDD mode is early implementable feature starting from Rel-17 for a UE supporting a IoT-NTN TDD mode band.

# References

- RP-243293	Revised WID on introduction of IoT-NTN TDD_rm, RAN #106 Madrid, Spain, December 2024

- R2-2505148	Introduction of capabilities for IoT NTN TDD, RAN2#131