3GPP TSG-RAN WG2 Meeting #131	R2-2505718

Bangalore, India, 25th – 29th of August 2025

Agenda item:	7.0.2.22

Source:	Nokia

Title:	Corrections to Rel-18 LTM (LTM Recovery to the source cell)

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we are addressing several remaining issues for Release 18 LTM. Our focus is predominantly on the issue of LTM recovery when the selected cell is the source cell. This problem has been discussed initially during RAN2#130 based on [1] and the following has been noted [2]:

R2-2504136 Corrections to Rel-18 LTM (interruptions during RACH and LTM Recovery to the source cell) Nokia discussion Rel-18 NR_Mob_enh2-Core Proposal 2: RAN2 to confirm that LTM Fast recovery procedure is not applicable if the selected cell for recovery is same as current serving cell. [Ericsson]: So far, we did not forbid the case where the UE select the source cell when performing the fast LTM recovery procedure and I don’t think that this causes confusion at the network, since the network knows which candidates has configured. One it may argue why a UE selects a cell which is the previous failed cell, but I think that we can rely on the UE to be smart enough. [Qualcomm]: Do not agree with proposal. [Huawei]: Do not understand what network issue really is. [ZTE]: Candidate configuration id is included in RRC connection reconfiguration and network is aware of which configuration used by the UE. Do not see an issue. Noted.

As can be seen above, the TDoc was noted and the companies were given additional time to consider whether the topic we have raised in [1] is indeed problematic.

# 2	Discussion

## 2.1 	On LTM Recovery to the source cell

The fast recovery was introduced for CHO to allow the UE to recover directly using the conditional configuration if the selected cell during re-establishment is one of the prepared CHO candidates. This was further extended for SCPAC and LTM in Rel-18 for fast recovery. In Rel-18 LTM this functionality is enabled through attemptLTM-Switch parameter in LTM-Config information element. In some recovery scenarios it is possible that the best cell selected for recovery can be the current/last serving cell (i.e. source cell). In this scenario, if there is a valid candidate configuration available for this cell, the LTM recovery procedure is triggered instead of re-establishment procedure.

Observation 1: When fast recovery procedure is configured for LTM it is possible that UE attempts LTM recovery to the same serving cell using the candidate configuration stored for the current serving cell.

The key issues in handling the above recovery procedure for serving cell are explained below.

When candidate cells are prepared for LTM switching the resources and configurations are reserved for incoming handover for the UE when the UE is served by other candidate cells. However, these configurations are maintained at UE for subsequent mobility. From gNB perspective, when cell switching is completed, the network activates the candidate configuration of the cell and it becomes the current active UE configuration.

Observation 2: To support LTM Recovery to same serving cell the gNB needs to maintain two configurations for the active UE and also associated resources.

It is expected that the C-RNTI allocated to the UE in both of these configurations to be the same. The fact that the same C-RNTI is used for two UE contexts causes an issue particularly at MAC layer at the DU. The reason is that the MSG3 sent during RACH procedure includes C-RNTI in case of RACH procedure use for serving cell, for instance expiry of TimeAlignmentTimer without SR resources or beam failure recovery and in the case of LTM recovery procedure. Serving cell MAC layer receives this MSG3 and assumes that the UE uses the active UE context. In case this MSG3 carries an RRCReconfigurationComplete message for LTM recovery procedure, the DU forwards this RRC message to CU. For the case of LTM recovery, the CU will send a UE context modification request to DU to indicate to DU that the procedure is a recovery but not a serving cell RACH procedure. Before this UE context modification request is received by the DU, in case the active UE context and pre-configured candidate configuration differs on MAC layer, i.e., different PUCCH configuration. The UE may use some PUCCH configuration that the DU does not expect UE to use as it assumes it shall use the active configuration.

Observation 3: The DU may experience user configuration mismatch after LTM recovery to serving cell, as it may be perceived as a serving cell RACH procedure after loss of TA. This complicates handling of serving cell RACH procedure.

The self-recovery using LTM configuration also leads to mismatch in UE and network configuration in many scenarios where the gNB modifies the current serving cell configuration after cell switch but does not want to prepare LTM candidate again for each of these changes.  For example, the LTM candidates are prepared with one secondary cell based on current configuration. However, after LTM preparation depending on data volume secondary cells may be added or released dynamically in current serving cell. For each of these configuration changes LTM candidates need not be prepared again and gNB may update other candidate configurations only for essential changes. In such case, if the RLF or LTM switch failure happens and if the UE selects the candidate configuration of serving cell for recovery this configuration will be an outdated configuration, and additional reconfiguration will be required to switch to the latest configuration.

Observation 4: Self recovery using stored LTM candidate configuration will result in UE using the older configuration on LTM recovery and another RRC Reconfiguration will be needed on completing the recovery procedure.

Moreover, similar behavior as achieved with LTM recovery to serving cell can already be achieved with existing recovery procedures such as beam failure recovery or RACH to serving cell in case of TAT expiry without SR resources. From the network’s side it is simpler to use these procedures instead of LTM recovery for the serving cell with the same end result.

To avoid additional complexity associated with handling two configurations for active UE and also possible configuration mismatch associated with the self-recovery we propose that UE uses LTM candidate configuration for recovery only if the selected cell is not the current serving cell.

Proposal 1: RAN2 to confirm that LTM Fast recovery procedure is not applicable if the selected cell for recovery is the same as current serving cell (i.e. the UE uses the legacy re-establishment in this case).

Once Proposal 1 is agreed, we should think how this can be reflected in the existing RAN2 specifications. Our suggestion is to modify the procedural text in 5.3.7.3 of [3], as shown in Annex A.

Proposal 2: Adopt the text proposal for TS 38.331 shown in Annex A.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: When fast recovery procedure is configured for LTM it is possible that UE attempts LTM recovery to the same serving cell using the candidate configuration stored for the current serving cell.

Observation 2: To support LTM Recovery to same serving cell the gNB needs to maintain two configurations for the active UE and also associated resources.

Observation 3: The DU may experience user configuration mismatch after LTM recovery to serving cell, as it may be perceived as a serving cell RACH procedure after loss of TA. This complicates handling of serving cell RACH procedure.

Observation 4: Self recovery using stored LTM candidate configuration will result in UE using the older configuration on LTM recovery and another RRC Reconfiguration will be needed on completing the recovery procedure.

Proposal 1: RAN2 to confirm that LTM Fast recovery procedure is not applicable if the selected cell for recovery is the same as current serving cell (i.e. the UE uses the legacy re-establishment in this case).

Proposal 2: Adopt the text proposal for TS 38.331 shown in Annex A.

# References

- R2-2504136 Corrections to Rel-18 LTM (interruptions during RACH and LTM Recovery to the source cell) 3GPP TSG-RAN WG2 Meeting #130 Malta, May 19th – 23rd, 2025

- R2-2504671 Report from session on R18 SL, R18/19 MOB and R19 NES 3GPP TSG-RAN WG2 Meeting #130 Malta, May 19th – 23rd, 2025

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification, V18.6.0 (2025-06)

# Annex A – LTM recovery to the source cell (NR RRC changes)

#### 5.3.7.3	Actions following cell selection while T311 is running

[…]

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG and is not the cell from which the LTM cell switch command was received prior to detection of radio link failure or re-configuration with sync failure:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

[…]