3GPP TSG-RAN WG2 #131	R2-2505726

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5430 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fast failover via pre‑configured egress candidate list for multi‑hop L2 U2N relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_multihop |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-19 introduces multi-hop L2 U2N relay. RAN2 agreed to use NotificationMessageSidelink when AS-layer issues trigger upstream relay (re)selection, with legacy causes and idle/inactive handling left to upper layers; this avoids inventing a new message or enumerant. At the same time, the 38.351 running CR anchors egress link/channel determination on RRC-configured mappings keyed by (UE ID, BEARER ID) and still carries FFS on the configuration model for indirectly-connected UEs. This CR provides a pre-configured egress candidate list per (LocalIdentity, RB) so an intermediate/last relay can locally switch to a backup next hop on AS-layer failure—with no new on-air header—while remaining consistent with service-continuity flows in 38.300. Degradation-based triggers are FFS; this version specifies failure-only triggers. This procedure does not alter key or PDCP contexts; upper layers remain authoritative. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new sl-FastFailoverConfig to SL-L2RelayUE-Config, containing a priority-ordered list of egress candidates keyed by (sl-LocalIdentity, RB). Each candidate specifies exactly one interface via a CHOICE {PC5 ¦ Uu}. No new enumerants are introduced for NotificationMessageSidelink. - Specify relay behavior: upon AS‑layer link failure on the current egress (PC5 RLF/release or Uu RLF/release depending on hop), the relay selects the next available egress candidate by priority without requiring new RRC; then, if the selected change impacts the child, the relay uses NotificationMessageSidelink as already agreed in R2. (No change to the message itself.) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local fast failover in multi‑hop relies on full RRC reconfiguration during failures, increasing interruption time and complexity in scenarios A/B service continuity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.15.2; 5.3.5.15.3; 5.3.5.15.3a (new); 5.8.9.10.4; 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.3.5.15	L2 U2N or U2U Relay UE configuration

##### 5.3.5.15.1	General

The network configures the L2 U2N or U2U Relay UE with relay operation related configurations. For each connected L2 U2N or U2U Remote UE indicated in sl-L2IdentityRemote, the network provides the configuration parameters used for relaying.

The L2 U2N Relay UE shall:

1>	if sl-L2RelayUE-Config is set to setup:

2>	if the sl-L2RelayUE-Config contains the sl-RemoteUE-ToReleaseList:

3>	perform the L2 U2N Remote UE release as specified in 5.3.5.15.2;

2>	if the sl-L2RelayUE-Config contains the sl-RemoteUE-ToAddModList:

3>	perform the L2 U2N Remote UE addition/modification as specified in 5.3.5.15.3;

1>	else if sl-L2RelayUE-Config is set to release:

2>	release the L2 U2N relay operation related configurations.

The L2 U2U Relay UE shall:

1>	if sl-L2RelayUE-Config is set to setup:

2>	if the sl-L2RelayUE-Config contains the sl-U2U-RemoteUE-ToReleaseList:

3>	perform the L2 U2U Remote UE release as specified in 5.3.5.15.2;

2>	if the sl-L2RelayUE-Config contains the sl-U2U-RemoteUE-ToAddModList:

3>	perform the L2 U2U Remote UE addition/modification as specified in 5.3.5.15.3;

1>	else if sl-L2RelayUE-Config is set to release:

2>	release the L2 U2U relay operation related configurations.

##### 5.3.5.15.2	L2 U2N or U2U Remote UE Release

The L2 U2N Relay UE shall:

1>	if the release is triggered by reception of the sl-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5 RRC connection to a L2 U2N Remote UE with SL-DestinationIdentity:

4>	indicate upper layers to trigger PC5 unicast link release.

The L2 U2U Relay UE shall:

1>	if the release is triggered by reception of the sl-U2U-RemoteUE-ToReleaseList:

2>	for each SL-DestinationIdentity value included in the sl-U2U-RemoteUE-ToReleaseList:

3>	if the current UE has a PC5-RRC connection to a L2 U2U Remote UE with this SL-DestinationIdentity:

4>	release the configuration associated with the L2 U2U Remote UE.

Fast failover interaction:

1> If the release is triggered due to AS-layer failure of an upstream egress link and sl-FastFailoverConfig is configured for the affected sl-L2IdentityRemote,

2> the relay UE may perform local egress re-selection in accordance with 5.3.5.15.3a.

3> If the re-selection impacts the child UE (e.g., parent change), the relay shall indicate the change to the child using NotificationMessageSidelink as specified in 5.8.9.10.4.

##### 5.3.5.15.3	L2 U2N or U2U Remote UE Addition/Modification

The L2 U2N Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is not part of the current UE configuration (L2 U2N Remote UE Addition):

2>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigRelay;

2>	if SRB1 is included in sl-MappingToAddModList, and sl-EgressRLC-ChannelPC5 is configured:

3>	release SL-RLC1, if established;

3>	associate the PC5 Relay RLC channel as indicated by sl-EgressRLC-ChannelPC5 with SRB1;

2>	else: (i.e. SRB1 is not included in sl-MappingToAddModList, or SRB1 is included in sl-MappingToAddModList, but sl-EgressRLC-ChannelPC5 is not configured)

3>	if SL-RLC1 is not established:

4>	apply the default configuration of SL-RLC1 as specified in clause 9.2.4 and associate it with the SRB1;

1>	for each sl-L2IdentityRemote value included in the sl-RemoteUE-ToAddModList that is part of the current UE configuration (L2 U2N Remote UE modification):

2>	modify the configuration in accordance with the sl-SRAP-ConfigRelay;

The L2 U2U Relay UE shall:

1>	if no SRAP entity has been established:

2>	establish a SRAP entity as specified in TS 38.351 [66];

1>	for each target L2 U2U Remote UE indicated in sl-L2IdentityRemoteUE value included in the sl-U2U-RemoteUE-ToAddModList that is not part of the current UE configuration (target L2 U2U Remote UE Addition):

2>	for each source L2 U2U Remote UE indicated in sl-SourceUE-Identity in accordance with one entry of the sl-SourceRemoteUE-ToAddModList:

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

1>	for each target L2 U2U Remote UE indicated in sl-L2IdentityRemote value included in the sl-U2U-RemoteUE-ToAddModList that is part of the current UE configuration (target L2 U2U Remote UE modification):

2>	for each source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToReleaseList (source L2 U2U Remote UE Release):

3>	release the configuration associated with the source L2 U2U Remote UE;

2>	for the source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToAddModList that is not part of the current UE configuration (source L2 U2U Remote UE Addition):

3>	configure the parameters to SRAP entity in accordance with the sl-SRAP-ConfigU2U;

2>	for the source L2 U2U Remote UE indicated in sl-SourceUE-Identity included in the sl-SourceRemoteUE-ToAddModList that is part of the current UE configuration (source L2 U2U Remote UE modification):

3>	modify the configuration in accordance with the sl-SRAP-ConfigU2U;

1> if sl-FastFailoverConfig is included:

2> for each (sl-L2IdentityRemote, sl-RemoteUE-RB-Identity):

3> store the priority-ordered egress candidates for local egress re-selection upon AS-layer failure on the current egress link.

##### 5.3.5.15.3a Local egress re-selection (fast failover) at L2 U2N Relay UE

When sl-FastFailoverConfig is configured for a (sl-L2IdentityRemote, RB) and the relay detects AS-layer failure (PC5 link failure/release or Uu RLF/release) on the current egress, the relay UE shall:

1> select the highest-priority entry in sl-EgressCandidateList that is not barred and for which lower layers indicate the link is available;

2> update the egress link and RLC channel used for that (sl-L2IdentityRemote, RB) accordingly;

3> if the selected change modifies the parent as seen by the child UE, submit NotificationMessageSidelink as specified in 5.8.9.10.4; otherwise, no signaling to the child is required;

4> if no candidate is available, indicate failure to upper layers.

NOTE 1: The network may subsequently send RRCReconfiguration; this procedure does not preclude it.

NOTE 2: This subclause does not introduce any new on-air headers; it builds on the SRAP mapping model and existing RRC lists for indirectly-connected UEs.

NEXT CHANGE

##### 5.8.9.10.4	Actions related to reception of NotificationMessageSidelink message

Upon receiving the NotificationMessageSidelink, the Remote UE shall:

1>	if the UE is acting as U2N Remote UE:

2>	if the indicationType is included:

3>	if the UE is L2 U2N Remote UE in RRC_CONNECTED:

4>	if MP is configured and MCG transmission (i.e. direct path) is not suspended;

5>	if the indicationType is relayUE-HO;

6>	suspend indirect path transmission;

5>	else:

6>	initiate the indirect path failure information procedure as specified in 5.7.3c to report indirect path failure;

4>	else if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7;

3>	else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE):

4>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

5>	indicate upper layers to trigger PC5 unicast link release;

4>	else (i.e., maintain the PC5 RRC connection):

5>	if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection:

6>	consider cell re-selection occurs;

When the upstream relay (re)selection is triggered by AS-layer failure and the Remote UE is in RRC_IDLE/INACTIVE, further handling is up to upper layers; the UE may keep or release the PC5 link as already specified for Notification handling.

NOTE 1:	For L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, it is up to Remote UE implementation whether to release or keep the PC5 unicast link.

NOTE 2:	The L2 U2N Remote UE may ignore the NotificationMessageSidelink if it does not release the PC5 unicast link in source side yet during an indirect-to-direct path switch, i.e. T304 is running.

1>	if the UE is acting as L2 U2U Remote UE:

2>	if sl-IndicationType is relayUE-PC5-RLF:

3>	indicate PC5 RLF received from L2 U2U Relay UE to the upper layers for the indicated L2 U2U Remote UE based on the received sl-DestinationIdentityRemoteUE;

3>	consider the end-to-end PC5 connection failure for the end-to-end PC5 connection(s) over the per-hop PC5 link between the L2 U2U Relay UE and the L2 U2U Remote UE identified by sl-DestinationIdentityRemoteUE;

3>	perform the end-to-end PC5 connection failure related actions as specified in 5.8.9.3a;

NOTE 3:	It is up to the upper layers on whether to trigger U2U Relay reselection and whether to keep or release the PC5 link with the U2U Relay UE after the PC5 RLF indication received from U2U Relay UE.

NEXT CHANGE

### 6.3.5	Sidelink information elements

#### –	SL-L2RelayUE-Config

The IE SL-L2RelayUE-Config is used to configure L2 U2N relay operation related configurations used by L2 U2N Relay UE, or L2 U2U relay operation related configurations used by L2 U2U Relay UE.

SL-L2RelayUE-Config information element

-- ASN1START

-- TAG-SL-L2RELAYUE-CONFIG-START

SL-L2RelayUE-Config-r17 ::=        SEQUENCE {

sl-RemoteUE-ToAddModList-r17       SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17    OPTIONAL,    -- Need N

sl-RemoteUE-ToReleaseList-r17      SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16  OPTIONAL,    -- Need N

...,

[[

sl-U2U-RemoteUE-ToAddModList-r18   SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-U2U-RemoteUE-Config-r18   OPTIONAL,    -- Need N

sl-U2U-RemoteUE-ToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16   OPTIONAL     -- Need N

]],

[[

sl-FastFailoverConfig-r19          SL-FastFailoverConfig-r19														OPTIONAL     -- Need N

]]

}

SL-RemoteUE-ToAddMod-r17 ::=       SEQUENCE {

sl-L2IdentityRemote-r17            SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17            SL-SRAP-Config-r17                                                      OPTIONAL,    -- Need M

...

}

SL-U2U-RemoteUE-Config-r18 ::=      SEQUENCE {

sl-L2IdentityRemoteUE-r18           SL-DestinationIdentity-r16,

sl-SourceRemoteUE-ToAddModList-r18  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-SourceRemoteUE-Config-r18   OPTIONAL,    -- Need N

sl-SourceRemoteUE-ToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-SourceIdentity-r17          OPTIONAL,    -- Need N

...

}

SL-SourceRemoteUE-Config-r18 ::=   SEQUENCE {

sl-SourceUE-Identity-r18           SL-SourceIdentity-r17,

sl-SRAP-ConfigU2U-r18              SL-SRAP-ConfigU2U-r18,

...

}

-- TAG-SL-L2RELAYUE-CONFIG-STOP

-- ASN1STOP

SL-L2RelayUE-Config field descriptions

sl-RemoteUE-ToAddModList List of L2 U2N Remote UEs to be added and modified to the L2 U2N Relay UE.

sl-RemoteUE-ToReleaseList List of L2 U2N Remote UEs to be released by the L2 U2N Relay UE.

sl-U2U-RemoteUE-ToAddModList List of target L2 U2U Remote UEs for which the related configuration is to be added and modified to the L2 U2U Relay UE.

sl-U2U-RemoteUE-ToReleaseList List of target L2 U2U Remote UEs for which the related configuration is to be released by the L2 U2U Relay UE.

sl-U2U-SourceRemoteUE-ToAddModList List of Source L2 U2U Remote UEs for which the related configuration is to be added and modified relative to the destination L2 U2U Remote UE identified by the sl-L2IdentityRemoteUE.

sl-U2U-SourceRemoteUE-ToReleaseList List of Source L2 U2U Remote UEs for which the related configuration is to be released relative to the destination L2 U2U Remote UE identified by the sl-L2IdentityRemoteUE.

sl-FastFailoverConfig Provides a priority-ordered list of egress candidates per (sl-LocalIdentity, sl-RemoteUE-RB-Identity) to enable local egress re-selection at the L2 U2N Relay UE upon AS-layer failure of the current egress link. No change to SRAP header content.

#### –	SL-FastFailoverConfig

The IE SL-FastFailoverConfig provides a pre-configured, priority-ordered list of egress candidates per (LocalIdentity, RB) for local egress re-selection upon AS-layer failure of the current egress link at an L2 U2N Relay UE.

-- ASN1START

-- TAG-SL-FASTFAILOVERCONFIG-START

SL-FastFailoverConfig-r19 ::= SEQUENCE {

sl-EgressCandidateList-r19    SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-EgressCandidateSet-r19  OPTIONAL,  -- Need N

failoverEnabled-r19           BOOLEAN DEFAULT TRUE

}

SL-EgressCandidateSet-r19 ::= SEQUENCE {

sl-L2IdentityRemote-r19       SL-DestinationIdentity-r16,   -- key: child UE L2 identity

sl-RemoteUE-RB-Identity-r19   INTEGER,                      -- key: RB identity (SRB/DRB)

egressCandidates-r19          SEQUENCE (SIZE (1..4)) OF SL-EgressCandidate-r19

}

SL-EgressCandidate-r19 ::= SEQUENCE {

interface CHOICE {

pc5  SEQUENCE {

sl-NextHopL2ID-r19          SL-DestinationIdentity-r16,

sl-EgressRLC-ChannelPC5-r19 INTEGER

},

uu   SEQUENCE {

sl-NextHopL2ID-r19          SL-DestinationIdentity-r16,

sl-EgressRLC-ChannelUu-r19  INTEGER

}

},

priority-r19                  INTEGER (1..8)   -- 1 = highest

}

-- TAG-SL-FASTFAILOVERCONFIG-STOP

-- ASN1STOP

SL-FastFailoverConfig field descriptions

sl-EgressCandidateList Priority-ordered candidate next-hops per (sl-LocalIdentity, sl-RemoteUE-RB-Identity) to be used upon AS-layer failure on the current egress; re-selection is a local relay action (no SRAP header change).

failoverEnabled Enables/disables local fast failover for the configured sets.

sl-L2IdentityRemote / sl-RemoteUE-RB-Identity Keys identifying the mapping the candidates apply to.

egressCandidates 1..4 candidates, each with sl-NextHopL2ID and the egress RLC channel to use. – pc5: { sl-NextHopL2ID, sl-EgressRLC-ChannelPC5 } – uu : { sl-NextHopL2ID, sl-EgressRLC-ChannelUu } and a priority (1 = highest).

END OF CHANGE