3GPP TSG-RAN WG2 Meeting #R2-131		        R2-2505730     Bangalore, India, Aug 25-29,2025

Agenda Item:	8.6.2

Source: 	Rakuten

Title:  	Remaining issues of Inter-CU LTM

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

The WID objective mentions that for inter-CU LTM Rel-18 intra-CU LTM is the baseline. This implies that any enhancements for the inter-CU LTM use cases are going to be on top of the Rel 18 framework.

In this contribution, we describe and discuss some aspects involved in extending LTM to support inter-CU use-cases. We also propose a way forward for some of these.

# Discussion

## Successful completion of a RACH-less Inter-CU LTM completion

In Release 18, in the context of a successful RACH-less LTM cell switch, the UE indicates a successful LTM cell switch to the network by sending a UL data packet to the target cell. Further, the UE determines successful reception of its first UL data at the network, based on the UE receiving a PDCCH addressing the UE’s C-RNTI in the new serving cell scheduling a new transmission after the first UL data is sent.

This is captured in TS 38.321 in section 5.3.1 and an excerpt is provided below.

Let us consider a scenario where,

- Post execution of the RACH-less LTM cell switch, the source gNB-DU sends the RACH preamble and beam information delivered to the UE in the LTM cell switch command (MAC CE), to the target gNB-DU.

- Based on this information, the target gNB-DU schedules a PDCCH addressing the UE’s C-RNTI and provides dynamic UL scheduling grants.

- In parallel, the UE completes a RACH-less LTM cell switch and uses a Configured Grant (CG) to send the first UL data to signal a successful RACH-less LTM cell switch.

In such a scenario, the UE determining that there was a successful reception of the first UL data (sent using CG) at the network based on the dynamic grants being scheduled and concluding that the LTM cell switch is successful may not be correct as the dynamic grants were not scheduled based on the reception of the first UL data. In such a scenario, the first UL data packet sent by the UE was not received successfully, but the gNB allocating dynamic grants for pre-scheduling was successful.

Hence, there is probably a need for the UE to initiate a fresh scheduling request (despite receiving the dynamic grants) and ensure that the gNB has successfully received the UL data. If the scheduling request is answered by the gNB, the UE can conclude that the UL transmission was successful.

UE determining a successful reception of its first UL data at the gNB, based on the dynamic grants being scheduled and concluding the success of the LTM cell switch may not be appropriate as the dynamic grants may not have been scheduled based on the reception of the first UL data.

The UE initiates a fresh scheduling request with the new serving gNB-DU, despite allocation of the dynamic grants, to ensure that the first UL data was successfully received at the gNB.

If the new serving gNB-DU issues dynamic grant corresponding to the new scheduling request, the UE concludes that the UL transmission was successful and the RACH-less LTM is completed successfully.

# Conclusion

We have the following observations:

Observation 1	UE determining a successful reception of its first UL data at the gNB, based on the dynamic grants being scheduled and concluding the success of the LTM cell switch may not be appropriate as the dynamic grants may not have been scheduled based on the reception of the first UL data.

We have the following proposals:

Proposal 1	The UE initiates a fresh scheduling request with the new serving gNB-DU, despite allocation of the dynamic grants, to ensure that the first UL data was successfully received at the gNB.

Proposal 2	If the new serving gNB-DU issues dynamic grant corresponding to the new scheduling request, the UE concludes that the UL transmission was successful and the RACH-less LTM is completed successfully.

# References

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RAN2 Chairman Notes R2_127bis_ChairNotes

RAN2 Chairman Notes R2_128_ChairNotes

TS 38.321 NR; Medium Access Control (MAC) protocol specification (Release 18)

RP-241106 : Rel-18 LTM correction for LTM with gNB-CU-UP change