3GPP TSG-RAN WG2 Meeting #131	R2-2505736

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.9.3

Source:	Huawei, HiSilicon

Title:	Further consideration on UL capacity enhancement

Document for: Discussion and Decision

# 1	Introduction

According to the RAN plenary meeting, the following objectives on IoT-NTN were agreed:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In the last meeting, RAN2 has made the following agreements:

Agreements:

1.	The maximum TBS could be different for different CE levels.

2.	Due to only CE mode A is supported for eMTC NTN, only 1 separate RSRP thresholds and 2 CE levels are supported (revised agreement)

3.	Multiple contention resolution IDs could be included in CB-MSG4, the information related to multiple UEs can be multiplexed in the MAC PDU.

4.	The number of Msg3 replies in one Msg4 can be left to eNB implementation. Expect no SPEC impact.

5.	The HARQ feedback resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE. RAN2 could revisit this proposal if RAN1 has some concern.

6.	Whether to send the HARQ feedback for CB-Msg4 can be controlled by NW. UE does not send HARQ NACK.

7.	For NB-IoT, the SubCarrierSpacing of the HARQ feedback for CB-Msg4 is same as the CB-Msg3.

8.	Reuse the existing format of HARQ ACK allocation signalling in the DCI. There is 2-bit HARQ ACK resource for eMTC and 4-bit HARQ ACK resource for NB-IoT. Reuse the meaning of DCI field in R1 SPEC. Send LS to RAN1 for information on all RAN2 decisions related to HARQ feedback

9.	Introduce a new MAC PDU for CB-Msg4 including new types of MAC sub-header and a new type of MAC payload

10.	The MAC PDU for CB-Msg4 consists of sub-header(s) followed by MAC payload and optional padding if needed.

11.	Introduce a new CB BI MAC sub-header in CB-MSg4 for backoff parameter. There is 4 bits BI for backoff indication.

12.	Introduce a new CB-Msg3 Response (CBR) MAC sub-header in CB-Msg4. It has 1bit E for sub-header/payload indication, 2 bits T for sub-header type, 1bit T2 for HARQ ACK resource present, 1 bit T3 for TAC present, 1 bit T4 for C-RNTI present and 2bit R for reservation (the field names and sub-header names in above agreements could be further changed during MAC running CR review)

Agreements – part 2:

1.	The CB-Msg3-EDT configuration is configured per carrier (including anchor and non-anchor carrier). Within each carrier, the CB-Msg3-EDT could be configured per CE level

2.	Regarding the mapping of NPUSCH resource to enhanced coverage levels, enhanced coverage levels are numbered from 0 and the mapping of NPUSCH resources to enhanced coverage levels are done in increasing [number of repetition] order (as legacy RACH)

3.	For NB-IoT, when multiple carriers provide CB-Msg3-EDT resources for the same enhanced coverage level, the NB-IoT UE selects the carrier based on the probabilities of each carrier. A new probability parameter for anchor carrier is introduced in SIB22-NB. The remaining probability is evenly split among the non-anchor carriers.

4.	When max re-attempt number for current CE level has been reached, the UE does not move to the next CE level (FFS on the details of the failure behaviour)

5.	The TAC is optionally used in the CB-Msg3 response.

6.	RAN2 assumes that NTA=0 for initial CB-msg3 transmission. Include this in the LS to RAN1 and RAN4

7.	RAN2 assumes the length of the TAC field is 6 bits (we can revisit this if there is major R1 impact on TA calculation)

Agreements – part 3:

1.	Introduce a new CB Data MAC sub-header in CB-MSg4 for MAC SDU for logical channel data. It has 1 bit E for subhead/payload indication, 2 bits T for subhead type, 5 bits LCID, 7 bits or 15 bits L for MAC SDU length, 1 bit F for 15 bits L indication. There is one L field per CB Data sub-header except for the last sub-header.

2.	Introduce a new CB-Msg3 Response (CBR) with variable length. It has 48-bit contention resolution ID, optional HARQ ACK, optional TAC, optional 16-bit C-RNTI.

Agreements:

1.	HARQ process 0 is used to transmit all the CB-Msg3 replicas in the transmission window (RV0 is used to transmit the first repetition of each CB-Msg3 replica in the transmission window)

2.	We do not specify another way of starting Msg4 monitoring window, i.e. it is confirmed that the Msg4 monitoring window always starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not)

3.	A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution.

Working Assumption:

The formula for RNTI for mMsg4 monitoring is:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

•	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

•	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

•	Y is ceil (Msg4_WS/Msg3_WP),

•	CE_level is the CE level, 0 <= CE_level < 3

•	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

0 <= carrier_id < 16

Can come back to check if the NW can also simply configure RNTI = X

Agreements – part 2:

1.	The value of X is 4097 for NB-IoT and 2401 for eMTC

2.	The value of Msg4_WS is the maximum Msg4 window size

3.	The value of Msg3_WP is the minimum Msg3 window periodicity

In this contribution, we will further discuss the remaining issues on CB-msg3-EDT.

# 2	Discussion

## 2.1 OCC

Both RAN1 and RAN2 have already discussed multiple rounds regarding OCC for CB-msg3-EDT. Due to the power imbalance issue identified by RAN1 and consider this is the last meeting before completion of this WI, it is almost impossible to support OCC for CB-msg3-EDT in R19.

In RAN2#129, some companies assume that one possibility to take power imbalance under control is to define RSRP ranges. However, without RAN1’s further evaluation, RAN2 cannot make sure this really works, e.g., how small the power imbalance needs to be. Even if this works under extreme conditions, there will be rarely use cases in reality.

Proposal 1: OCC for CB-msg3 EDT is not supported in this release.

## 2.2 MAC Open Issues

Open issue MAC-6: The power ramping parameters and how the power ramping is done.

In the last meeting, RAN2 has send LS R2-2503175 to RAN1 to ask power ramping for CB-Msg3-EDT. RAN1 replies that there will not be sufficient time to evaluate the performance of power ramping for CB-msg3-EDT. Considering that this is the last meeting for this WI , we prefer not to support power ramping for CB-Msg3 transmission.

Proposal 2 (MAC-6): Power ramping for CB-Msg3 transmission is not supported.

Open issue MAC-9: Whether NW/UE processing time is added before starting the CB-Msg4 monitoring window.

In RAN2#129, it has been agreed that the CB-Msg4 monitoring window starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT. Whether NW/UE processing time needs to be considered is discussed as a MAC open issue.

In our understanding, the processing time is negligible compared to the RTT. So there is no strong motivation to have this. Besides, if we have this, we need to further discuss the values which may be different from the TN considering the multiplexing. The window length can cover the processing time.

Proposal 3 (MAC-9): No need to introduce processing time before starting the CB-Msg4 monitoring window.

Open issue MAC-15: RRC behavior when receiving the CB-Msg3 EDT failure indication from MAC.

When the CB-Msg3 procedure fails, that RRC layer should be notified and performs the fallback. After that, RRC layer should follow the legacy procedure to determine which procedure to fall back to, e.g., legacy 4-step RACH, EDT or PUR).

Proposal 4 (MAC-15): When RRC layer is notified of CB-Msg3 transmission failure, the RRC can follow legacy procedure to determine which procedure to fall back to.

Open issue MAC-17:  Whether to support multiple TBSs.

In legacy EDT, the small TBS subset is supported to reduce the redundant padding and UE power consumption. If enabled in system information, the UE can use a smaller TBS that still allows to fit the data. Since this was introduced by RAN1, it may needs further RAN1 work to support this for CB-Msg3-EDT. At this stage, we are reluctant to do this enhancement in case more issues are found.

Proposal 5 (MAC-17): Small TBS subset is not supported for CB-Msg3-EDT.

# 3	Conclusion

This contribution analyses the UL capacity enhancement, and proposes the following:

Proposal 1: OCC for CB-msg3 EDT is not supported in this release.

Proposal 2 (MAC-6): Power ramping for CB-Msg3 transmission is not supported.

Proposal 3 (MAC-9): No need to introduce processing time before starting the CB-Msg4 monitoring window.

Proposal 4 (MAC-15): When RRC layer is notified of CB-Msg3 transmission failure, the RRC can follow legacy procedure to determine which procedure to fall back to.

Proposal 5 (MAC-17): Small TBS subset is not supported for CB-Msg3-EDT.