3GPP TSG-RAN WG2 Meeting #131	R2-2505737

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.4.3

Source:	Huawei, HiSilicon

Title:	Further discussion on the criteria for RRM measurement relaxation and offloading

Document for:	Discussion and Decision

- Introduction

The following agreements are reached on RRM measurement relaxation and offloading in the latest RAN2 meeting.

RAN2#130 - It is up to NW to configure the condition for LP-WUS monitoring and/or R19 serving/ neighboring cell RRM relaxation /R19 serving cell RRM offloading, as in the current RRC running CR. - The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation (serving cell relaxation and neighboring cell relaxation), if there is such configuration. Capture this in the field description. Details will be discussed in the running CR. - The threshold of the MR based entry condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, which is the maximum of {SIntraSearchP, SnonIntraSearchP}. Capture this in the field description. Details will be discussed in the running CR.

In this contribution, we further discuss the criteria for RRM measurement relaxation and offloading for IDLE/INACTIVE mode.

- Discussion

Reducing the number of LP-WUS related thresholds

In the RAN2#129bis meeting, a concern was raised regarding the tremendous number of LP-WUS related thresholds including entry/exit condition for LP-WUS monitoring, serving cell measurement relaxation/offloading, and neighboring cell relaxation. This complexity may pose implementation challenges for LP-WUS. Consequently, RAN2 is deliberating on potential optimizations, such as merging the thresholds across different UE behaviors for LP-WUS operation.

One possible way to reduce the number of thresholds for LP-WUS is to merge the entry/exit condition for LP-WUS monitoring and RRM measurement relaxation (i.e., RAN4 CASE#3) or RRM measurement fully offloading (i.e., RAN4 CASE#1). We first discuss the UE behaviors for different cases of RRM measurement relaxation and LP-WUS monitoring as below.

Table 1. UE behaviors for different cases of RRM measurement relaxation and LP-WUS monitoring

| Cases |  |  | UE behaviors |
| --- | --- | --- | --- |
| No. | Criteria for RRM measurement relaxation | Criteria for LP-WUS monitoring |  |
| ① | Fulfilled | Fulfilled | UE can monitor LP-WUS, perform LR measurement and relaxed MR serving/neighbour cell measurement |
| ② | Not fulfilled | Not fulfilled | Legacy behaviors for paging reception and RRM measurements |
| ③ | Fulfilled | Not fulfilled | UE only performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS If UE can perform LR measurement, why cannot UE monitor LP-WUS? |
| ④ | Not fulfilled | Fulfilled | UE only monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (note that fully offloading criteria is not fulfilled either) There is no power saving gain for this case |

For case ④, it is not reasonable that UE enters LP-WUS coverage but still performs normal RRM measurement in which case there is no power saving gain, so case④ should not be supported. Then for case③, if the LR measurement can be used, then LP-WUS can also be used for further reducing power consumption for paging reception, otherwise, the thresholds for RRM measurement relaxation criteria and LP-WUS monitoring criteria are not properly set. Hence, it makes sense to only support case① and case②.

Observation 1: The following two cases should not be supported:

- UE performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS;

- UE monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (i.e., no RRM measurement relaxation).

RAN2 agreed that it is up to NW to configure either RRM measurement relaxation or RRM measurement fully offloading or both in one cell, we discuss based on the following scenarios:

Scenario 1: NW configures only RRM measurement relaxation (CASE#3)

If the network configures the entry/exit condition for only RRM measurement relaxation (CASE#3), then the it can be merged with the entry/exit condition for LP-WUS monitoring, i.e., if the entry condition of RRM measurement relaxation (CASE#3) is fulfilled, the UE starts LP-WUS monitoring, and performs RRM serving/neighbor cell measurement relaxation. If the exit condition of RRM measurement relaxation (CASE#3) is fulfilled, the UE starts legacy procedure by MR and performs legacy RRM measurement by MR.

Scenario 2: NW configures only RRM measurement fully offloading (CASE#1)

Similar to the above discussion on RRM measurement relaxation (CASE#3), if the network configures the entry/exit condition for only RRM measurement fully offloading (CASE#1), then it is not reasonable for UE to monitor LP-WUS but still perform legacy measurement, or to perform measurement only on LR without LP-WUS monitoring.

In this case, the entry/exit condition for RRM measurement fully offloading (CASE#1) can be reused for LP-WUS monitoring, which means if the entry condition of RRM measurement fully offloading (CASE#1) is fulfilled, the UE starts LP-WUS monitoring, and performs RRM serving cell offloading. If the exit condition of RRM measurement fully offloading (CASE#1) is fulfilled, the UE starts legacy procedure by MR and performs legacy RRM measurement by MR.

Scenario 3: NW configures both RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1)

If the network configures both RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1), for CASE#1, since neighbor cell of equal or lower priority (including intra-frequency, NR inter-frequency and inter-RAT frequency) is off, it means UE is not likely to perform cell re-selection. For example, UE is near the center of the cell. On the other hand, for CASE#3, UE may be near the cell edge so that neighboring cell measurement is on.

Observation 2: RAN4 CASE#1 is the RRM measurement behaviors for UEs in center of the cell and RAN4 CASE#3 is the RRM measurement behaviors for UEs near the edge of the LP-WUS coverage.

Based on above, how LP-WUR and MR perform RRM measurement is illustrated in the figure below.

Fig. 1. UE behaviors for RRM measurement fully offloading and relaxation cases

It is obvious that once both RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1) are configured, then UE shall start LP-WUS monitoring if the entry condition of RRM measurement relaxation (CASE#3) is fulfilled, and the UE starts legacy procedure by MR and performs legacy RRM measurement by MR if the exit condition of RRM measurement relaxation (CASE#3) is fulfilled.

From the discussion on the above 3 scenarios, we can conclude that:

Proposal 1: (RRC-8) RAN2 to agree on the following cases for merging the thresholds of RRM measurement relaxation (CASE#3) or RRM measurement fully offloading (CASE#1) with LP-WUS monitoring:

- If the network configures only RRM measurement relaxation (CASE#3), the UE can regard the entry/exit condition of RRM measurement relaxation (CASE#3) as the entry/exit condition of LP-WUS monitoring, regardless of whether the RRM measurement fully offloading (CASE#1) is configured.

- If the network configures only RRM measurement fully offloading (CASE#1), the UE can regard the entry/exit condition of RRM measurement fully offloading (CASE#1) as the entry/exit condition of LP-WUS monitoring.

The exit condition for RRM measurement relaxation

Regarding the exit condition of RRM measurement relaxation (CASE#3), a separate exit condition based on LR measurement is preferred. On the one hand, since the MR RRM measurement is relaxed and RAN4 agreed that the scaling factor for MR RRM relaxation is 16, MR measurement results cannot be obtained in a timely manner, which delays the exit of LP-WUS monitoring and may result in paging loss. On the other hand, with different conditions for exit and entry, the ping-pong issue can be avoided by NW implementation when configuring the conditions and there is no need to define additional hysteresis mechanism. Furthermore, it is also aligned with Proposal 1 above, since “exit condition based on LR measurement” was agreed for LP-WUS entry/exit condition.

Observation 3: Defining a separate exit condition based on LR measurement has the following advantages:

- the LR measurement results is more timely compared with MR measurement results, since RAN4 agreed that the scaling factor for MR RRM relaxation is 16;

- with different conditions for exit and entry, the ping-pong issue can be avoided by NW implementation when configuring the conditions and there is no need to define additional hysteresis mechanism.

Proposal 2: (38304-3/RRC-7) The exit condition for RRM measurement relaxation is based on the LR measurement, i.e., when the LR measurement is below a threshold.

The entry condition for RRM measurement fully offloading

In RAN2#127 meeting, RAN2 discussed the entry/exit condition for RRM measurement fully offloading (CASE#1) without taking CASE#3 (which was agreed in parallel by RAN4) and scaling factor for MR RRM relaxation (which was agreed by RAN4) into account. Currently, with both CASE#1 and CASE#3 configured by the network, RAN2 needs to further think about the entry/exit condition considering: switch between CASE#1 and CASE#3.

If UE is in the RRM measurement relaxation case, RAN4 agreed that the scaling factor for MR RRM relaxation is 16. In this case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading case. Thus, the serving cell quality measured by LR is more appropriate to be used for the entry condition of fully offloading.

Observation 4: In the RRM measurement relaxation case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading.

Proposal 3: If UE is currently in the RRM measurement relaxation case, serving cell measurement by LR is used for the entry condition of fully offloading.

RRM measurement with low mobility criterion

For the existing Rel-16 relaxed measurement rules, in addition to the criteria for not being at cell edge (based on the absolute serving cell quality), the criteria for low mobility (based on serving cell quality variation over time) was also defined. However, introducing mobility-based criteria would significantly increase the complexity of the current decision rules for entering/exiting both RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1), which are merely based on the serving cell quality.

Furthermore, implementing mobility criteria could potentially impact the availability of the LP-WUS function for high-mobility UEs, such as smartphones or in-vehicle devices. Rather than imposing network-defined mobility restrictions, we recommend allowing UE implementation to determine when to exit LP-WUS monitoring during high mobility state.

Based on these considerations, we propose to leave it to UE implementation to determine whether to exit LP-WUS monitoring when in high mobility state for CASE#1 and CASE#3.

Proposal 4: (38304-6/RRC-10) The low mobility criteria is not supported for the entry/exit condition of RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1).

Proposal 5: It is up to UE implementation to determine whether to exit LP-WUS monitoring when in high mobility state.

Impact on Rel-16 RRM relaxation criteria

In Rel-16, the criteria of RRM relaxation (i.e., not-at-cell-edge criterion and low mobility criterion) were introduced for UEs in Idle/Inactive mode, to enable UEs to perform RRM measurement at lower frequencies, thereby reducing power consumption. Companies are proposing to discuss whether the LR can be used to evaluate the Rel-16 RRM measurement relaxation criterion, if the criteria of LP-WUS monitoring and Rel-16 RRM relaxation are configured, but Rel-19 RRM relaxation is not configured.

Due to the tremendous transition energy, the requirement determined by Rel-16 measurement relaxation (e.g., scaling factor to be 3 for Rel-16 and 6 for Rel-17 as an enhancement) cannot provide any power saving gain for the UEs monitoring LP-WUS in Ultra Deep Sleep mode. Even for the UEs in Deep Sleep mode, the gain from using LR to perform Rel-16 measurement relaxation decision can also be almost negligible compared to existing power saving features such as Rel-17 PEI, while bringing the great complexity to both Rel-16 and Rel-19 RRM relaxation. Since there only remains one meeting for RAN2, discussing this direction will take more time. Thus, we prefer not to have any enhancement for the current Rel-16 RRM measurement relaxation.

Proposal 6: (RRC-15) No enhancement (e.g., LR to evaluate the Rel-16 “not-at-cell-edge” and “low mobility” criterion) is needed for the Rel-16 RRM measurement relaxation.

- Conclusion

In this contribution, we discuss the remaining issues on the criteria for RRM measurement relaxation and offloading for IDLE/INACTIVE mode, and have the following proposals:

Reducing the number of LP-WUS related thresholds

Observation 1: The following two cases should not be supported:

- UE performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS;

- UE monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (i.e., no RRM measurement relaxation).

Observation 2: RAN4 CASE#1 is the RRM measurement behaviors for UEs in center of the cell and RAN4 CASE#3 is the RRM measurement behaviors for UEs near the edge of the LP-WUS coverage

Proposal 1: (38304-4/RRC-8) RAN2 to agree on the following cases for merging the thresholds of RRM measurement relaxation (CASE#3) or RRM measurement fully offloading (CASE#1) with LP-WUS monitoring:

- If the network configures the RRM measurement relaxation (CASE#3), the UE can regard the entry/exit condition of RRM measurement relaxation (CASE#3) as the entry/exit condition of LP-WUS monitoring, regardless of whether the RRM measurement fully offloading (CASE#1) is configured.

- If the network ONLY configures the RRM measurement fully offloading (CASE#1), the UE can regard the entry/exit condition of RRM measurement fully offloading (CASE#1) as the entry/exit condition of LP-WUS monitoring.

The exit condition for RRM measurement relaxation

Observation 3: Defining a separate exit condition based on LR measurement has the following advantages:

- the LR measurement results is more timely compared with MR measurement results, since RAN4 agreed that the scaling factor for MR RRM relaxation is 16;

- with different conditions for exit and entry, the ping-pong issue can be avoided by NW implementation when configuring the conditions and there is no need to define additional hysteresis mechanism.

Proposal 2: (38304-3/RRC-7) The exit condition for RRM measurement relaxation is based on the LR measurement, i.e., when the LR measurement is below a threshold.

The entry condition for RRM measurement fully offloading

Observation 4: In the RRM measurement relaxation case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading.

Proposal 3: If UE is currently in the RRM measurement relaxation case, serving cell measurement by LR is used for the entry condition of fully offloading.

RRM measurement with low mobility criterion

Proposal 4: (38304-6/RRC-10) The low mobility criteria is not supported for the entry/exit condition of RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1).

Proposal 5: It is up to UE implementation to determine whether to exit LP-WUS monitoring when in high mobility state.

Impact on Rel-16 RRM relaxation criteria

Proposal 6: No enhancement (e.g., LR to evaluate the Rel-16 “not-at-cell-edge” and “low mobility” criterion) is needed for the Rel-16 RRM measurement relaxation.

- References

RAN2#129bis chairman notes.

3GPP TS 38.304 V16.11.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state