3GPP TSG-RAN WG2 Meeting #131	R2-2505749

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.5.3

Source:	OPPO

Title:	Consideration on on-demand SIB1

Document for:	Discussion, Decision

Introduction

This paper provides our views on the following issues for on-demand SIB1.

- Whether to support SUL in the OD-SIB1 cell

- UE behaviour upon reception of SIBxx(Samsung002)

- Issue 1: Q12

- Issue 2: vivo002

Discussion

SUL co-exist with OD-SIB1

In the legacy, the UE needs to select SUL or NUL during the RA procedure initialization.

1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier.

As RAN1 has agreed that SupplementaryUL is not included in the UL-WUS configuration and the current RRC running CR does not include rsrp-ThresholdSSB-SUL (for which the main reason is that R1 tends to see the typical case of OD-SIB1 is for the small cell, and thus SUL is not the target scenario for OD-SIB1), the UE should always select NUL when the UE requests SIB1 of an NES cell, by assuming the network would not configure SUL for the NES cell of OD-SIB1 feature. And it is also aligned with the existing MAC specification, i.e., it is handled as a case where rsrp-ThresholdSSB-SUL is not available and thus the else-branch for NUL is adopted.

For OD-SIB1, based on R1 conclusion, no specific handling for SUL is needed.

UE behaviour upon reception of SIBxx (Samsung002)

In the current RRC running CR, the UE behavior upon reception of SIBxx is depicted as below.

- 5.2.2.4.2x Actions upon reception of SIBxx Upon receiving SIBxx, the UE shall: 1> if the UE has reselected to a cell providing OD-SIB1, the UE stores the configuration for SIB1 request for this cell and considers it valid while camping in this cell:

The text is not super clear in terms of which cell the term is referring to:

- Interpretation-1: the cell where the SIBxx is obtained;

- Interpretation-2: the cell where the UE has reselected to;

There are some problems either way, e.g.,

1) restrict the usage of the configuration too much, since obviously the configuration can be used during a reselection attempt to another cell, while

2) is not always true either, since the SIBxx of the reselected cell may not of the same area ID and value tag, and thus UE has to re-acquire the SIBxx from the cell (i.e., the stored version is not valid any more).

In our understanding, RAN2 has already agreed to apply the legacy validity rule to SIBxx.

The UE follows the legacy validity principle of stored SIB.

It means that the UE uses the stored SIBxx after cell reselection if the stored SIBxx is still valid; otherwise, the UE acquires and uses a new SIBxx acquired from the current camping cell.

Thus, we suggest some clarification for the current version:

Upon receiving SIBxx from a cell, the UE shall: - store the SIBxx; - apply the SIB1 request configuration for acquiring OD-SIB1 of this cell; - apply the SIB1 request configuration of another cell in this stored SIBxx for acquiring OD-SIB during reselection to that cell, and after reselection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1;

Where

- The yellow part clarifies that the SIBxx is valid at least for the cell where the SIBx was acquired;

- The green part clarifies that the SIBxx may or may not be used for the reselected cell, i.e., the validity check, depending on the area ID and value tag, as in legacy.

Update Clause 5.2.2.4.2x in the RRC running CR to reflect the following UE behaviours upon reception of SIBxx.

- store the SIBxx;

- apply the SIB1 request configuration for acquiring OD-SIB1 of this cell;

- apply the SIB1 request configuration of another cell in this stored SIBxx for acquiring OD-SIB during reselection to that cell, and after reselection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1;

Issue 1: Q12

In the post email discussion, companies have discussed how to reflect the connected UE behaviour to resolve the FFS issue below.

Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

During the discussion, the Rapporteur suggested adding in the field description of si-BroadcastStatus with a sentence of “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1”, whereas companies had diverse views. We, sharing a similar view as many companies, agree with the Rapporteur’s intention, but now understand that it is more accurate to apply this connected UE behaviour only when SIB1 of the serving cell is being transmitted on demand.

Add “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1 if the associated SIB1 is being transmitted on-demand” in the field description of si-BroadcastStatus.

Issue 2: vivo002

Regarding the RRC running CR, the following issue was raised and suggested for further discussion.

| vivo002 | 5.2.2.3.1 Acquisition of MIB and SIB1 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED while T311 is running: 5> perform the actions as specified in clause 5.2.2.3.3x; 3> else: 4> perform the actions as specified in clause 5.2.2.5. NOTE 1: The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located. NOTE 2: UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]). NOTE x: It is up to UE’s implementation to check if SIB1 is currently being broadcasted for that cell before triggering OD-SIB1 request procedure of that cell. [comment] During last meeting, at offline session [AT130][104][NES] (Ericsson), P6 in R2-2504037 about the UE behavior after receiving SI change notifcation was discussed. Rapporteur responded that a condition was added to address the issue: 5> if SIB1 is not broadcasted 6> perform the actions as specified in clause 5.2.2.3.3x; And therefore, at the Comeback session, it was first agreed that: Keep RRC CR as it is for the short message and UE behaviour.. Then, it is commented that the high-lighted part mandates the UEs to check whether SIB1 is broadcasting before requesting OD-SIB1 (not the case upon receiving SI change notification), which is against the RAN1 agreement that it is up to the UE’s implementation to check if SIB1 is broadcasting. Thus, the high-lighted part was removed, and instead the NOTE x is added. However, RAN2#126 agrees that: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.. Therefore, we encourage RAN2 to review the agreement again ‘Keep RRC CR as it is for the short message and UE behaviour.’, whether the NOTE x can cover the above RAN2#126 agreement. The discussion is comprised of the following questions: Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update? [HONOR] For Q1, we understand that this agreement simply shows the UE can acquire the updated SIB1 from the NES cell without returning to Cell A. It does not imply that the NES cell will immediately transmit CD-SSB for SIB1 update upon broadcasting a SIB change notification (e.g., at the start of the next MP). Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this? [HONOR] For Q2, we think the UE camping on the NES cell can initiate OD-SIB1 request based on the WUS configuration to acquire the updated SIB1. We think for Q2, if the CORESET#0 in the stored OD-SIB1 request configuration has changed, the UE camping on the NES cell will have no way by implementation to receive the updated SIB1. As a result, all the UEs camping on the NES cell will trigger OD-SIB1 request since SIB1 cannot be received, which is not correct. Therefore, we propose Q1/Q2 to be listed as open issues for further discussion. [HONOR] Same view as Vivo, i.e., add Q1, Q2 and the issue of WUS configuration changes caused by CORESET#0 modifications to open issues for further study. | Listed as open issue as requested since it can be seen there would be conflicting agreements. |
| --- | --- | --- |

The issue mainly concerns whether the NOTE x in clause 5.2.2.3.1 can cover the previous RAN2 agreement on receiving the SI change notification, since, as mentioned by Honor above, the intention of the agreement was to avoid UE fallback to cell-A for SIB acquisition.

Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

The Q1 and Q2 eventually relate to whether the Kssb in MIB will change or not, for which we see some issues either way:

- If Kssb value is change to <= 24/12, it may allow some legacy UE to camp in the cell, which is OK but then the side effect is that afterwards when Kssb is changed back to > 24/12, those legacy UE has to leave and thus another cell reselection wave triggered by short message is necessary, and thus causing unnecessary signaling and load re-distribution in the network. Furthermore, Kssb content change may cause a problem to the RRC_CONNECTED UE which may rely on its content for synchronization.

- If Kssb value is not changed, the CoreSet#0 content cannot be changed, as mentioned by vivo above.

However, we tend to believe this is more of an R1 issue; a similar discussion of Kssb value change has already happened in R1, when discussing the scenario of OD-SIB1 request (although the scenario here is short message-based SIB content change). So interested company should raise this issue in R1 rather than in R2.

For Vivo002, R2 does not pursue further specification effort for it, and interested companies can raise it in R1.

Conclusion

We have the following proposals:

Proposal 1	For OD-SIB1, based on R1 conclusion, no specific handling for SUL is needed.

Proposal 2	Update Clause 5.2.2.4.2x in the RRC running CR to reflect the following UE behaviours upon reception of SIBxx.

-	store the SIBxx;

-	apply the SIB1 request configuration for acquiring OD-SIB1 of this cell;

-	apply the SIB1 request configuration of another cell in this stored SIBxx for acquiring OD-SIB during reselection to that cell, and after reselection to that cell if the stored SIBxx is a valid version for that cell in accordance with clause 5.2.2.2.1;

Proposal 3	Add “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1 if the associated SIB1 is being transmitted on-demand” in the field description of si-BroadcastStatus.

Proposal 4	For Vivo002, R2 does not pursue further specification effort for it, and interested companies can raise it in R1.

Reference

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR