3GPP TSG RAN WG2#131	R2-2505752

Bengaluru, India, 25th - 29th August 2025

Agenda item:	8.4.2

Source:	Nokia

Title:	LP-WUS in IDLE and INACTIVE

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a new work item (RP-240801) was agreed for Low Power Wake Up Signal and Receivers designs. These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and receiver architectures.

In this contribution we focus on the remaining issues for the following objective:

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# 2	Discussion

RAN2 agreed the following:

RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

There is a common understanding that the UE may not support LP-WUS on all the bands the UE support. UE may be camping on the cell on the band where LP-WUS is not supported by the UE/NW, although cell supporting LP-WUS maybe available on the other band.  In this case LP-WUS UE power saving gains would be lost. Since it was agreed that any overload issues can be addressed by current mechanism in spec it would be beneficial to allow the UE to camp on the cells supporting LP-WUS more often to save battery.

Proposal 1: Cell (re)selection rules are enhanced so that UE supporting LP-WUS prioritizes bands supporting LP-WUS

It was agreed in the previous meeting that it is up to UE implementation to choose whether SSB measurement based or OOK LP-SS measurement-based conditions are used for LP-WUS monitoring entry/exit condition, if UE support both measurement types. However, in this case it would seem preferable that if UE is capable to do measurements based on the SSB (i.e. supports OFDM-based WUR), UE would apply the thresholds based on SSB. As the accuracy expected for the SSB-based measurements in better (as per RAN4 agreements ±3.5dB for SSB-based versus ≥±5.5dB for LP-SS based), network can be less conservative when setting the SSB-based threshold, hence enabling UE to use LP-WUS on wider area and benefit more for the power saving.

Proposal 2: Consider reverting RAN2 agreement to: If both LP-SS based and SSB based thresholds are configured, OFDM-based WUR applies the SSB-based threshold (and metric) for evaluation.

Small data transmission (SDT) was specified in R17 and R18. SDT is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT can be initiated either by the UE in case of MO-SDT (Mobile Originated SDT) or by the network in case of MT-SDT (Mobile Terminated SDT). MO-SDT is initiated by the UE only if less than or equal to a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available. MT-SDT is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for radio bearers configured for SDT; based on the indication, the UE initiates the MT-SDT only if the DL RSRP is above a configured threshold. SDT was specified also for power saving purposes like LP-WUS and therefore it can be assumed that it may be beneficial to implement both SDT and LP-WUS features in the same device.

The following is capture in the TS 38.304 running CR:

##### 5.2.4.x.3 Serving cell measurement offloading rules LP-WUS UE may choose to perform serving cell measurement offloading (i.e., serving cell measurement is fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled.

If the UE evaluates conditions for SDT initiation when LP-WUS UE is performing serving cell measurement offloading, then MR RSRP measurement is not available since measurements are performed by LR. It needs to be decided whether LP-WUS UE can use RSRP measurement for SDT condition evaluation in this case. Also, MR RSRP measurement can be considered for this case, but using it causes delay (2 times DRX cycle), since RSRP needs to be measured according to RAN4 requirements.

Proposal 3: The UE supporting LP-WUS may support MO-SDT and/or MT-SDT.

Proposal 4: The UE supporting LP-WUS uses LR RSRP measurement for MO/MT SDT initiation condition evaluation when the evaluation is started while the UE is performing serving cell measurement offloading

# 3	Conclusion

The following is proposed based on the discussion:

Proposal 1: Cell (re)selection rules are enhanced so that UE supporting LP-WUS prioritizes bands supporting LP-WUS

Proposal 2: Consider reverting RAN2 agreement to: If both LP-SS based and SSB based thresholds are configured, OFDM-based WUR applies the SSB-based threshold (and metric) for evaluation.

Proposal 3: LP-WUS UE may support MO-SDT and/or MT-SDT.

Proposal 4: The UE supporting LP-WUS uses LR RSRP measurement for MO/MT SDT initiation condition evaluation when the evaluation is started while the UE is performing serving cell measurement offloading

# 4	References

[1] RP-243266 Way forward on Rel-19 LP-WUS separate band issues RAN2 chair, Interdigital

[2] RP-242656 On LR and MR operating in different bands in release 19, Vodafone, Vivo, Huawei, HiSilicon