3GPP TSG-RAN WG2 Meeting #131	R2-2505754

# Bangalore, India, 25th - 29th August 2025

Agenda item:	8.10.2

Source:	Samsung

Title:	Correlation Indication and other issues of CHO with Candidate SCG(s)

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the remaining issues for CHO with Candidate SCG(s).

# Discussion

## Correlation Indication SCG(s)

In the meeting RAN2#130 [1], we agreed the following.

- We will not add any optimizations to avoid duplicated info in correlated reports

- FFS if we add some correlation indication.

This was discussed briefly in the post meeting offline discussion [2] and the following proposal is made.

- [Open issue RRC-1] For correlation of SHR and SHR generated for a CHO with Candidate SCG, RAN2 discuss and agree to one of the following options

  - Correlation indication in RRC_Complete message (a flag beside availability indication)

  - Correlation indication inside the SHR and SPR

  - Logging C-RNTI of PCell and timeSinceSPR in SPR

  - The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

For CHO with candidate SCG(s), UE may receive the condRRCReconfig along with  condExecutionCond  and condExecutionCondPSCell.

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]],

[[

condExecutionCondPSCell-r18      SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigCHO-WithSCG

subsequentCondReconfig-r18       SubsequentCondReconfig-r18                            OPTIONAL,    -- Need M

securityCellSetId-r18            SecurityCellSetId-r18                                 OPTIONAL,    -- Need M

scpac-ConfigComplete-r18         ENUMERATED {true}                                     OPTIONAL     -- Cond CPAC

]]

}

When the condition in the condExecutionCond  and condExecutionCondPSCell are fulfilled, UE executes the RRC Reconfiguration  in condRRCReconfig. This means that the applied RRC Reconfiguration at the time of execution of CHO with candidate SCG(s) are received at the same time.

According to the latest running CR, In SHR, UE logs timeSinceCHO-Reconfig to indicate the time elapsed between the reception of last applied conditional reconfiguration and the generation of SHR. Similarly, the UE logs timeSinceCPAC-Reconfig to indicate the time elapsed between the reception of last applied conditional reconfiguration and the generation of SPR. If the SHR and SPR are generated due to the execution of same CHO with candidate SCG(s), both these fields will be same.

Observation 1: If the SHR and SPR are generated due to the execution of same CHO with candidate SCG(s), timeSinceCHO-Reconfig in SHR and timeSinceCPAC-Reconfig in SPR will be the same.

Similarly, for CHO with candidate SCGs, In SHR, UE logs source cell measurements (sourceCellMeas) corresponding to the PCell and source PSCell measurements corresponding to the PSCell. In SPR, UE logs source cell measurements of the PSCell and source PCell measurements of the PCell. If SHR and SPR are at the same time, these measurement will be same. We think this information is sufficient for correlation.

Even for PCell change which simultaneously changes PSCell (non CHO with candidate SCGs), the correlation information is available, as the UE logs the same measurements configured by MN in SHR and SPR.

Observation 2: If the SHR and SPR are generated due to the execution of same CHO with candidate SCG(s), there will be same measurements along with the same cell identifiers in SHR and SPR.

Based on the above observations, it is clear that there is sufficient information in the SHR and SPR for the correlation. Thus we propose that

Proposal 1: [Open issue RRC-1] For correlation of SHR and SHR generated for a CHO with Candidate SCG, RAN2 to agree option d (The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation).

## Other issues in RRC CR

In RAN2#129, the following agreement was made:

Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

RAN3 also made the following agreement in RAN3#127bis

Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

Based on these agreements, additional information is logged in SCGFailureInformation as below in the running CR. We observe that the pCellId and psCellId are logged for all the candidates, while the agreement is for the candidates that fulfilled execution conditions.

1>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows:

2>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

3>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first;

3>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

2>	else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList is fulfilled:

3>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled;

3>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the SCG failure;

2>	set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

2>	set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;

Since the information in the SCGFailureInformation is not a stored information, the network is well aware of the CHO with Candidate SCG configuration. It is also noted that it is important not to include not required information in the SCGFailureInformation as this may indeed delay the recovery for SCG. Thus it is important to log the information only for the agreed cases.

Proposal 2: Update the RRC Spec so that the pCellId and psCellId are logged in choWithCandidateSCGInfoList for SCGFailureInformation, only for the candidates with fulfilled execution conditions as below.

- for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell,where all triggering events of any of condExecutionCond and condExecutionCondPSCell are fulfilled include an entry in choWithCandidateSCGInfoList and set the values as follows

# Conclusion

Based on the discussion, we have the following observation and proposals.

Observation 1: If the SHR and SPR are generated due to the execution of same CHO with candidate SCG(s), timeSinceCHO-Reconfig in SHR and timeSinceCPAC-Reconfig in SPR will be the same.

Observation 2: If the SHR and SPR are generated due to the execution of same CHO with candidate SCG(s), there will be same measurements along with the same cell identifiers in SHR and SPR.

Proposal 1: [Open issue RRC-1] For correlation of SHR and SHR generated for a CHO with Candidate SCG, RAN2 to agree option d (The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation).

Proposal 2: Update the RRC Spec so that the pCellId and psCellId are logged in choWithCandidateSCGInfoList for SCGFailureInformation only for the candidates with fulfilled execution conditions as below.

- for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell,where all triggering events of any of condExecutionCond and condExecutionCondPSCell are fulfilled include an entry in choWithCandidateSCGInfoList and set the values as follows

# References

[1] RAN2#130 meeting report