3GPP RAN WG2 Meeting #131	R2-2505763

Bengaluru, India, August 25th – 29th 2025

Agenda Item:	8.1.2.3

Source:	InterDigital

Title:	Remaining open issues: LCM for Positioning use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback.

A list of remaining open issues has been provided as part of a post meeting TS 37.355 running CR review [2]. This contribution discusses these remaining issues.

# Remaining open issues

The following issues and descriptions have been extracted from the Open issues list as part of the TS 37.355 running CR review.

## [LPP-6a] Number of PRUs in NR-PRU-DL-Info

In RAN2#130 the following was agreed and subsequently included in the running CR:

[LPP-6] The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed. FFS if more PRUs are needed to be included for training purposes.

The FFS in the agreement above is captured as new issue LPP-6a. As described by the Rapporteur, the IE NR-PRU-DL-Info currently provides PRU measurements for a single PRU. However, the issue was raised at RAN2#130 whether more PRUs are needed for training purposes (i.e., to speed up training). One option is to introduce an indication to request additional PRUs (e.g., a number of PRUs) in the Request Assistance Data message.

If more PRUs are needed an LMF can already provide multiple Provide Assistance Data messages based on existing specification, possibly in response to multiple Request Assitance Data messages. Considering the use case is limited to training (not inference) and the existing specification can already support the reception of multiple PRUs, there seems to be limited motivation to introduce additional signaling impact.

Proposal 1:	[LPP-6a] Do not introduce a request for additional PRUs (e.g., a number of PRUs) in the Request Assistance Data message.

## [LPP-19] Applicability of BM related agreements

RAN2 intention is to have the same design for BM case and positioning case as much as possible. It is currently open whether the following agreements made for BM applicability reporting apply to positioning case 1, and whether there are any associated LPP impacts related to their adoption:

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

According to positioning LCM procedure, in Step 3 the NW may provide the NW side additional conditions via the LPP Provide Assistance message. Other information such as UE-side additional conditions and model availability in device are already known to the UE. The BM agreement on information used for applicability determination can therefore be extended to positioning with minimal (or no) additional effort since all the existing procedure already supports providing the necessary information.

Proposal 2:	[LPP-19] For POS use case, UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

The second agreement clarifies what is reported in the initial and subsequent applicability report when a UE is configured with multiple functionalities. The BM discussion concluded that a UE reports the explicit (in)applicability status of all configured functionalities (instead of only those which are applicable) in the initial report for clarity and simplicity. If a subsequent report is triggered by an applicability change the UE only reports those functionalit(ies) whose status has changed. This avoids redundantly including all other functionalities whose applicability status hasn’t and improves signalling overhead.

Similar arguments may be made for the positioning use case, so it seems easiest to simply adopt the same agreements from BM on initial and subsequent reporting format. As it is already agreed that the UE reports both the initial and change of applicable functionality to the LMF via the LPP provide capabilities message, any impacts to LPP signalling should be minimal.

Proposal 3a:	[LPP-19] For POS use case, UE explicitly reports the (in)applicability of all configured functionalities in initial applicability report.

Proposal 3b:	[LPP-19] For POS use case, UE reports only the functionality(ies) where applicability has changed in subsequent applicability report.

## [LPP-21] "Associated ID" for TRP Location Coordinates

The open issues list describes several issues regarding the use of Associated ID in the LPP specification. One such issue is whether the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both?

Providing the explicit TRP coordinates provides an accurate and clear description of the TRP location. However, there may be several reasons (e.g., security) why the network may choose not to do this, which is the purpose of introducing NR-TRP-ImplicitLocationInfo-r19.  The NW will provide explicit location when it can provide it, and if it wants to hide TRP location, it will provide implicit information. Additionally, implicit location info (associated ID) is used only to implicitly indicate consistency of TRP locations; no other NW conditions are implied by the associated ID in AIML POS. As there seems little motivation (at least from a network point of view) to provide both NR-TRP-LocationInfo-r16 and NR-TRP-ImplicitLocationInfo-r19, they should be mutually exclusive.

Proposal 4:	[LPP-21] NR-TRP-LocationInfo-r16 and NR-TRP-ImplicitLocationInfo-r19 are mutually exclusive.

Similarly, it is also open whether a UE supporting explicit TRP location info (NR-TRP-LocationInfo-r16), must also support implicit location info (nr-TRP-ImplicitLocationInfo-r19) (and vice versa). Support for both explicit and implicit location info may be useful if a general AIML model can use both as input. This however may not always be the case (e.g., a model may only use explicit TRP locations) and support for both should not be mandated.

Proposal 5:	[LPP-21] A UE is not mandated to support both explicit and implicit location information.

# Conclusion

In this contribution the following observations and proposals are made regarding open issues for AIML  Positioning use case:

Proposal 1:	[LPP-6a] Do not introduce a request for additional PRUs (e.g., a number of PRUs) in the Request Assistance Data message.

Proposal 2:	[LPP-19] For POS use case, UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

Proposal 3a:	[LPP-19] For POS use case, UE explicitly reports the (in)applicability of all configured functionalities in initial applicability report.

Proposal 3b:	[LPP-19] For POS use case, UE reports only the functionality(ies) where applicability has changed in subsequent applicability report.

Proposal 4:	[LPP-21] NR-TRP-LocationInfo-r16 and NR-TRP-ImplicitLocationInfo-r19 are mutually exclusive.

Proposal 5:	[LPP-21] A UE is not mandated to support both explicit and implicit location information.

# References

RP-243244 – WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface – Qualcomm

[POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm)