3GPP TSG-RAN WG2 Meeting #131	R2-2505765

Bengaluru, India, August 25-29, 2025

Agenda item:	8.1.2.3

Source: 	Qualcomm Incorporated

Title: 	Remaining LPP open issues for "AI/ML Positioning Case 1"

Document for:		Discussion and Decision

# 1.	Introduction

In this contribution we discuss the remaining issues for the LPP support of "AI/ML Positioning Case 1".

The following LPP Open Issues according to [1] are discussed in this contribution:

-	LPP-6a: Number of PRUs in NR-PRU-DL-Info

-	LPP-10a: Applicability of "batch reporting" for AI/ML positioning

-	LPP-18: Consistency between training and inference

-	LPP-21: "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19)

# 2.	LPP Open Issues

## 2.1	LPP-6a: Number of PRUs in NR-PRU-DL-Info

At RAN2#130, the following agreement was made [2]:

The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed. FFS if more PRUs are needed to be included for training purposes.

PRU measurements could be useful for model training and model performance monitoring. The key property of the PRU is that a PRU has a known (and possibly fixed) location. Therefore, a "ground-truth label" may be accurately known. The PRU measurements supported in NR-PRU-DL-Info are the existing NR positioning measurements (e.g., RSTD, multipath profile (NR-AdditionalPathList), LoS/NLoS indicator, RSRP, RSRPP, etc.). Although, the model input for Case 1 is implementation specific, machine learning models for Case 1 may be trained or monitored with any of the existing NR positioning measurements, possibly together with implementation specific additions or variations. However, similar to the issue in section 2.3 below ("consistency between training and inference"), there are the following limitations for Case 1:

-	The PRU(s) for which the NR-PRU-DL-Info is provided is selected by the LMF. However, the PRU selection depends on the use case. For Case 1, a different PRU may be more suitable or desired than for e.g., carrier phase positioning.

-	There is only a single bit in the Request Assistance Data message to request the NR-PRU-DL-Info (bit 'pruInfo' in nr-PositionCalculationAssistanceReq). Therefore, an LMF would decide which PRU measurements are provided without any UE assistance (i.e., NR-DL-TDOA-SignalMeasurementInformation, NR-DL-AoD-SignalMeasurementInformation, and/or NR-PRU-RSCP-MeasurementInformation). However, for Case 1, a target UE may only be interested in e.g., NR-DL-TDOA-SignalMeasurementInformation (which could also include the NR-AdditionalPathList) in NR-PRU-DL-Info.

Since the NR-PRU-DL-Info is used for multiple purposes (e.g., carrier phase positioning, DL AI/ML positioning), the Request Assistance Data should allow at least for the following:

-	Indication of which PRU measurements are desired (at least on SignalMeasurementInformation level (i.e., whether NR-DL-TDOA-SignalMeasurementInformation, NR-DL-AoD-SignalMeasurementInformation, NR-PRU-RSCP-MeasurementInformation are desired/requested).

-	Number of PRUs desired and desired PRU location region. A target UE may be interested of "ground-truth labels" in a specific area only for which training data and/or monitoring is needed. This could be for example a geographic region or the DL-PRS IDs possibly together with PCI/NCGI for which the SignalMeasurementInformation is desired (i.e., the LMF would then select a PRU which can measure at least one of the desired DL-PRS-IDs, e.g., as the "RSTD reference" TRP).

An example is shown below.

-- ASN1START

NR-PRU-DL-InfoRequest-r19 ::= SEQUENCE {

nr-PRU-MeasInfoRequest-r19		BIT STRING {

nr-DL-TDOA-SignalMeasurementInformation	(0),

nr-DL-AoD-SignalMeasurementInformation	(1),

nr-PRU-RSCP-MeasurementInformation		(2)

} (SIZE (1..8)),

nr-PRU-Region-r19				CHOICE {

geographicalArea	Polygon,

cellArea			CellAreaList-r19,

...

}										OPTIONAL,

numberOfPRUsRequest-r19			INTEGER (0..255),

...

}

CellAreaList-r19 ::= SEQUENCE (SIZE(1..maxCellIDsPerArea-r17)) OF NR-DL-PRS-IDs-r19

NR-DL-PRS-IDs-r19 ::= SEQUENCE {

dl-PRS-ID-r19					INTEGER (0..255)			OPTIONAL,

nr-PhysCellID-r19				NR-PhysCellID-r16			OPTIONAL,

nr-CellGlobalID-r19				NCGI-r15					OPTIONAL,

nr-ARFCN-r19					ARFCN-ValueNR-r15			OPTIONAL,

...

}

-- ASN1STOP

NR-PRU-DL-InfoRequest field descriptions

nr-PRU-MeasInfoRequest This field specifies the PRU measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular PRU measurement is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-PRU-DL-TDOA-MeasInfo in IE NR-PRU-DL-Info is requested or not; - bit 1 indicates whether the field nr-PRU-DL-AoD-MeasInfo in IE NR-PRU-DL-Info is requested or not; - bit 2 indicates whether the field nr-PRU-RSCP-MeasInfo in IE NR-PRU-DL-Info is requested or not.

nr-PRU-Region This field specifies a desired region where the PRUs should be located for which the PRU measurements are requested. - geographicalArea indicates a geographical area in which the PRUs should be located. - cellArea indicates the DL-PRS IDs and/or Cell IDs in which the PRUs should be located.

numberOfPRUsRequest This field specifies the desired number of different PRUs for which the PRU measurements are requested.

Proposal 1 (LPP-6a):	The assistance data request for PRU measurements should allow for the following:

-	Indication of which PRU measurements are desired (at least on SignalMeasurementInformation level (i.e., whether NR-DL-TDOA-SignalMeasurementInformation, NR-DL-AoD-SignalMeasurementInformation, NR-PRU-RSCP-MeasurementInformation are desired/requested).

-	Indication of a geographic area or DL-PRS/Cell ID list where a desired number of PRUs should be located for which the measurements are requested.

## 2.2	LPP-10a: Applicability of "batch reporting" for AI/ML positioning

"Batch-reporting", i.e., reporting of up to 32 location results in a single report, has been introduced in Rel-17 for the NR positioning methods. Multiple sets of positioning measurements or results are collected over time and then reported together in a single message rather than being sent individually. The location request can include the multiMeasInSameReport request flag [3] to indicate to the target UE to provide multiple measurement instances in a single measurement report (multiple location measurements (in case of UE-assisted) or multiple location estimates (in case of UE-based)). For example, instead of sending a location update every second, the UE might collect 10 location estimates over 10 seconds and send them all at once in a batch (a kind of "logged result reporting"). This enables for example motion tracking and other behavioral analysis but with reduced signalling. The corresponding capabilities (multiMeasInSameMeasReport, multiLocationEstimateInSameMeasReport) can be included in the LPP Provide Capability messages for each NR positioning method [3].

Since the "NR AI/ML Positioning Case 1" also falls into the category of NR positioning methods (e.g., measurements are based on NR DL-PRS), it seems consequent that batch reporting is also applicable to NR AI/ML positioning. When multiple NR positioning methods are requested (e.g., "hybrid positioning" or "fallback"), it may be inconsistent if for example DL-TDOA batch-reporting is requested e.g., as "fallback", but for "NR AI/ML Positioning Case 1" not. I.e., a LMF may be interested in multiple location estimates regardless of which NR positioning method is being used.

Proposal 2 (LPP-10a):	"Batch reporting", i.e., reporting of up to 32 location results in a single report as supported for the current NR positioning methods, is also applicable to "NR AI/ML Positioning Case 1".

## 2.3	LPP-18: Consistency between training and inference

At RAN2#130, contribution R2-2503403 "Discussion on consistency between training and inference for AI POS", vivo et al. [4] was discussed which resulted in the following agreement:

To ensure the consistency between training and inference, the UE should be able to request assistance data associated with a specific group of TRPs. FFS the request associated information in on demand prs request.

For DL AI/ML positioning, the on-demand DL-PRS request is included in the IE NR-DL-AIML-RequestAssistanceData using the Rel-17 NR-On-Demand-DL-PRS-Request IE [5]. This IE can request a predefined DL-PRS configuration previously provided by the LMF in the assistance data, or specific DL-PRS parameter like bandwidth, periodicity, etc. However, the TRPs for which the on-demand DL-PRS request is applicable will be selected by the LMF. To ensure consistency between training and inference, the group of TRPs for which the on-demand DL-PRS request is applicable should be included in the request.

For DL-PRS, a TRP is identified by the DL-PRS ID, possibly together with physical and/or global cell-IDs to avoid any potential ambiguities. The list of these IDs in all NR assistance data is per frequency layer, which can be included in the IE NR-On-Demand-DL-PRS-Request, as shown below:

-- ASN1START

NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE {

dl-prs-StartTime-and-Duration-r17			DL-PRS-StartTime-and-Duration-r17		OPTIONAL,

nr-on-demand-DL-PRS-Information-r17			NR-On-Demand-DL-PRS-Information-r17		OPTIONAL,

dl-prs-configuration-id-PrefList-r17		SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF 
													DL-PRS-Configuration-ID-r17			OPTIONAL,

...,

[[

dl-PRS-AggregationID-PrefList-r18			SEQUENCE (SIZE (1.. maxOD-DL-PRS-Configs-r17)) OF

INTEGER (1.. maxOD-DL-PRS-Configs-r17)

OPTIONAL,

nr-OnDemandDL-PRS-AggregationReqList-r18	SEQUENCE (SIZE (1.. maxOD-DL-PRS-Configs-r17)) OF

NR-OnDemandDL-PRS-AggregationReqElement-r18

OPTIONAL

]],

[[

nr-TRP-RequestList-r19 						NR-TRP-RequestList-r19					OPTIONAL

]]

}

DL-PRS-StartTime-and-Duration-r17 ::= SEQUENCE {

dl-prs-start-time-r17		INTEGER (1..1024)						OPTIONAL,

dl-prs-duration-r17			SEQUENCE {

seconds-r17		INTEGER (0..59)		OPTIONAL,

minutes-r17		INTEGER (0..59)		OPTIONAL,

hours-r17		INTEGER (0..23)		OPTIONAL,

...

}										OPTIONAL,

...

}

NR-OnDemandDL-PRS-AggregationReqElement-r18 ::= SEQUENCE (SIZE (2..3)) OF

INTEGER (1..nrMaxFreqLayers-r16)

NR-TRP-RequestList-r19 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-TRP-RequestListPerFreqLayer-r19

NR-TRP-RequestListPerFreqLayer-r19 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

TRP-RequestInfoElement-r19

TRP-RequestInfoElement-r19 ::= SEQUENCE {

dl-PRS-ID-r19					INTEGER (0..255)			OPTIONAL,

nr-PhysCellID-r19				NR-PhysCellID-r16			OPTIONAL,

nr-CellGlobalID-r19				NCGI-r15					OPTIONAL,

nr-ARFCN-r19					ARFCN-ValueNR-r15			OPTIONAL,

...

}

-- ASN1STOP

NR-On-Demand-DL-PRS-Request field descriptions

dl-prs-StartTime-and-Duration This field specifies the requested start time and duration for the on-demand DL-PRS and comprises the following subfields: - dl-prs-start-time specifies the desired start time for the requested DL-PRS. It indicates the time in seconds from the time the IE NR-On-Demand-DL-PRS-Request was received. - dl-prs-duration specifies the desired duration of the requested DL-PRS. The desired duration is the sum of the seconds, minutes, hours fields. If this field is included, at least one of the seconds, minutes, hours fields shall be present.

nr-on-demand-DL-PRS-Information This field specifies the on-demand DL-PRS configuration information requested by the target device. NOTE: If the network provided predefined on-demand DL-PRS configurations (NR-On-Demand-DL-PRS-Configurations), the target device can only request explicit parameters (nr-on-demand-DL-PRS-Information) within the scope of those configurations.

dl-prs-configuration-id-PrefList This field specifies the on-demand DL-PRS configuration associated with DL-PRS-Configuration-ID in IE NR-On-Demand-DL-PRS-Configurations the target device wishes to obtain in the order of preference. The first DL-PRS-Configuration-ID in the list is the most preferred configuration, the second DL-PRS-Configuration-ID the second most preferred, etc.

dl-PRS-AggregationID-PrefList This field specifies a list of identities i.e., DL-PRS-Configuration-ID, for On-demand DL-PRS Configuration information i.e., On-Demand-DL-PRS-Configuration, that the target device wishes to obtain, for DL-PRS aggregation, in the order of preference. The first integer value in the list is the most preferred On-demand DL-PRS Configuration information; the second integer value in the list is the second most preferred, etc.

nr-OnDemandDL-PRS-AggregationReqList This field specifies a list of DL-PRS for specific PFL combinations for which the DL-PRS information i.e., NR-On-Demand-DL-PRS-Information, is requested by the target device for DL-PRS aggregation, listed in the order of preference. The first NR-OnDemandDL-PRS-AggregationReqElement in the list is the most preferred PFL combination for DL-PRS aggregation; the second element in the list is the second most preferred, etc.

nr-TRP-RequestList This field specifies a list of TRPs for which the NR-On-Demand-DL-PRS-Request is desired and comprises the following subfields: - dl-PRS-ID specifies the DL-PRS ID of the TRP for which the on-demand DL-PRS is requested. - nr-PhysCellID specifies the physical Cell-ID of the TRP for which the on-demand DL-PRS is requested. - nr-CellGlobalID specifies the NCGI, the globally unique identity of a cell in NR, of the TRP for which the on-demand DL-PRS is requested, as defined in TS 38.331 [35]. - nr-ARFCN specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

Proposal 3 (LPP-18):	The IE NR-On-Demand-DL-PRS-Request can include the DL-PRS ID, Physical Cell ID/ARFCN, and/or NCGI per TRP per frequency layer for which the on-demand DL-PRS request is applicable/desired.

## 2.4	LPP-21: "Associated ID" for TRP Location Coordinates (IE NR-TRP-LocationInfo-Implicit)

RAN1 made the following general agreement at RAN1#121 [2]:

Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

The LPP Running CR discussion discovered a lack of clarity [6]:

(1)	Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell?

(2)	Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs?

(3)	Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"?

(4)	Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both?

(5)	If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (NR-TRP-ImplicitLocationInfo-r19) (and vice versa)?

Assuming the intention of the above RAN1 agreement is to provide an implicit indication of the TRP location coordinates to ensure consistency between training and inference, the simplest LPP implementation seems to just replace the location coordinates in the IE NR-TRP-LocationInfo-r16 with an "Associated ID". For example:

-- ASN1START

NR-TRP-LocationInfo-Implicit-r19 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-TRP-LocationInfo-Implict-PerFreqLayer-r19

NR-TRP-LocationInfo-Implicit-PerFreqLayer-r19 ::= SEQUENCE {

trp-LocationInfo-Implicit-List-r19	SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

TRP-LocationInfo-Implicit-Element-r19,

...

}

TRP-LocationInfo-Implicit-Element-r19 ::= SEQUENCE {

dl-PRS-ID-r19					INTEGER (0..255),

nr-PhysCellID-r19				NR-PhysCellID-r16			OPTIONAL,

nr-CellGlobalID-r19				NCGI-r15					OPTIONAL,

nr-ARFCN-r19					ARFCN-ValueNR-r15			OPTIONAL,

trp-Location-Implicit-r19		NR-AIML-AssociatedID-r19,

trp-DL-PRS-ResourceSets-r19		SEQUENCE (SIZE(1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-ResourceSets-TRP-Imlicit-Element-r19		OPTIONAL,

...

}

DL-PRS-ResourceSets-TRP-Implicit-Element-r19 ::= SEQUENCE {

dl-PRS-ResourceSetARP-r19			NR-AIML-AssociatedID-r19						OPTIONAL,

dl-PRS-Resource-ARP-List-r19		SEQUENCE (SIZE(1..nrMaxResourcesPerSet-r16)) OF

DL-PRS-Resource-ARP-Implicit-Element-r19	OPTIONAL,

...

}

DL-PRS-Resource-ARP-Implicit-Element-r19 ::= SEQUENCE {

dl-PRS-Resource-ARP-location-r16	NR-AIML-AssociatedID-r19			OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

In this way, an "Associated ID" would be available at each level (TRP location, Resource Set ARP location, Resource ARP location). Some implementations may be sensitive only on the TRP-level of the implicit location information, others may also be sensitive on the e.g., DL-PRS Resource Set ARPs implicit location information. Therefore, using a single "Associated ID" for all ARPs under a TRP may be too restrictive, since the "Associated ID" would have to change if e.g., any Resource ARP has changed, which however, may not impact a particular implementation.

Proposal 4a (LPP-21):	The new IE for the implicit TRP location information (NR-TRP-LocationInfo-Implicit-r19) has the same structure as the existing IE NR-TRP-LocationInfo-r16, but with the location coordinate information being replaced by an "Associated ID" at each level (TRP, ARPs level).

In all NR positioning assistance data (and measurement information), a TRP is identified by the DL-PRS ID. However, given that there is usually some time elapsed between training and inference phases, the DL-PRS ID only may be ambiguous. To resolve this potential ambiguity, a cell-ID can be provided in addition. There is the following common structure in all existing NR positioning assistance data elements:

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,

For example, in the assistance data used for training, a particular value for the DL-PRS ID may be the same as in the assistance data for inference but may indicate different physical TRPs. A DL-PRS ID would be unambiguous only within the same LPP session, but there is no guarantee that a DL-PRS ID is unambiguous across different LPP sessions (since only 256 values are available). Therefore, for DL AI/ML positioning, whenever assistance data are provided, a LMF must always include the Cell-IDs in all NR assistance data elements when the TRP is associated to a cell (i.e., when the TRP is not a DL-PRS only TP). For PRS-only TPs, a network must ensure that DL-PRS IDs are unambiguous (as far as possible) across sessions. If the Cell-IDs are always included to disambiguate the DL-PRS ID, an "Associated ID" of 8-bits (i.e., same granularity as DL-PRS ID) seems enough.

Proposal 4b (LPP-21):	In all NR AI/ML assistance data IEs where Cell IDs (NCGIs, PCIs) can optionally be included for a TRP, the Cell IDs (NCGIs, PCIs) are always present in the IEs if the TRP is associated to a cell.

Proposal 4c (LPP-21):	If Proposals 4a/4b are agreeable, the "Associated ID" can be defined with 8-bits.

The above Proposals 4a-4c cover the items (1)-(3) above. For item (4), we say no reason why the NR-TRP-LocationInfo-r16 and the NR-TRP-LocationInfo-Implicit-r19 must be mutually exclusive in a Provide Assistance Data message. This should depend only on the UE and network capabilities.

Proposal 4d (LPP-21):	Whether the IEs NR-TRP-LocationInfo-r16 and NR-TRP-LocationInfo-Implicit-r19 can be both provided together or not is left to implementation/deployment and does not need to be specified.

Similar, whether a UE supports both variants (NR-TRP-LocationInfo-r16, NR-TRP-LocationInfo-Implicit-r19) or only one of them is up to model implementation. The current version of the Stage 2 CR specifies [7]:

"The inference technique using AI/ML model for UE positioning is up to implementation and beyond the scope of this specification."

An implementation may train a ML model based on e.g., RSTD measurements (see also section 2.1 above (PRU measurements)) and "conventional" position calculation methods could be used, which require TRP location coordinates. Note, even with existing LPP (IE NR-TRP-LocationInfo), the geographical coordinates of the TRPs are not necessarily "disclosed" (i.e., the ReferencePoint may be a LocalOrigin with a coordinateID only).

Without TRP location coordinates, only some kind of "pattern matching" may be possible. Also, implementations may use/support a combination of both. Therefore, support for NR-TRP-LocationInfo-r16 should not imply support for NR-TRP-LocationInfo-Implicit-r19) (and vice versa). This should be up to deployment/implementation.

Proposal 4e (LPP-21):	If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16) a UE must not mandatorily support also implicit location info (NR-TRP-LocationInfo-Implicit-r19) (and vice versa).

# 3.	Summary

In this contribution we discussed the remaining issues for the LPP support of "AI/ML Positioning Case 1". The following proposals were made.

LPP-6a: Number of PRUs in NR-PRU-DL-Info:

Proposal 1 (LPP-6a):	The assistance data request for PRU measurements should allow for the following:

-	Indication of which PRU measurements are desired (at least on SignalMeasurementInformation level (i.e., whether NR-DL-TDOA-SignalMeasurementInformation, NR-DL-AoD-SignalMeasurementInformation, NR-PRU-RSCP-MeasurementInformation are desired/requested).

-	Indication of a geographic area or DL-PRS/Cell ID list where a desired number of PRUs should be located for which the measurements are requested.

LPP-10a: Applicability of "batch reporting" for AI/ML positioning:

Proposal 2 (LPP-10a):	"Batch reporting", i.e., reporting of up to 32 location results in a single report as supported for the current NR positioning methods, is also applicable to "NR AI/ML Positioning Case 1".

LPP-18: Consistency between training and inference:

Proposal 3 (LPP-18):	The IE NR-On-Demand-DL-PRS-Request can include the DL-PRS ID, Physical Cell ID/ARFCN, and/or NCGI per TRP per frequency layer for which the on-demand DL-PRS request is applicable/desired.

LPP-21: "Associated ID" for TRP Location Coordinates (IE NR-TRP-LocationInfo-Implicit):

Proposal 4a (LPP-21):	The new IE for the implicit TRP location information (NR-TRP-LocationInfo-Implicit-r19) has the same structure as the existing IE NR-TRP-LocationInfo-r16, but with the location coordinate information being replaced by an "Associated ID" at each level (TRP, ARPs level).

Proposal 4b (LPP-21):	In all NR AI/ML assistance data IEs where Cell IDs (NCGIs, PCIs) can optionally be included for a TRP, the Cell IDs (NCGIs, PCIs) are always present in the IEs if the TRP is associated to a cell.

Proposal 4c (LPP-21):	If Proposals 4a/4b are agreeable, the "Associated ID" can be defined with 8-bits.

Proposal 4d (LPP-21):	Whether the IEs NR-TRP-LocationInfo-r16 and NR-TRP-LocationInfo-Implicit-r19 can be both provided together or not is left to implementation/deployment and does not need to be specified.

Proposal 4e (LPP-21):	If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16) a UE must not mandatorily support also implicit location info (NR-TRP-LocationInfo-Implicit-r19) (and vice versa).

# References

[1]	R2-2505702, "LPP open issues for feature 'AI/ML for NR air interface' ", Qualcomm Incorporated (Rapporteur).

[2]	RP-251185, "Status Report to TSG", RAN#108.

[3]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[4]	R2-2503403, "Discussion on consistency between training and inference for AI POS", vivo et al.

[5]	R2-2505704, "CR to 37.355: Introduction of AI/ML Positioning Accuracy Enhancements", Qualcomm Incorporated
 	(Rapporteur).

[6]	R2-2505703, "Summary of [POST130][025][AI PHY] 37.355 CR (Qualcomm)".

[7]	R2-25xxxxx, "38.305 running CR for AIML Positioning", CATT.