3GPP TSG-RAN WG2 #131	R2-2505766

Bangalore, India, August 25th– 29th, 2025

Agenda Item:	8.5.3

Source:	Ericsson

Title:	Discussion on on-demand SIB1 for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised [4]. The core part objective relevant to OD-SIB1 based on the revised WID is:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify the corresponding core requirements, for the above features [RAN4].

RAN2 agreements from RAN2#130:

Agreements on stage-3 details

- Proposal (include ARFCN inside the list with the PCI to have a NES-CellId list associated to a OD-SIB1 config) is not proceeded.

- Keep RRC CR as it is for the short message and UE behaviour.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

- The maximum offset value is extended to 32 radio frames.

- The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

- A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum number of PEI occasion per paging frame is extended to 8.

- The maximum offset value is extended to 32 radio frames.

- Rel-19 excluded cell list can be configured as an empty list. No new UE behavior is introduced (i.e. the UE supporting OD-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present).

- Same as the other 2 agreed cases of OD-SIB1 acquisition failure, capture the failure case of OD-SIB1 window expiry in TS 38.304. RAN2 confirm no other cases need to be considered and specified in TS 38.304.

- The capability for paging adaption to be included in UE-RadioPagingInfo is a separate capability from other NES features and no need to define a common capability for all NES features.

- Use a “container” using OCTET STRING with content generated by UE for paging adaption capability.

- The paging adaption capability is per UE.

- Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band.

- Rely on PHY layer specs to capture availability conditions related to usage of additional PRACH resources. It can be revisited if needed after checking PHY running specifications.

- A note in TS 38.321 is added to clarify that UE selects only amongst preambles configured for the next valid RO(s) for the selected SSB, when CB-PreamblesPerSSB is configured for additional ROs. Wording of the note can be refined during the running CR review discussion.

- The following two proposals were discussed and not pursued:

- (P9 R2-2503712) If UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

- (P8 R2-2503945) RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

Agreements on OD-SIB1

- Not to include access related information of NES into WUS configuration.

- Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

- Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

- If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

With these two general guidelines in mind, in this paper, we discuss the remaining open questions concerning the following aspects:

- The open issue list identified in [POST130][107][NES] (Ericsson)

- On-demand SIB1 request,

- Impact on Random Access report,

- On-demand SIB1 and Automatic neighbour relation.

# 2	On the open issues

Open issues from [POST130][107][NES] (Ericsson) [3] related to the RRC CR in [2]:

OD-SIB1:

Issue 1: Q12

Issue 2: Vivo002

## 2.1 Q12 on si-BroadcastStatus

si-BroadcastStatus

Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. When SIB19 is scheduled in an NTN cell, the si-BroadcastStatus for the mapped SIB19 is set to broadcasting. When SIB22 is scheduled in an ATG cell, the si-broadcastStatus for the mapped SIB22 is set to broadcasting. FFS: how to capture that a CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1. E.g. “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1.”

Q12: Can “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1” be added to the field description of si-BroadcastStatus to address the FFS?

While majority of companies actually supported addition of the “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1.” there were also views that it is not needed and that it is not correct. Hence it is better to further discuss before resolving the item. The agreement that we are attempting to capture reads:

A CONNECTED MODE UE supporting OD-SIB1 who is in a cell that does not broadcast SIB1, understands that the stored SIB1 is the latest SIB1.

The comments which shared concern reads:

| vivo | We don’t think it’s needed in the Field description as we already have the normative text for it. It’s redundant. |
| --- | --- |
| LGE | It is unclear whether the ‘latest SIB1’ means ‘SIB1 including latest si-BroadcastStatus. A clearer way to express it might be: “The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 is up-to-date” In addition, such consideration should not be made when the UE is connected to a legacy cell with always-on SIB1. An additional condition, such as ‘if SIB1 is being transmitted in on-demand in this cell’, is needed. |
| Xiaomi | We don’t think it is appropriate to add to the field description of si-BroadcastStatus as OD-SIB1 does not use that parameter. |

Another attempt to capture the agreement could be to add the following sentence, which is a modified version from what LGE suggest:

“The UE in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as up-to-date when served by a cell with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster”

Add “The UE in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as up-to-date when served by a cell with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster” in the field description of  si-BroadcastStatus

## 2.2 Vivo002 UE checking if SIB1 is broadcasted before sending a request

The issue raised by Vivo and related discussion reads as:

5.2.2.3.1 Acquisition of MIB and SIB1 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> if the UE is in RRC_IDLE or in RRC_INACTIVE; or 4> if the UE is in RRC_CONNECTED while T311 is running: 5> perform the actions as specified in clause 5.2.2.3.3x; 3> else: 4> perform the actions as specified in clause 5.2.2.5. NOTE 1: The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e., the broadcast and unicast/MBS multicast beams are quasi co-located. The UE in RRC_INACTIVE state while T319a is running, is only required to acquire broadcasted SIB1 and MIB if the UE can acquire them without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located. NOTE 2: UE in RRC_INACTIVE that does not support inactiveStateNTN-r17 enters RRC_IDLE upon cell reselection between TN cell and NTN cell, and initiates the NAS signalling connection recovery (see TS 24.501 [23]). NOTE x: It is up to UE’s implementation to check if SIB1 is currently being broadcasted for that cell before triggering OD-SIB1 request procedure of that cell. [comment] During last meeting, at offline session [AT130][104][NES] (Ericsson), P6 in R2-2504037 about the UE behavior after receiving SI change notifcation was discussed. Rapporteur responded that a condition was added to address the issue: 5> if SIB1 is not broadcasted 6> perform the actions as specified in clause 5.2.2.3.3x; And therefore, at the Comeback session, it was first agreed that: Keep RRC CR as it is for the short message and UE behaviour.. Then, it is commented that the high-lighted part mandates the UEs to check whether SIB1 is broadcasting before requesting OD-SIB1 (not the case upon receiving SI change notification), which is against the RAN1 agreement that it is up to the UE’s implementation to check if SIB1 is broadcasting. Thus, the high-lighted part was removed, and instead the NOTE x is added. However, RAN2#126 agrees that: Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.. Therefore, we encourage RAN2 to review the agreement again ‘Keep RRC CR as it is for the short message and UE behaviour.’, whether the NOTE x can cover the above RAN2#126 agreement. The discussion is comprised of the following questions: Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update? [HONOR] For Q1, we understand that this agreement simply shows the UE can acquire the updated SIB1 from the NES cell without returning to Cell A. It does not imply that the NES cell will immediately transmit CD-SSB for SIB1 update upon broadcasting a SIB change notification (e.g., at the start of the next MP). Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this? [HONOR] For Q2, we think the UE camping on the NES cell can initiate OD-SIB1 request based on the WUS configuration to acquire the updated SIB1. We think for Q2, if the CORESET#0 in the stored OD-SIB1 request configuration has changed, the UE camping on the NES cell will have no way by implementation to receive the updated SIB1. As a result, all the UEs camping on the NES cell will trigger OD-SIB1 request since SIB1 cannot be received, which is not correct. Therefore, we propose Q1/Q2 to be listed as open issues for further discussion. [HONOR] Same view as Vivo, i.e., add Q1, Q2 and the issue of WUS configuration changes caused by CORESET#0 modifications to open issues for further study.

Q1. For RAN2#126 agreement ‘ Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.’, does it mean that the UE always assumes the NES cell will turn from transmitting NCD-SSB to transmitting CD-SSB for SIB1 update?

In our understanding a cell that is providing SIB1 on demand has set K_SSB in a way that it the SSB looks like NCD-SSB to legacy UEs. The cell can provide SIB1, even in periodic manner and does not have to change the value of K_SSB. This is completely up to network implementation. Hence the answer is NO.

Q2. If the answer is ‘NO’ for Q1, i.e. the NES cell can remain transmitting NCD-SSB and at the same time transmit the updated SIB1, how does the UE camping on the NES cell to receive the updated SIB1? Shall we rely on the NOTE x to say it’s up to UE’s implementation to handle this?

We agree that the current note is confusing and would like to add back the procedural text:

5> if SIB1 is not broadcasted

6> perform the actions as specified in clause 5.2.2.3.3x;

In our view this is not conflicting to RAN1 agreement but we see the situation that Ran1 did not see the need for normative specification for UE to check for SIB1 whereas RAN2 does. This is quite normal situation that since the scope and expreties fields of different working groups differ, the viewpoints also differ. If there is a concern that RAN1 actually has an issue with UE checking if it can receive SIB1, LS can be considered.

Remove the Note x from 5.2.2.3.1 and add back the procedure text “5> if SIB1 is not broadcasted” before “6> perform the actions as specified in clause 5.2.2.3.3x”.

Discuss if LS is needed to check with RAN1 if they have concern with Proposal 1.

# 4	On-demand SIB1 request for connected mode, capability

In the RAN1#120bis, the following question was raised as a note in [8]:

“Note: whether/how to address SIB1 request for connected UEs including the need of capability signaling is up to RAN2”

In our opinion, there is no need to consider the capability signalling for connected UEs, as the OD-SIB1 is considered for the UEs in the connected mode only in the RLF case as agreed in the RAN plenary.

No need to consider the capability signalling for connected UEs.

# 5	Impact on Random Access report

Another important feature that may be affected by on-demand SIB1 is Random Access (RA) report, which comprises vital information for RACH optimization. Since on-demand SIB1 procedure uses RA procedure, this case should be supported. One of the mandatory IEs of RA report is RA purpose, e.g., “access related”. One issue is that none of the existing values for RA purpose covers the case where RA procedure was used for request for SIB1. Note that the existing value “requestForOtherSI” specifies that RA procedure was used to request other SI, i.e., SIB2, SIB3, etc. SIB1, however, is a part of the so-called minimum SI (the MIB in the SSB is the other). Hence the value does not qualify for being reused.

None of the existing values for RA purpose in RA report covers the case where RA procedure was used for request for SIB1.

With this issue, the introduction of on-demand SIB1 leaves RA report ill-defined, and the UE behaviour unclear. Hence, the issue must be resolved in Rel-19.

The introduction of on-demand SIB1 has an impact on RA report that must be addressed in Rel-19.

Fortunately, there are many spare values for RA purpose. A simple solution could be to add a new value, e.g., “requestForSIB1” (the name is FFS). We propose RAN2 to discuss the impact of on-demand SIB1 on RA report and consider the addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

RAN2 to agree on addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

# 6 On-demand SIB1 and Automatic Neighbour Relation

One of the important features that may be affected by on-demand SIB1 is Automatic Neighbour Relation (ANR). Introduction of on-demand SIB1 affects the ANR in a way that NES Cell with on-demand SIB1 cannot be discovered by the ANR. According to the current specification (c.f., Section 15.3.3 of 38.300 [3]), if a cell is not broadcasting SIB1, the UE aborts the Cell Global Identity (CGI) reporting procedure and reports "noSIB1":

“In case the detected NR cell does not broadcast SIB1, the UE may report noSIB1 indication as specified in TS 38.331 [2].”

Therefore, if a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally. Furthermore, if the coexistence of on-demand SIB1 and ANR is not addressed, operators may need to manually provision and manage NES cells with on-demand SIB1 in the neighbour relation tables. Since this is not a practical and desirable solution, there is a need for new means that would allow for a coexistence of on-demand SIB1 and ANR.

If a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally.

Discuss how to support a functional coexistence of on-demand SIB1 and ANR.

One solution to support the coexistence of on-demand SIB1 and ANR could be that the NW configures the UE whether CGI-Info is to be captured for NES Cells with on-demand SIB1. Another solution could be that CGI-Info (or parts of it) of the NES cell is included in the UL WUS configuration such that the UE already knows or only needs to capture the UL WUS configuration and hence avoids asking for SIB1 for the sake of ANR. The later may be possible if the UE can obtain the UL WUS configuration. In the following, we discuss how to address the problem if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell. We refer to this problem as the problem of a missing UL WUS.

The problem of a missing UL WUS configuration is illustrated in Figure 1, and it occurs in a scenario where a part of the coverage area of the NES Cell is not covered by a Cell A, but it is covered by another cell, here denoted Cell X. The scenario depicted in Figure 1 can occur in reality, for various reasons. For example, if the coverage area of a cell has a very irregular shape which is typical e.g., in cities, due to various obstructions or if the coverage area of a cell changes over time, e.g., due to Coverage and Capacity Optimization (CCO). In these cases, the UE can find the NES Cell, but it cannot find (any) Cell A (i.e., the UE cannot camp on the NES Cell). Therefore, the problem of a missing UL WUS configuration may lead to additional issues in the context of the coexistence of on-demand SIB1 and ANR.

Figure 1: An example of the scenario in which the missing UL WUS configuration problem occurs.

The coverage area of a Cell A (or NES Cell) can have an irregular shape (e.g., in cities due to various obstructions) and/or can change over time (e.g., due to performing CCO), and thus the coverage overlap of Cell A and a NES Cell can also vary.

Additional issues in the context of the coexistence of on-demand SIB1 and ANR can occur if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell.

The above observations imply that there is a need to support the detection of problems with missing UL WUS configuration and other similar problems with on-demand SIB1 provisioning. On the one hand, there is a need to support the means that would allow the NES Cell to identify itself as a NES Cell without having to rely on a Cell A. For example, the means that would allow a UE to detect that a cell is a NES Cell (i.e., that is using on-demand SIB1 transmission) from the SSB/MIB of the cell itself. On the other hand, there is a need for means that would allow a UE to inform the NW about problems with provisioning the on-demand SIB1 and the UL WUS configuration.

A NES Cell needs to be able to identify itself as a NES cell without relying on a Cell A so that UEs can efficiently detect missing UL WUS configuration.

There is a need for means that would allow a UE to inform the NW about problems with the provisioning of the on-demand SIB1 and the UL WUS configuration.

Related to this aspect, RAN1 agreed the following during the RAN1#118bis meeting:

RAN1#118bis

Agreement

UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A

According to the above agreement from RAN1, a UE can use at least the UL WUS configuration from Cell A to determine a cell with on-demand SIB1. It is important to observe that if a UE relies only on Cell A when determining a cell supporting on-demand SIB1 (as captured in the RAN1 agreement) a UE may not be able to detect faulty configuration if a Cell A has not been visited (as illustrated in Figure 1). Since the RAN1 agreement does not exclude the other means that would allow a UE to determine a cell supporting on-demand SIB1 (i.e., the agreement states that “at least UL WUS configuration from Cell A” is used), we propose the following.

RAN2 to support NES Cell identifying itself as a NES cell based on SSB content without having to rely on a Cell A.

Discuss the means that would allow a UE to inform the NW about problems with on-demand SIB1 and UL WUS configuration provisioning.

# 7	Conclusion

In this contribution we discuss the second objective highlighted in the WID. Based on the discussion in the previous sections we made the following observations:

Observation 1	None of the existing values for RA purpose in RA report covers the case where RA procedure was used for request for SIB1.

Observation 2	The introduction of on-demand SIB1 has an impact on RA report that must be addressed in Rel-19.

Observation 3	If a UE relies on legacy ANR means according to which the UE indicates “noSIB1” only, it may be challenging to distinguish the case of a faulty cell not having SIB1 from the case of a NES Cell that does not provide SIB1 at certain times intentionally.

Observation 4	The coverage area of a Cell A (or NES Cell) can have an irregular shape (e.g., in cities due to various obstructions) and/or can change over time (e.g., due to performing CCO), and thus the coverage overlap of Cell A and a NES Cell can also vary.

Observation 5	Additional issues in the context of the coexistence of on-demand SIB1 and ANR can occur if the UE finds a NES Cell, but it cannot obtain the UL WUS configuration for the NES Cell.

Observation 6	A NES Cell needs to be able to identify itself as a NES cell without relying on a Cell A so that UEs can efficiently detect missing UL WUS configuration.

Observation 7	There is a need for means that would allow a UE to inform the NW about problems with the provisioning of the on-demand SIB1 and the UL WUS configuration.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Add “The UE in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as up-to-date when served by a cell with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster” in the field description of  si-BroadcastStatus

Proposal 2	Remove the Note x from 5.2.2.3.1 and add back the procedure text “5> if SIB1 is not broadcasted” before “6> perform the actions as specified in clause 5.2.2.3.3x”.

Proposal 3	Discuss if LS is needed to check with RAN1 if they have concern with Proposal 1.

Proposal 4	No need to consider the capability signalling for connected UEs.

Proposal 5	RAN2 to agree on addition of a new value for RA purpose to accommodate for the case where RA procedure was used for request for SIB1.

Proposal 6	Discuss how to support a functional coexistence of on-demand SIB1 and ANR.

Proposal 7	RAN2 to support NES Cell identifying itself as a NES cell based on SSB content without having to rely on a Cell A.

Proposal 8	Discuss the means that would allow a UE to inform the NW about problems with on-demand SIB1 and UL WUS configuration provisioning.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

R2-2505699, “Introduction of enhancements for network energy efficiency

R2-2505708, “Report of collection of comments to 38.331 CR for NES”

# Appendix A

### RAN WG2 Meeting #125bis

1.	At least RAN2 starts scenario 1a. Other scenarios are not excluded.

a. 	Scenario 1a: Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell. with below potential RAN2 impacts:

i.	Add WUS configuration in SIB of cell A.

ii.	Cell reselection from cell A to NES cell, including trigger condition and cell barring changes.

iii.	Whether allow camping, paging and SIB update in NES cell.

iv.	Cell reselection from NES cell to cell A or normal cell.

2.	RAN2 assumes that RACH procedure is reused for UE to request on-demand SIB1.

3.	UL WUS configuration includes at least below information:

a.	RACH configuration

4.	A UE needs to know a UL WUS configuration to request SIB1 of which cell.

5.	Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

6.	The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

### RAN WG2 Meeting #126

1.	Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

a.	More than one Cell A may provide configuration for the same NES cell.

b. 	The same Cell A may assist more than one NES Cells.

2.	RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

3.	Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

4.	For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

5.	Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

6.	If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

7.	After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

8.	RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

9.	NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

10.	How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

11.	If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

12.	After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

13.	RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

14.	As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

15.	Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

16.	RAN2 to wait for RAN1’s progress whether to support scenario 3.

### RAN WG2 Meeting #127

1.	No consensus for RAN1 case 3 in RAN2.

2.	We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

3.	Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

4.	Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

5.	Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

6.	Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

a.	Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

b.	Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

c.	Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

7.	RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

8.	RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

### RAN WG2 Meeting #127bis

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

### RAN WG2 Meeting #128

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

### RAN WG2 Meeting #129

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

### RAN WG2 Meeting #129bis

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.