3GPP TSG-RAN WG2 Meeting #131                                 R2-2505770

Bangalore, India, Aug.25th- 29th, 2025

Agenda item:		8.2.4

Source:		Samsung

Title:	Discussion on the remaining issues on A-IoT data transmission

Document for:	Discussion & Decision

- 1 Introduction

In RAN2#130, the AIoT RACH procedures were extensively discussed with the following agreements:

Agreements

- R2D message scheduling non-first segment (re)transmission does not include upper layer command.

- For the first segment and unsegmented packet (re)transmission, the “offset” indicator in R2D is not present.

- This implies that the R2D message will either have command or offset (but not both).  FFS whether we define two message types or one message type with optional fields.

Agreements

- The device is expected to send a MAC response to the reader in the D2R occasion.   The MAC response contains the NAS message if available at the D2R occasion.   If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

- Send LS to CT1 to inform the agreement 1 to CT1 and explain that we have an issue with delayed NAS write success response.   RAN2 would prefer that this is handled by CT1 (and give the example of sending NAS response upon successful reception of write command).    Ask if this can be handled by CT1

Agreement on MAC PDU format

- A mandatory length field directly indicates the length of D2R data MAC SDU to support varying lengths of D2R data.    The size of length field is 7-bit in bytes.

- The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 7-bit length in bytes.  Segmented SDUs are also byte aligned.

- FFS D2R message type.  Current running CR will capture no message type,  but we can revisit this next meeting and also consider if any other bits are needed for the MAC header

- The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed.  FFS can come back if padding is needed depending on granularity of TBS  (only if needed)

Agreements

-	For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not.   FFS for CFRA

-	FFS for need for release message

In this contribution, we will share our views on the remaining issues.

- 2 Discussion

Issue 1: More data indication for no data available case

This issue occurs because the NAS layer cannot generate upper layer data before the allocated resource. In last meeting, an LS to CT1 was sent to check whether CT1 can handle this issue. Thus, we can assume there is no RAN 2 work till now. Even if the case happens, the device can send the more data indication with 0 and zero SDU to the reader. Since the reader expects a NAS PDU from the device, zero SDU means that the device has not generate the data in time at upper layer; then, the reader can send another R2D message later to schedule the D2R upper layer data transmission.

Proposal 1: RAN2 is kindly asked to agree that the device can set more data indication to 0 with zero SDU in D2R Upper Layer Data Transfer message if NAS PDU is not available when sending such message.

Issue 2: AS ID release

During the discussion, some companies propose to introduce a dedicated R2D message for AS ID release. The intention is to release the AS ID as early as possible. The AS ID has a length of 16 bits, which can be allocated to 2^16=65536 devices. The timely release of AS ID is needed only if the number of devices exceeds 65536 since the reader needs reuse some AS IDs among different devices. Given 65536 is a big number, the reader may not need to reuse the AS IDs. On the other hand, if new message is introduced for AS ID release, more discussion in RAN2 is needed, e.g., device behaviour if such new message is lost. Thus, we prefer to not define a dedicated message for AS ID release.

Proposal 2: RAN2 is kindly asked to agree that a dedicated R2D message for AS ID release is not needed.

Issue 3: forward compatibility

So far, the Paging message contains one R bit for the forward compatibility. While the other R2D messages do not contain such R bit. For the future extensibility, it is better to add R bit in other R2D messages.

Proposal 3: RAN2 is kindly asked to agree that R bit is added to other R2D messages (i.e., Access trigger, Msg.2, R2D upper layer data transfer, NACK feedback message) for forward compatibility.

- 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 is kindly asked to agree that the device can set more data indication to 0 with zero SDU in D2R Upper Layer Data Transfer message if NAS PDU is not available when sending such message.

Proposal 2: RAN2 is kindly asked to agree that a dedicated R2D message for AS ID release is not needed.

Proposal 3: RAN2 is kindly asked to agree that R bit is added to other R2D messages (i.e., Access trigger, Msg.2, R2D upper layer data transfer, NACK feedback message) for forward compatibility.