3GPP TSG-RAN WG2 Meeting #131                                                              R2-2505774

Bangalore, India, Aug. 25th- 29th, 2025

Agenda item:	8.13.3

Source:	Samsung

Title:	Discussion on remaining issues of MH SL relay control plane procedures

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the control plane procedures were extensively discussed for multi-hop SL relay, and reached the following agreement:

Agreements:

Do not pursue fast forwarding.

Continue during this meeting to discuss including the RRC state in relay discovery, including:

- 	Spec impact to capture it in the RRC container

-	Implications for (re)selection procedure

Continue discussion of whether/how to extend timers.

Agreements:

There is no remote UE ID in SIB request or SIB delivery, i.e. RemoteUEInformationSidelink and UuMessageTransferSidelink.

In-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire SIB broadcast by the last relay UE’s serving cell via Uu interface directly for it’s own concerned SIB.

Agreements:

For the SIB requested by child UE, if it is also the concerned SIB of intermediate relay UE, in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly.

For the SIB requested by child UE, if it is NOT the concerned SIB of intermediate relay UE, regarding in-coverage intermediate relay UE in IDLE/INACTIVE is allowed to acquire such SIB broadcast by the last relay UE’s serving cell via Uu interface directly.

Agreements:

Below b), c), and e) are applied also in IDLE/INACTIVE states for intermediate relay UE without additional specification change.

The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

For the SIB requested by child node, if the SIB has not been included in SIB request in parent link, if IDLE/INACTIVE intermediate relay UE obtain the requested SIB via Uu interface or unsolicited SIB1 forwarding and CAN respond SIB request of child UE, adopt legacy condition(“if the UE has SIB request information to provide”) to determine SIB request in parent link is needed or not.  Applicability of this legacy language to the multihop case to be documented in RRC.

Legacy condition “if the UE has SIB request information to provide” is enough, no additional specification change is needed to support  “when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu” (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

Agreement:

Do not implement reflective bearer mapping in the running CR(s) now; interested companies are asked to converge a single detailed TP for final go/no-go decision next meeting.

Agreement:

Extend the T300, T301 and T319 timers for multi-hop U2N relay operation.  The UE and gNB multiply the signalled timer value by the hop count.

Working assumption:

The Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container.  As a baseline, remote UE relay (re)selection behaviour based on this information is left to implementation, and it can be discussed in spec implementation/maintenance if some guidance is needed.  No SA2 spec impact is expected.

Companies are asked to confirm with SA2 colleagues if there is any concern with a candidate first relay UE sending the RRC container with its own information.

Agreements:

The intermediate relay UE can have its own traffic acting as a remote UE simultaneously.  Running CRs will be checked to make sure this functionality is supported.

Clarify in the normative text that the UE can be a relay and remote UE simultaneously (to be determined case by case where it needs to be documented).

In this contribution, we will address the remaining issues for control plane procedures.

Discussion

Issue (RRC-13): Timer301 setting for T300/T301/T319

In last meeting, the following agreements were reached:

Agreement:

Extend the T300, T301 and T319 timers for multi-hop U2N relay operation.  The UE and gNB multiply the signalled timer value by the hop count.

The effective T300/301/319 are set based on T300/301/319 for one hop SL relay and the number of hops. This method will result in quite large timer since the T300/301/319 of single hop SL relay considers both the delay of Uu link and PC5 link. However, in multi-hop case, the additional link delay is only due to the PC5 links. Thus, it is better to have reasonable values. During the post-email discussion, the following options are given by Rapp.:

Option 1 -  Instead of multiplying the timers in UE-TimersAndConstantsRemoteUE  with the hop count we multiply the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count.

Option 2 – We introduce a new timer for a PC5 hop, e.g., t300-PC5/t301-PC5/t319-PC5 and calculate the value as t300 for n-hop relay = t300 + t300-PC5*n.

Option 3 – We multiply the timers in  UE-TimersAndConstantsRemoteUE  with the hop count as currently captured in the CR

Option 4 - Further extend the timer T300 to cover setup times that grow exponentially with the number of hops in UE-TimersAndConstantsRemoteUE

Among them, we prefer to Option 2 since the increasing step along with the number of hops can be reasonably set considering the delay of one hop PC5 link.

Proposal 1: RAN2 is kindly asked to agree that the T300/T301/T319 for multihop SL relay is extended based on the following method:

- Effective T300/T301/T319 = T300/T301/T319-basic + T300/T301/T319-hop* n (number of hops)

To realize the above method, SIB12 can be enhanced. Currently, SIB12 contains t300/t301/t319-RemoteUE, which can be considered as T300/T301/T319-basic. To indicate the increase step, a new parameter can be introduced in SIB2, e.g., increasing step. Moreover, such step can be shared by three timers.

Proposal 2: RAN2 is kindly asked to agree to introduce an increasing step in SIB2 to help the remote UE determine the timer setting of T300/T301/T319.

Conclusion

In this contribution, we address some issues for control plane procedures, and have the following proposals:

Proposal 1: RAN2 is kindly asked to agree that the T300/T301/T319 for multihop SL relay is extended based on the following method:

- Effective T300/T301/T319 = T300/T301/T319-basic + T300/T301/T319-hop* n (number of hops)

Proposal 2: RAN2 is kindly asked to agree to introduce an increasing step in SIB2 to help the remote UE determine the timer setting of T300/T301/T319.

References