3GPP TSG-RAN WG2 Meeting #131	R2-2505776

Bengaluru, India, August 25-29, 2025

Agenda item:	8.15

Source: 	Qualcomm Incorporated

Title: 	Missing Capabilities for SSR Orbit/Clock Corrections Set2

Document for:		Discussion and Decision

# 1.	Introduction

In the "agreed in principle" CR for the introduction of NavIC L1 SPS A-GNSS in LPP [1], the IEs GNSS-SSR-OrbitCorrectionsSet2, GNSS-SSR-ClockCorrectionsSet2, and GNSS-SSR-URA-Set2 have been updated to support NavIC L1 reference ephemeris. However, the corresponding capabilities have not been updated. Although, this oversight has already been made when these IEs were introduced in Rel-17 [2], it is proposed to correct this as part of the 'LCS_NAVIC_L1_SPS_NR_LTE-Core' work.

# 2.	Discussion

The SSR orbit and clock corrections are provided relative to a broadcast ephemeris. However, modernized GNSS navigation messages have multiple ephemeris formats defined to calculate the satelite position and clocks (e.g., for different bandwidth efficiency). The default broadcast messages used for SSR Orbit and Clock Corrections in the LPP IEs GNSS-SSR-OrbitCorrections, GNSS-SSR-ClockCorrections, and GNSS-SSR-URA are summarized in the Table 1 below.

Table 1: Default broadcast ephemeris for SSR corrections

| GNSS | Broadcast Message Type | LPP Model # |
| --- | --- | --- |
| GPS and QZSS | NAV Data | Model-2 |
| GLONASS | GLONASS M Data | Model-4 |
| Galileo | I/NAV data | Model-1 |
| BDS | D1 NAV/D2 NAV Data | Model-6 |
| NavIC | L5 NAV | Model-8 |

To support additional broadcast messages as reference for the corrections, the corresponding "Set2" IEs were introduced [2] (GNSS-SSR-OrbitCorrectionsSet2, GNSS-SSR-ClockCorrectionsSet2, and GNSS-SSR-URA-Set2), which currently support the messages as summarized in Table 2 below [1].

Table 2: Broadcast Messages supported for SSR Orbit and Clock Corrections Set 2

| GNSS | Broadcast Message Type | LPP Model # |
| --- | --- | --- |
| BDS | B-CNAV1 | Model-7 |
| NavIC | L1 NAV | Model-9 |

This support was implemented via the field refEph-r17 as ENUMERATED type in the Provide and Request Assistance Data IEs as shown below [2].

However, the corresponding capabilities cannot indicate a supported reference ephemeris for the "Set2 corrections". For example:

Since only a single additional reference ephemeris per GNSS is currently supported, the refEph-r17 ENUMERATED definition in the Set2 elements would not strictly be needed, since it would be clear from the GNSS-ID (with proper field description). However, the "Set2" definitions are supposed to be future proof, and in case of more than a single reference ephemeris for "Set2" corrections are supported in the future, the refEph-r17 would be needed. However, the corresponding indication would then also be needed in the capability definitions. In any case, the specification is inconsistent in this regard.

# 3.	Proposal

The following additions are proposed to be included in the NavIC L1 baseline CR [1].

---- Begin TP ----

#### –	GNSS-SSR-OrbitCorrectionsSet2Support

-- ASN1START

GNSS-SSR-OrbitCorrectionsSet2Support-r17 ::=	SEQUENCE {

...,

[[

refEphSupport-r19	BIT STRING 	{	b1c 		(0),

navicL1	(1)

} SIZE (1..8))				OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-OrbitCorrectionsSet2Support field descriptions

refEphSupport This field specifies the reference ephemeris that the target device supports for the SSR orbit corrections. This is represented by a bit string, with a one-value at the bit position means the particular reference ephemeris is supported; a zero-value means not supported. - In the case that gnss-ID indicates 'bds' and refEphSupport indicates 'b1c', the SSR orbit corrections are supported for BDS B1C/B2a in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel (i.e. the B-CNAV1 broadcast ephemeris [52]). - In the case that gnss-ID indicates 'navic' and refEphSupport indicates 'navicL1', the SSR orbit corrections are supported for NavIC L1 [xx] in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel.

#### –	GNSS-SSR-ClockCorrectionsSet2Support

-- ASN1START

GNSS-SSR-ClockCorrectionsSet2Support-r17 ::=	SEQUENCE {

...,

[[

refEphSupport-r19	BIT STRING 	{	b1c 		(0),

navicL1	(1)

} SIZE (1..8))				OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-ClockCorrectionsSet2Support field descriptions

refEphSupport This field specifies the reference ephemeris that the target device supports for the SSR clock corrections. This is represented by a bit string, with a one-value at the bit position means the particular reference ephemeris is supported; a zero-value means not supported. - In the case that gnss-ID indicates 'bds' and refEphSupport indicates 'b1c', the SSR clock corrections are supported for BDS B1C/B2a in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel (i.e. the B-CNAV1 broadcast ephemeris [52]). - In the case that gnss-ID indicates 'navic' and refEphSupport indicates 'navicL1', the SSR clock corrections are supported for NavIC L1 [xx] in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel.

#### –	GNSS-SSR-URA-Set2Support

-- ASN1START

GNSS-SSR-URA-Set2Support-r17 ::= SEQUENCE {

...,

[[

refEphSupport-r19	BIT STRING 	{	b1c 		(0),

navicL1	(1)

} SIZE (1..8))				OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-URA-Set2Support field descriptions

refEphSupport This field specifies the reference ephemeris that the target device supports for the SSR URA information. This is represented by a bit string, with a one-value at the bit position means the particular reference ephemeris is supported; a zero-value means not supported. - In the case that gnss-ID indicates 'bds' and refEphSupport indicates 'b1c', the SSR URA information is supported for BDS B1C/B2a in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel (i.e. the B-CNAV1 broadcast ephemeris [52]). - In the case that gnss-ID indicates 'navic' and refEphSupport indicates 'navicL1', the SSR URA information is supported for NavIC L1 [xx] in table GNSS to iod Bit String (11) relation in IE GNSS-NavigationModel.

--- End TP ---

# References

[1]	R2-2504893, "CR to 37.355: Introduction of NavIC L1 SPS A-GNSS in LPP", Ericsson, Reliance Jio, ISRO, MediaTek, CEWiT.

[2]	R2-2313817, "Correction on transmission of SSR Assistance Data based on BDS B1C", CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, ZTE Corporation, MediaTek Inc., OPPO, xiaomi, vivo, Spreadtrum, Ericsson, Swift Navigation.