3GPP RAN WG2 Meeting #130	R2-2505788

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.6.4

Source:	Ofinno

Title:	Discussion on remaining issues of CLTM

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss open issues on conditional LTM.

# Identified open issues:

## TA timer of candidate cell (MAC-14)

A UE may be configured with up to 8 candidate cells for CLTM. UE may perform early UL sync to the candidate cells for CLTM. Based on performing the early UL sync, the UE may receive TA value for the candidate cells. The UE starts a TA timer corresponding to each candidate cell for which TA value is received. If The UE satisfies an execution condition associated with the candidate cel, the UE may execute CLTM to one of these candidate cells, for which TA timer is running. The candidate cell becomes the target PCell. While executing CLTM to the target PCell, the UE stops the TA timer of this candidate cell target PCell and starts TA timer for PCell with the remaining value of the TA timer of this candidate cell. The UE continues the TA timer for the other candidate cells.

RAN2 agreed that network can send an LTM cell switch command MAC CE indicating a candidate cell for CLTM. The candidate cell may already have a TA timer running based on performing early UL sync procedure. The TA value of the candidate cell is also known to the network. Further, the network may have its own TA management mechanisms e.g., the network may maintain the TA of the candidate cell and knows whether the TA value of the candidate cell is valid or not. The LTM cell switch command MAC CE can indicate a TA value for the candidate cell. If the network determines the TA time value of the candidate cell to be valid, then the network indicates TA value in the LTM cell switch command MAC CE.

Observation 1: (MAC-14) If TA of a candidate cell is valid, the LTM cell switch command MAC CE includes the TA value (i.e., Timing Advance Command field is not set as FFF).

It is possible that the network determines that the TA value of the candidate cell is no longer valid. When a cell switch to the candidate cell is initiated, the network sends LTM cell switch command MAC CE indicating that TA value is not valid (i.e., Timing Advance Command field is set as FFF).

Observation 2: (MAC-14) If TA of a candidate cell is not valid, the LTM cell switch command MAC CE does not include the TA value (i.e., Timing Advance Command field is set as FFF).

Some companies proposed to use the TA value of the candidate cell, maintained on the UE based on receiving the TA value during/ after early UL sync procedure, if LTM cell switch command MAC CE is received for the candidate cell while TA timer for the candidate cell is running. The Timing Advance Command field is always included in the LTM cell switch command MAC CE i.e., it is not an optional field. The network always includes the Timing Advance Command field in an LTM cell switch command MAC CE. This means that the network always provides either a valid TA value (i.e., Timing Advance Command field is not set as FFF) or an indication that TA is not available ((i.e., Timing Advance Command field is set as FFF). If a network does not include TA value for the candidate cell, it means that that network sends the LTM cell switch command MAC CE with Timing Advance Command field set to FFF. If the network already has valid TA value for the candidate cell, there is no reason why the network would not include this in the LTM cell switch command MAC CE (i.e., there is no signalling gain or complexity reduction by not including the TA value). This value of the Timing Advance Command field already has a defined meaning/ purpose – to indicate to UE that a valid TA value is not included. The network may use this if the TA value is no longer valid. If the UE executes LTM cells switch to the candidate cell using the TA value received prior to receiving the LTM cell switch command, the LTM execution can fail.

Proposal 1: (MAC-14) If UE receives (Enhanced) LTM cell switch command MAC CE for LTM to a candidate cell while TA timer for the candidate cell is running, the UE uses the TA value indicated in the LTM cell switch command MAC CE and not the TA value received prior to receiving the LTM cell switch command MAC CE.

Proposal 2: (MAC-14) if UE receives (Enhanced) LTM cell switch command MAC CE for LTM to a candidate cell while TA timer for the candidate cell is running, the UE stops the TA timer for this candidate cell.

## 5.12 MAC Reset … Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall: … 1> else if upper layers indicate the reset is triggered by conditional LTM, or triggered by LTM while there is CLTM candidate configuration(s), or triggered by handover while there is CLTM candidate configuration(s): 2> stop (if running) all timers, except MBS broadcast DRX timers, ltm-Candidate-TimeAlignmentTimers, and tm-Candidate-TimeAlignmentTimerTAG2, if configured; 2> consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2;

UE may execute a legacy handover of a conditional handover to a cell configured for CLTM. Therefore, it is possible that the UE may have a candidate TA timer for a CLTM candidate cell and the UE executes a legacy handover or CHO to this cell. The current running CR allows to continue all the candidate TA timers. However, RAN2 has not discussed the handling of candidate TA timers for CLTM candidate cell that becomes the target cell of this legacy handover or CHO. RAN2 needs to discuss the handling of candidate TA timer for the target cell for legacy handover or CHO.

We think there are 2 options to handle this case. In a first option, the UE stops the candidate TA timer of the CLTM candidate cell that is the target cell for legacy handover or CHO while continuing the candidate TA timers for the other CLTM candidate cells. In a second option, the UE continues the candidate TA timers for all the CLTM candidate cells, including the CLTM candidate cell that is the target cell for the legacy handover or CHO. In this option (i.e., the second option), the actions related to handling of the candidate TA timer for the target cell, after handover/ CHO to the target cell is unclear. For example, the UE can receive a TA timer for the target cell during RACH to the target cell. This leads to a case where the UE has 2 TA values for the same cell i.e., first TA timer for LTM candidate cell, and a second TA for the target cell (i.e., current serving cell). The UE cannot use the first TA timer on the current serving cell and can use this only for subsequent CLTMs.

Proposal 3: RAN2 to discuss and agree one of the following options for handling TA timer of a CLTM candidate cell that is target cell for L3 handover (i.e., legacy HO or CHO):

Option 1: stop the TA timer for this CLTM candidate cell while continuing the TA timers for the other CLTM candidate cells.

Option 2: continue the TA timer for all CLTM candidate cells.

RAN2 agreed that when an LTM or CLTM is executed to a candidate cell, the UE continues (i.e., does not stop) the TA timer for other candidate cells, if running. The candidate cells are maintained on the UE until they are released by the network, or until a condition to release the candidate cells is met e.g., RRC release procedure, MR-DC release etc. In the current running CR, the same principle is extended to support legacy handovers as well. However, we think the same principle can be applied for CHO case well. For example, when a UE executes CHO, the UE continues the TA timer for the candidate cells.

We think it is beneficial to extend the same principle to any mobility mechanism i.e., to any use case where the UE performs a reconfiguration with sync procedure on the MCG. This can includesa PCell handover, a CHO, LTM and CLTM. RAN2 already agreed that DAPS and (C)LTM cannot be configured to a UE concurrently. For simplicity, it may be better to generalize that for any reconfiguration with sync resulting an a MAC reset of the MCG, when CLTM candidate cell is configured to the UE, the UE continues the TA timers for the CLTM candidate cells.

Proposal 4: TA timer for a CLTM candidate cell is continued (not stopped) when MAC reset is performed due to a reconfiguration with sync procedure.

A TA value of a candidate cell can be used only when the candidate cell is still available for CLTM. If a candidate cell configuration of the candidate cell for CLTM is released, the UE can no longer use the TA value associated with this candidate cell. After the candidate cell configuration is released, it is possible that the UE is configured with the same candidate cell configuration ID. If this happened while the TA timer for the candidate cell for which the candidate cell configuration is released is still running, this may cause problems with accessing the candidate cell during LTM cell switch. We think it is beneficial to stop the TA timer of the candidate cell when the corresponding candidate cell configuration is released.

Proposal 5: Stop TA timer for a CLTM candidate cell when the candidate cell configuration corresponding to the CLTM candidate cell is released.

## Fast MCG failure information

If a UE is configured with timer T316 and the UE detects a failure, the UE initiates MCG failure information procedure. When MCG failure information procedure is triggered, the UE stops CLTM evaluation and starts the timer T316. Since CLTM supports subsequent cell switching, the UE needs to restart or resume CLTM evaluation at some point. There is no procedure based on which the UE can resume or restart CLTM evaluation.

According to the current running CR, UE starts performing CLTM condition evaluation only in 2 cases:

- Start case 1) if the received LTM-Config includes the field ltm-ServingCellExecutionCondition set to setup

- Start case 2) On executing LTM to a candidate cell and ltm-ExecutionCondition is configured within the LTM-Candidate IE for the selected LTM candidate configuration

There are 2 scenarios where the CLTM condition evaluation is stopped:

- Stop case 1) During CLTM execution. In this case, the UE restarts performing the CLTM evaluation after applying the candidate cell configuration, according to Start case 2 above.

- Stop case 2) On initiating MCG failure information procedure. In this case, the current running CR does not have a procedure for the UE to resume/ start performing the CLTM condition evaluation.

We think the UE needs to restart/ resume the CLTM evaluation on successfully recovering from the MCG failure. The successful recovery can be determined based on receiving an RRC reconfiguration message for the MCG, while timer T316 is running.

There are multiple ways to handle this. In a first option, the UE stops CLTM evaluation on initiating MCG failure information procedure and resumes CLTM evaluation if RRC reconfiguration for MCG is received while T316 is running. This requires clear handling of starting and stopping of the CLTM evaluation. The purpose of stopping CLTM evaluation during MCG failure recovery procedure is to avoid a case where the UE executes a CLTM while the MCG failure recovery is in progress, as this can lead to misalignment between the UE and the network. In a second option, the UE executes CLTM only if MCG failure is not in progress. This is simpler in stage 3 as clear handling of starting and stopping of CLTM evaluation is not required in the specification.

Proposal 6: UE resumes CLTM evaluation after successfully recovery from MCG failure based on one of the following options:

- Option 1: resume CLTM evaluation if RRC reconfiguration for MCG is received while T316 is running;

- Option 2: executing CLTM only when timer T316 is not running.

## MAC CE for early TA acquisition

In RAN2 #128, RAN2 agreed that a network may transmit new MAC CE to inform the candidate cell’s TA information to UE via LTM Candidate Timing Advance Command MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. The network can transmit the LTM Candidate Timing Advance Command MAC CE in response to receiving a RA preamble from a UE for early UL sync procedure. The UE may have performed early UL sync for multiple candidate cells. As a result, the network may have more than one TA values to be sent to the UE i.e., 1 TA value per candidate cell. The network may transmit the LTM Candidate Timing Advance Command MAC CE to manage the TA of a candidate cell (e.g., for restarting TA timer or updating TA value). To improve the efficiency of signaling and simplify downlink scheduling, it is beneficial for the network to transmit the LTM Candidate Timing Advance Command MAC CE to with more than one TA values where each TA value corresponds to a different candidate cell.

Proposal 7: LTM Candidate Timing Advance Command MAC CE to include more than one TA values.

# Conclusions

The contribution discussed aspects related to open issues for CLTM. The following observations and proposals are made:

Proposal 1: (MAC-14) If UE receives (Enhanced) LTM cell switch command MAC CE for LTM to a candidate cell while TA timer for the candidate cell is running, the UE uses the TA value indicated in the LTM cell switch command MAC CE and not the TA value received prior to receiving the LTM cell switch command MAC CE.

Proposal 2: (MAC-14) if UE receives (Enhanced) LTM cell switch command MAC CE for LTM to a candidate cell while TA timer for the candidate cell is running, the UE stops the TA timer for this candidate cell.

Proposal 3: RAN2 to discuss and agree one of the following options for handling TA timer of a CLTM candidate cell that is target cell for L3 handover (i.e., legacy HO or CHO):

Option 1: stop the TA timer for this CLTM candidate cell while continuing the TA timers for the other CLTM candidate cells.

Option 2: continue the TA timer for all CLTM candidate cells.

Proposal 4: TA timer for a CLTM candidate cell is continued (not stopped) when MAC reset is performed due to a reconfiguration with sync procedure.

Proposal 5: Stop TA timer for a CLTM candidate cell when the candidate cell configuration corresponding to the CLTM candidate cell is released.

Proposal 6: UE resumes CLTM evaluation after successfully recovery from MCG failure based on one of the following options:

- Option 1: resume CLTM evaluation if RRC reconfiguration for MCG is received while T316 is running;

- Option 2: executing CLTM only when timer T316 is not running.

Proposal 7: LTM Candidate Timing Advance Command MAC CE to include more than one TA values.