3GPP TSG-RAN WG2 Meeting #131	R2-2505789
Bengaluru, India, August 25 – 29, 2025

Agenda Item:	8.5.4

Source:	InterDigital

Title:	Remaining issues for time domain adaptation

Document for:	Discussion, Decision

# Introduction

The Rel-19 WID on Network Energy Savings for NR has the following objective on adaptation of common signalling and channels:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution discusses remaining open issues for this objective.

# Discussion

RAN2 already agreed that time domain RACH adaptation is not modelled as RA feature, and it can be supported for all RACH partitioning features, i.e. RedCap, SDT, and Slicing, and feature combinations. There were some exceptions agreed in RAN2#130, whereby RACH adaptation is not applied to the following cases (for non-serving cell case): PDCCH order RA for additional PCI, L3 HO command, BFR, LTM CSC MAC-CE, and PDCCH order for LTM early UL sync.

In the cases mentioned, R2 excluded the support of msg-1 based repetition (CE) feature when it comes to RACH partitioning for additional RACH resources. This is because additional RACH are configured in RACH-ConfigCommon, which also contains legacy RACH resources, which supports only one feature combination based partitioning parameter. One FFS therefore from RAN2#130 is how to deal with RACH resource set selection for such case:

RAN2 will select one of the following options. (CE here means MSG1 repetition).

Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17)

Both options will have some specification impact, e.g. either to MAC or to RRC specs, to clarify the intended behaviour. With option 1b, the limitation is specified for the UE, while with option 1c, the limitation is specified for the network. From our perspective, both options work. Option 1c avoid the scenario all together, while Option 1b clarifies the behaviour restriction on the R19 UE side. A text proposal for option 1b is provided in the Annex.

Observation 1: 	Both options 1b and 1c are acceptable. Option 1c limits the network configuration for a PRACH partition to mix ms1-based CE PRACH resources in a RACH-ConfigCommon with non-CE resources. Option 1b clarifies the behaviour restriction on the R19 UE side.

Proposal 1:	RAN2 to select between option 1b and 1c based on whether there is a need for the network to mix CE and non-CE RA in RACH-ConfigCommon. Consider TP in Annex if option 1b is agreed.

# Conclusion

This contribution addresses remaining open issues time domain adaptation for PRACH. The following proposals are provided:

PRACH partition to mix ms1-based CE PRACH resources in a RACH-ConfigCommon with non-CE resources. Option 1b clarifies the behaviour restriction on the R19 UE side.

Proposal 1:	RAN2 to select between option 1b and 1c based on whether there is a need for the network to mix CE and non-CE RA in RACH-ConfigCommon. Consider TP in Annex if option 1b is agreed.

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

# TP for TS 38.321 (option 1b)

### 5.1.1b	Selection of the set of Random Access resources for the Random Access procedure

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with msg3-Repetitions set to true and set(s) of Random Access resources without msg3-Repetitions set to true and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with msg3-Repetitions set to true:

2>	assume Msg3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume Msg3 repetition is not applicable for the current Random Access procedure.

1>	if contention-free Random Access Resources have been provided for this Random Access procedure in the LTM Cell Switch Command MAC CE and a non-zero Msg1 repetition number is indicated in the LTM Cell Switch Command MAC CE:

2>	assume that Msg1 repetition is applicable and that the Msg1 repetition number applicable for the current Random Access procedure is the Msg1 repetition number indicated in the LTM Cell Switch Command MAC CE.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and a Msg1 repetition number is indicated in rach-ConfigDedicated:

2>	assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure is the Msg1 repetition number indicated in rach-ConfigDedicated.

1>	else if contention free Random Access Resources have not been provided for this Random Access procedure and the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources with msg1-Repetitions set to true and set(s) of Random Access resources without msg1-Repetitions set to true:

2>	if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 8 and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum8:

3>	assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 8.

2>	if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 4 and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum4:

3>	assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 4.

2>	if the BWP selected for the Random Access procedure is configured with set(s) of Random Access resources associated with Msg1 repetition number 2 and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum2:

3>	assume Msg1 repetition is applicable and Msg1 repetition number applicable for the current Random Access procedure includes 2.

2>	else if the RSRP of the downlink pathloss reference is not less than any configured rsrp-ThresholdMsg1-RepetitionNumX:

3>	assume Msg1 repetition is not applicable for the current Random Access procedure.

1>	else if the BWP selected for Random Access procedure is configured only with the set(s) of Random Access resources with msg1-Repetitions set to true:

2>	assume Msg1 repetition is applicable for the current Random Access procedure;

2>	if at least one of rsrp-ThresholdMsg1-RepetitionNumX is configured:

3>	if rsrp-ThresholdMsg1-RepetitionNum8 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum8;

4>	assume Msg1 repetition number applicable for the current Random Access procedure includes 8.

3>	if rsrp-ThresholdMsg1-RepetitionNum4 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum4:

4>	assume Msg1 repetition number applicable for the current Random Access procedure includes 4.

3>	if rsrp-ThresholdMsg1-RepetitionNum2 is configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg1-RepetitionNum2:

4>	assume Msg1 repetition number applicable for the current Random Access procedure includes 2.

3>	else if the RSRP of the downlink pathloss reference is not less than any configured rsrp-ThresholdMsg1-RepetitionNumX:

4>	assume Msg1 repetition number applicable for the current Random Access procedure is the lowest Msg1 repetition number configured for this BWP.

2>	else (none of rsrp-ThresholdMsg1-RepetitionNumX is configured):

3>	assume Msg1 repetition number applicable for the current Random Access procedure is the Msg1 repetition number that configured for this BWP.

1>	if the Random Access procedure was initiated by SI request, SIB1 request, reconfiguration with sync, beam failure recovery, LTM Cell Switch, a PDCCH order for an LTM candidate cell, or a PDCCH order with the PRACH association indicator field in DCI set to 1; or

1>	if Msg1 repetition is applicable for the current Random Access procedure:

2>	PRACH occasions configured by addlRACH-Config-Adaptation in RACH-ConfigCommon of a set of Random Access resources are not applicable for this Random Access procedure.

1>	else:

2>	PRACH occasions configured by addlRACH-Config-Adaptation in RACH-ConfigCommon of a set of Random Access resources are applicable for this Random Access procedure, if available (as specified in TS 38.213 [6] and 38.212 [9]).

NOTE 1:	Void.

1>	if neither contention-free Random Access Resources nor Random Access Resources for SI request have been provided for this Random Access procedure and one or more of the features including (e)RedCap and/or Slicing and/or SDT and/or MSG3 repetition and/or MSG1 repetition is applicable for this Random Access procedure:

NOTE 2:	The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of NSAG-ID is determined by upper layers when the Random Access procedure is initiated. The applicability of (e)RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

NOTE 3:	SDT is not applicable for the Random Access procedure initiated by upper layers for MT-SDT.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else if there are more than one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure and Msg1 repetition is applicable for this Random Access procedure:

3>	select the set of Random Access resources that associated with highest repetition number among the sets of Random Access resources.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

1>	else if this Random Access procedure is initiated by PDCCH order with the PRACH association indicator field in DCI set to 1 and SSB-MTC-AdditionalPCI is configured by upper layers, as specified in clause 7.3.1.2.1 of TS 38.212 [9]:

2>	select the set of Random Access resources corresponding to the additionalPCI associated with active TCI states.

1>	else if this Random Access procedure is initiated by PDCCH order for an LTM candidate cell:

2>	select the set of Random Access resources configured in EarlyUL-SyncConfig corresponding to the carrier and the cell indicated by the field UL/SUL indicator and the field Cell indicator in the PDCCH order respectively, as specified in TS 38.212 [9].

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure by PDCCH order:

2>	if RedCap is applicable for the current Random Access procedure:

3>	if there is one set of Random Access resources available that is only configured with RedCap indication:

4>	select this set of Random Access resources for this Random Access procedure.

3>	else:

4>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if eRedCap is applicable for the current Random Access procedure:

3>	if there is one set of Random Access resources available that is only configured with eRedCap indication:

4>	select this set of Random Access resources for this Random Access procedure.

3>	else if there is one set of Random Access resources available that is only configured with RedCap indication:

4>	select this set of Random Access resources for this Random Access procedure.

3>	else:

4>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else:

3>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure in the LTM Cell Switch Command MAC CE:

2>	if RedCap is applicable for this Random Access procedure:

3>	if a non-zero Msg1 repetition number is indicated in the LTM Cell Switch Command MAC CE:

4>	select the set of Random Access resources that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

3>	else:

4>	if there is one set of Random Access resources available that is only configured with RedCap indication:

5>	select this set of Random Access resources for this Random Access procedure.

4>	else:

5>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if eRedCap is applicable for this Random Access procedure:

3>	if a non-zero Msg1 repetition number is indicated in the LTM Cell Switch Command MAC CE:

4>	select the set of Random Access resources that is only configured with eRedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

3>	else:

4>	if there is one set of Random Access resources available that is only configured with eRedCap indication:

5>	select this set of Random Access resources for this Random Access procedure.

4>	else if there is one set of Random Access resources available that is only configured with RedCap indication:

5>	select this set of Random Access resources for this Random Access procedure.

4>	else:

5>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else:

3>	if a non-zero Msg1 repetition number is indicated in the LTM Cell Switch Command MAC CE:

4>	select the set of Random Access resources that is only configured with Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

3>	else:

4>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure in rach-ConfigDedicated:

2>	if RedCap is applicable for this Random Access procedure:

3>	if Msg1 repetition number is indicated in rach-ConfigDedicated:

4>	select the set of Random Access resources that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

3>	else:

4>	if there is one set of Random Access resources available that is only configured with RedCap indication:

5>	select this set of Random Access resources for this Random Access procedure.

4>	else:

5>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if eRedCap is applicable for this Random Access procedure:

3>	if Msg1 repetition number is indicated in rach-ConfigDedicated:

4>	select the set of Random Access resources that is only configured with eRedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

3>	else:

4>	if there is one set of Random Access resources available that is only configured with eRedCap indication:

5>	select this set of Random Access resources for this Random Access procedure.

4>	else if there is one set of Random Access resources available that is only configured with RedCap indication:

5>	select this set of Random Access resources for this Random Access procedure.

4>	else:

5>	select the set of Random Access resources that not is associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else:

3>	if Msg1 repetition number is indicated in rach-ConfigDedicated:

4>	select the set of Random Access resources that is only configured with Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

3>	else:

4>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure in the BeamFailureRecoveryConfig:

2>	if RedCap is applicable for this Random Access procedure:

3>	if there is one set of Random Access resources available that is only configured with RedCap indication:

4>	select this set of Random Access resources for this Random Access procedure.

3>	else:

4>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if eRedCap is applicable for this Random Access procedure:

3>	if there is one set of Random Access resources available that is only configured with eRedCap indication:

4>	select this set of Random Access resources for this Random Access procedure.

3>	else if there is one set of Random Access resources available that is only configured with RedCap indication:

4>	select this set of Random Access resources for this Random Access procedure.

3>	else:

4>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else:

3>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

1>	else if Random Access resources for SI request have been provided for this Random Access procedure:

2>	if Random Access Resources associated with Msg1 repetition for SI request and Msg1 repetition number have been provided for this Random Access procedure:

3>	if the BWP selected for Random Access procedure is indicated by initialUplinkBWP-RedCap:

4>	if RedCap is applicable for the current Random Access procedure:

5>	select the set of Random Access Resources that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

4>	else if eRedCap is applicable for the current Random Access procedure:

5>	if there is one set of Random Access resources available that is only configured with RedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number:

6>	select this set of Random Access resources for this Random Access procedure.

5>	else:

6>	select the set of Random Access Resources that is only configured with eRedCap indication and Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

3>	else:

4>	select the set of Random Access resources that is only configured with Msg1 repetition indication and associated with the indicated Msg1 repetition number for this Random Access procedure.

2>	else:

3>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

1>	else:

2>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

=================================unchanged text omitted===================================