3GPP TSG-RAN WG2 Meeting #131			     R2-2505804

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.7.5 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining open issues on RLC enhancements for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses remaining open issues related to RLC enhancements.

Discussion

Avoid unnecessary RLC retransmissions

Following open issues are identified for unnecessary RLC retransmission avoidance during the RLC running CR discussion.

For the RLC-10, the issue is that when the stopReTxObsoleteSDU is configured at the transmitting entity but the t-RxDiscard is not configured at the peer receiving entity, the transmitting window stalling problem may not be resolved properly. To handle this problem, the proponent suggests to move the transmitting window forward based on the discard indication. We have sympathy on the issue if it is possible to configure stopReTxObsoleteSDU at the transmitting entity without configuring t-RxDiscard at the peer receiving entity. However, if the network can ensure that the transmitting entity is configured with stopReTxObsoleteSDU and the peer receiving entity is configured with the t-RxDiscard, there would be no problem, i.e., the network implementation can handle this. Thus, it does not need to capture anything about this in the specification and the UE can just rely on the network’s proper configuration for this issue.

Observation 1. To avoid window stalling problem, there should be dependency between Tx discard and Rx discard, i.e., if Tx entity is configured with Tx stop transmitting, the peer receiving entity should support Rx discard, but this can be avoided by the network implementation.

Proposal 1. (RLC-10) Nothing is specified for dependency between Tx discard and Rx discard in the specification, i.e., the network ensures proper configuration.

On top of the RLC-10, the following open issue (UE capability-04) can be discussed here.

4. Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

As explained in the observation 1, there should be dependency between Tx discard and Rx discard to avoid window stalling problem, but this actually does not mean to have association between the Tx discard capability and the Rx discard capability. This is because when the network configures Tx side discard to transmitting entity in the UE, the peer receiving entity is in the network which should support Rx side discard. This means that the Tx side discard can be configured by the network without UE’s Rx side capability. Thus, there is no need to have dependency between Tx discard capability and Rx discard capability.

Proposal 2. (UE capability-04) no dependency is defined between Tx discard capability and Rx discard capability, i.e., remove the FFS “FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.”.

For the RLC-11, the issue is that when t-PollRetransmit timer expires, if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with due to green highlight below. In our view, if all SDUs buffered whose transmissions have been stopped due to discard indication from PDCP after t-PollRetransmit timer expires, PDB of outdated SDUs is already passed and it may be useless for an application. This means that there is no need to retransmit the outdated SDU even after t-PollRetransmit timer expires and delaying the poll retransmission until a new SDU is received from the PDCP would be no problem. In addition, according to the current running RLC CR in the below box, if nothing is selected for retransmission after expiry of t-PollRetransmit due to green highlight below, the RLC include a poll in an AMD PDU, if any, as described in clause 5.3.3.2 as yellow highlight below. It means that if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP after expiry of t-PollRetransmit, an ongoing polling procedure would be stopped here since there is no more SDU to transmit and the RLC cannot check polling condition until another new SDU is received from upper layers. Therefore, we don’t think that additional condition to stop ongoing polling procedure (e.g., stopping the poll retransmission timer) and further enhancement on this part are needed.

Proposal 3. (RLC-11) No need to stop poll retransmission timer even though it already knows the status of all RLC SDUs (either positively/negatively acknowledged, or discarded) up to POLL_SN.

#### 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission (excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - consider any RLC SDU which has not been positively acknowledged for retransmission (excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1). - include a poll in an AMD PDU, if any, as described in clause 5.3.3.2.

For the RAN2 working assumption, the UE’s transmitting window stalling problem can occur when the AMD PDU including poll is missed or the STATUS report transmitted by the receiving entity is missed. This window stalling problem cannot be resolved until the STATUS report is received from the peer receiving entity. In Rel-19 XR RLC enhancement, RAN2 introduce an additional condition to trigger the STATUS report (i.e., Rx discard) at the Rx side and this can reduce missing possibility of the STATUS report. In addition, RAN2 introduce an additional condition to include a poll based on the remaining time of SDU and this can also reduce missing possibility of poll. To sum up, Rel-19 RLC would receive the STATUS report before reaching the transmitting window stalling problem in most cases. Another point is that if the UE is configured with Tx side discard, the peer entity is the network and the STATUS report can be transmitted whenever the network wants. Thus, transmitting window stalling problem would be rare case in Rel-19 RLC and RAN2 can confirm the working assumption.

Proposal 4. (RLC-11) No enhancement for poll retransmission after t-PollRetransmit timer expires, i.e., when t-PollRetransmit timer expires, if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, no SDU to retransmit the poll is selected as in the current RLC running CR.

For the RLC-14, it is about the Tx entity behaviour after receiving a discard indication for a RLC SDU from upper layers.

5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

The current RLC specification as shown above does not allow the Tx entity to discard a RLC SDU which is already submitted to the lower layer and introduce an RLC SN gap when discarding an RLC SDU. We think that this basic principle for discarding RLC SDU can be retained even for stopReTxDiscardedSDU configured. The important point in the Tx entity is to stop (re)transmission of outdated RLC SDU, not to discard the RLC SDU. Considering that anyway the RLC SDU indicated by the discard indication from upper layer will be positively acknowledged by the STATUS report with fake ACK from the Rx entity, the transmitting window should be moved forward based on this fake ACK and those RLC SDUs indicated by the discard indication would fall outside of the transmitting window soon and then it is up to UE implementation to discard the RLC SDU located outside of the transmitting window without the proposed change.

Observation 2. RLC SDUs indicated by the discard indication would fall outside of the transmitting window soon based on the STATUS report with fake ACK from the Rx entity and it is up to UE implementation to discard the RLC SDU located outside of the transmitting window.

With this in mind, buffering RLC SDUs indicated by the discard indication for a short time after receiving a discard indication is not a problem and we don’t see any beneficial to discard the RLC SDU indicated by the discard indication from upper layer, rather an RLC SN gap is introduced unnecessarily in the transmitting window. Thus, it would be better not to discard the RLC SDU indicated by the discard indication from upper layer and stopping transmission of the outdated RLC SDU would be sufficient as in the current running CR.

Proposal 5. (RLC-14) The RLC entity does not discard the RLC SDU or RLC SDU segment as in legacy, although the RLC SDU or RLC SDU segment indicated by the discard indication from upper layers has been submitted to the lower layers, i.e., no RLC specification change required.

PDCP open issue related to the RLC enhancement

The following PDCP open issue is related to the RLC enhancement.

PDCP-2 Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layers.

According to the current PDCP SN gap report triggering condition, if the discarded PDCP SDU(s) have been submitted by any RLC entity to lower layer, those PDCP SDU(s) are not considered to trigger PDCP SN gap report since the RLC entity does not discard those SDU(s) and keeps transmitting it to the peer receiving RLC entity.

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

Observation 3. When the discard indication for a SDU is received, the legacy RLC AM entity does not discard and keeps transmitting the SDU indicated by the discard indication if a segment of the SDU has been submitted by the RLC entity to the lower layer.

During the PDCP running CR discussion, one company indicates that if Rel-19 RLC AM entity is configured with Tx discard (i.e. stopReTxDiscardedSDU), the RLC AM entity stops transmitting/retransmitting a SDU segment of the discarded PDCP SDU and this can cause SN gap in the PDCP since the SDU segment of the discarded PDCP SDU will never be delivered to the peer receiving entity. We acknowledge this case can occur if Rel-19 RLC AM entity is configured with Tx discard.

Observation 4. When the discard indication for a SDU is received and Rel-19 RLC AM entity is configured with Tx discard, although a segment of the SDU has been submitted to the lower layers, SN gap in the PDCP can occur since the RLC AM entity stops transmitting a segment of the SDU.

For the observation 3, they suggest to change the SN gap report triggering condition to include a SDU segment case, i.e., “at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers”. However, we think that the proposed change is NBC and this is not limited to the SDU segment case because transmission of the discarded PDCP SDU even fully submitted by any RLC entity to lower layers can be stopped by Rel-19 RLC AM entity as well. Thus, whole possible cases should be carefully considered for this issue.

In the current text for triggering condition, ‘any’ (i.e., any RLC entity) is intentionally considered for the case that multiple RLC entities associated with one PDCP entity (see green highlight in the above box), and the following three cases should be considered.

- Case 1: stopReTxDiscardedSDU is not configured in any RLC entity;

- Case 2: stopReTxDiscardedSDU is not configured in at least one RLC entity, (i.e., some of RLC entity configured with stopReTxDiscardedSDU, but other RLC entity not configured with stopReTxDiscardedSDU);

- Case 3: stopReTxDiscardedSDU is configured in all RLC entities.

The case 1 is to support the legacy RLC entity and Rel-19 RLC AM entity which is not configured with the Tx discard. In this case, nothing needs to be changed in the current PDCP SN gap report triggering condition.

On the other hand, the case 3 is to support all Rel-19 RLC AM entities configured with the Tx discard. In this case, the current condition (i.e., “the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers”) is not needed, but a new condition should be included here since actual PDCP SN gap in the PDCP would be determined based on successful delivery of the discarded PDCP SDU by the RLC entity, (e.g., “successful delivery of the discarded PDCP SDU(s) have not been confirmed by any RLC entity.”)

In the case 2, triggering condition should be determined by the RLC entity which is not configured with Tx discard since this RLC entity will transmit the SDU or segment thereof submitted to lower layer until the ACK for the SDU is received. In this case, however, the third condition should be updated to cover the aforementioned situation, (i.e., “the discarded PDCP SDU(s) have not been submitted by any RLC entity which is not configured with stopReTxDiscardedSDU to lower layers.”).

Based on the explanation so far, if something needs to be changed, it should be updated like below:

For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - if stopReTxDiscardedSDU is not configured in any RLC entity: - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. - else if stopReTxDiscardedSDU is not configured in at least one RLC entity: - the discarded PDCP SDU(s) have not been submitted by any RLC entity which is not configured with stopReTxDiscardedSDU to lower layers; and - successful delivery of the discarded PDCP SDU(s) have not been confirmed by any RLC entity which is configured with stopReTxDiscardedSDU. - else (i.e., stopReTxDiscardedSDU is configured in all RLC entities): - successful delivery of the discarded PDCP SDU(s) have not been confirmed by any RLC entity.

Even though we suggest the above text proposal, we still doubt whether this much change is really needed to consider Rel-19 RLC AM enhancement since anyway the gain is to reduce only one re-ordering time in the PDCP. In addition, if there is at least one discarded PDCP SDU which has not be submitted by any RLC entity to lower layers, the PDCP SN gap report will be triggered by the current triggering condition. The only case not covered by the current triggering condition is that all discarded PDCP SDU(s) have been submitted by all RLC entities to lower layer, but this means that all discarded PDCP SDU(s) are already transmitted at least once. In this case, if at least one discarded PDCP SDU is not confirmed by successful delivery yet, this discarded PDCP SDU would already generates a PDCP SN gap at the receiving side, so the gain would be small and we prefer to keep the current PDCP SN gap triggering condition.

Proposal 6. (PDCP-2) PDCP SN gap report triggering condition is not updated considering Rel-19 RLC AM enhancement.

Rel-18 leftovers

For the RLC-12, the issue is that SN gap report is lost during handover and the t-Reordering starts unnecessarily at the target RAN node as explained in [1-2]. In RAN2#129, it was concluded that RAN2 doesn’t see a big problem to solve for Rel-18, but discuss whether this problem needs to be solved in Rel-19 XR. In our view, it is still doubtful whether this new triggering condition for a PDCP SN gap report is needed since this is not a critical issue and there is only one t-reordering delay without this enhancement. In addition, as explained below, one proposed solution causes unnecessary PDCP SN gap report retransmission at the target RAN node.

Proposal 7. (RLC-12) No enhancement is needed for PDCP SN gap report handling during UE mobility.

That being said, if majority companies want to introduce this enhancement in Rel-19 XR, the following point should be considered to choose a solution for this issue.

As per the contributions on this issue in the last RAN2 meeting, there would be two options for this issue:

- Option 1: The transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers. [1]

- Option 2: UE triggers a PDCP SN gap report when it receives a status report in which an SDU indicated in a previous SN gap report is negatively acknowledged. [2]

In option 1, the UE needs to retransmit a PDCP SN gap report to the target RAN node during PDCP Re-establishment whenever no lower-layer confirmation of successful delivery has been received for the PDCP SN gap report. The problem is that the PDCP SN gap report is successfully received at the source RAN node but just the STATUS report for the PDCP SN gap report is not yet received. In this case, the PDCP SN gap report is not lost and it doesn’t need to retransmit the PDCP SN gap report, but the option 1 retransmits the PDCP SN gap report unnecessarily. Considering that the issue would be a corner case as addressed in [2], the option 1 seems too much. We don’t think that it is desirable to have the option 1 for this issue.

Observation 5. If The transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers, unnecessary PDCP SN gap report retransmission can occur.

The option 2, on the other hand, retransmits the PDCP SN gap report to the target RAN node only when the PDCP SN gap is actually lost and there would be no unnecessary PDCP SN gap report retransmission. Thus, if RAN2 try to solve this issue in Rel-19 XR, we prefer to have the option 2.

Proposal 8. (RLC-12) If the proposal 7 is not agreeable, the UE retransmits a PDCP SN gap report when an SDU indicated in a previous SN gap report is negatively acknowledged in a PDCP status report.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. To avoid window stalling problem, there should be dependency between Tx discard and Rx discard, i.e., if Tx entity is configured with Tx stop transmitting, the peer receiving entity should support Rx discard, but this can be avoided by the network implementation.

Observation 2. RLC SDUs indicated by the discard indication would fall outside of the transmitting window soon based on the STATUS report with fake ACK from the Rx entity and it is up to UE implementation to discard the RLC SDU located outside of the transmitting window.

Observation 3. When the discard indication for a SDU is received, the legacy RLC AM entity does not discard and keeps transmitting the SDU indicated by the discard indication if a segment of the SDU has been submitted by the RLC entity to the lower layer.

Observation 4. When the discard indication for a SDU is received and Rel-19 RLC AM entity is configured with Tx discard, although a segment of the SDU has been submitted to the lower layers, SN gap in the PDCP can occur since the RLC AM entity stops transmitting a segment of the SDU.

Observation 5. If The transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers, unnecessary PDCP SN gap report retransmission can occur.

Proposal 1. (RLC-10) Nothing is specified for dependency between Tx discard and Rx discard in the specification, i.e., the network ensures proper configuration.

Proposal 2. (UE capability-04) no dependency is defined between Tx discard capability and Rx discard capability, i.e., remove the FFS “FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.”.

Proposal 3. (RLC-11) No need to stop poll retransmission timer even though it already knows the status of all RLC SDUs (either positively/negatively acknowledged, or discarded) up to POLL_SN.

Proposal 4. (RLC-11) No enhancement for poll retransmission after t-PollRetransmit timer expires, i.e., when t-PollRetransmit timer expires, if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, no SDU to retransmit the poll is selected as in the current RLC running CR.

Proposal 5. (RLC-14) The RLC entity does not discard the RLC SDU or RLC SDU segment as in legacy, although the RLC SDU or RLC SDU segment indicated by the discard indication from upper layers has been submitted to the lower layers, i.e., no RLC specification change required.

Proposal 6. (PDCP-2) PDCP SN gap report triggering condition is not updated considering Rel-19 RLC AM enhancement.

Proposal 7. (RLC-12) No enhancement is needed for PDCP SN gap report handling during UE mobility.

Proposal 8. (RLC-12) If the proposal 7 is not agreeable, the UE retransmits a PDCP SN gap report when an SDU indicated in a previous SN gap report is negatively acknowledged in a PDCP status report.

References

[1] R2-2502277	RLC enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19

[2] R2-2501764	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19