3GPP TSG-RAN WG2 #131	R2-2505836

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	  8.3.2.1

Source: 	Ericsson

Title:  	UE-side Model – Functionality Management for RRM Measurement Prediction

Document for:	Discussion and Decision

# Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved and revisited in RAN plenary #105 [1]. One of the objectives included in the SID is the Life Cycle Management (LCM) of the AI/ML model:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

Aspects like inference configuration and reporting as well as the applicability reporting are considered as part of the LCM and functionality management framework, and hence the relevant aspects for the UE-side model will be discussed in this contribution. In particular, we discuss the following topics:

- Inference configuration and reporting

- Applicability reporting

- Performance monitoring

# Discussion

## Inference and reporting configuration and applicability

### Inference configuration

In RAN2#129bis meeting the following was agreed:

The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

Additionally, the following agreements have been taken in RAN2#130 meeting:

- The following potential “inference parameters” can be considered for measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain - PW length (for temporal case A) - Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted frequency (For inter-freq domain prediction) - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

Based on above agreements, we believe that the RRM measurement configuration can be considered as the baseline. That is, the MeasConfig IE, including the triplet MeasId, MeasObjectNR and ReportConfigNR IEs can be reused and possibly extended to support the inference configuration of the UE sided AI/ML models. Therefore, we propose the following:

MeasConfig IE (including the triplet MeasId, MeasObjectNR and ReportConfigNR) is the baseline for the UE side inference and reporting configuration. Relevant enhancement to be considered in WI phase.

Association of Set A and Set B in inference configuration

The first step for inference configuration is to define Set A (the set of cells/beams the UE is going to predict) and Set B (the set of measurements the UE provides as input to the AI/ML model).

Set A and Set B depend on the type of predictions:

- Temporal domain prediction: Set A is the same as Set B, since the measurements and the predictions involve the same cells/beams set. No explicit association between Set A and Set B is needed at the inference configuration: all the cells configured in the MeasObject are considered as Set B, and the UE performs the temporal prediction to all those cells.

- Frequency domain prediction: Set A and Set B are two different sets of cells/beams in different frequencies, however no explicit association of Set A – Set B is needed. A key challenge is that some UEs may struggle to predict measurements on a given frequency (Set A) using measurements from another frequency if the frequency gap is too large. However, other UEs may still succeed in frequency domain prediction despite having the same large frequency difference from Set A. If the UE indicates via UE capabilities (or later via applicability) that to predict cells in a certain frequency (Set A), measurements on some cells in one or more other frequencies or within a frequency range(Set B) are needed, then the NW can configure the frequencies to measure and to predict by using legacy measurement framework (e.g., via measConfig, and measObject) accordingly. The UE selects the frequencies to measure among the configured frequencies and performs the frequency domain prediction.

In AI for PHY and for beam management use case the association between set A and set B has been agreed and explicitly captured by associating the inference resources (Set A) to the legacy CSI resources (Set B) configured as part of csi-ReportConfig. This was necessary as the set B resources could potentially change dynamically and might be unknown to the UE if not explicitly associated to the set A for inference purpose.

In AI for PHY there is a direct association between Set A and Set B, given that Set B can be unknown to the UE if not explicitly configured.

However, in AI for Mobility, such a tight association of the input and output is not needed, and network should be able to figure out the required measurement resources from UE capability or applicability information. Based on this understanding, the following is proposed for the inference configuration of temporal and frequency predictions:

Unlike AI for PHY, for temporal and frequency domain predictions configuration, no explicit association between Set A and Set B is needed.

For temporal prediction, all the cells detected by the UE can be potentially considered as Set B, and the UE performs the temporal prediction to all/some of those cells.

For frequency prediction, the UE indicates via UE capability or applicability, which frequencies or frequency range are needed to measure (Set B), to perform predictions on target frequencies (Set A) and the NW configures those frequencies via the MeasObject(s). The UE selects its Set B among the configured MeasObject(s).

### Prediction Report

In RAN2#129bis meeting the following agreement have been taken:

For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

Based on this agreement and since the RRM measurement framework is used for inference configuration and reporting, the simplest solution is to use the ReportConfigNR IE to configure the inference reporting.

The reporting information (and so the relevant configuration in ReportConfigNR IE), depends on the type of predictions:

- Temporal prediction (Case A): one or more prediction samples inferred in the prediction window

- Temporal prediction (Case B): latest L3 measurement/predicted RSRP values available at the time of reporting

- Frequency prediction: list of cell IDs and their associated predicted values

For temporal domain prediction Case A, network configures the UE to include one or more prediction samples collected in the prediction window in the Measurement Report. The report can additionally include the current measurement results, if requested by network.

For temporal domain prediction Case B UE reports latest L3 measurement/predicted RSRP values available at the time of reporting (if UE fulfils RAN4 requirements it does not matter the latest sample is prediction or measurements).

The reporting for frequency domain prediction includes a list of cell/beam identifiers and their predicted measurement results.

### Applicability Report

In RAN2#129bis meeting, the following agreement were taken:

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference

Concerning the applicability indication the following agreements are made in the meeting Ran2#130.

  - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case)

  - Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress)

  - Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

  - Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

  - Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes.

  - Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress).

  - No prohibit timer is introduced.

  - Associated ID should be optionally configurable for training and inference (i.e. it may not be mandatorily required for all training/inference configurations).  FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

Partial configuration

In particular, in RAN2#130 meeting the following has been agreed:

Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress)

Option A refers to the scenario when the UE receives the full inference configuration from the network and based on that, the applicability is reported. Alternatively, with Option B the UE receives a subset of the inference configuration (i.e. a partial configuration) and uses it to decide if the AI model is applicable.

It is expected that an agreement on the selection of the inference parameters to use for the Option B would require a lot of discussion in RAN2 and, considering the definition of the full inference configuration (which is used in Option A) for the different type of prediction is still not completed, we propose to finalize the definition of the full inference configuration before discussing Option B.

RAN2 first discuss full inference configuration (Option A), before discussing partial inference configuration (Option B).

Prior to supporting partial inference configuration in WI phase, RAN2 answer the question of what are the meaningful differences (if any) between Option A and Option B?

Toggling between inference and legacy RRM measurements

Concerning the applicability reporting for the AI/ML for Mobility use case, as shown above AI for PHY agreements are taken as baseline, however, we think there is a substantial difference between AI for PHY and AI for Mobility. In AI for PHY the existing mechanism in activating and deactivating the CSI reports based on the measurement and/or prediction is very handy and helps in designing the solution merely based on applicability reporting. However, in AI for Mobility upon receiving an applicability reporting indicating that AI based functionality is not applicable, network should use RRC signal to reconfigure the UE with the legacy RRM measurements. This might become a bottleneck as the UE might bounce between applicability and non-applicability often. Instead, we think network should be able to pre-configure the UE with legacy RRM measurement configuration as well as inference configurations to avoid unnecessary signalling and unnecessary coordination upon change of the applicability status.

Unlike AI for Beam Management, wherein upon UE decision on model/functionality becoming in-applicable, gNB-DU deactivates the model using a MAC CE, in AI for Mobility, deactivating the AI/ML model/functionality implies for the gNB-CU a more costly RRC reconfiguration.

In this approach, the UE should be able to toggle between different configurations (legacy RRM measurement configuration and inference configuration) depending on the applicability status.

- If the inference configuration becomes inapplicable, the UE falls back to the (preconfigured) RRM measurement configuration and performs measurements

- If the inference configuration becomes applicable, the UE switches to the (pre-configured) inference configuration

- Sending applicability indication when toggling between RRM measurements and/or prediction configurations should be optionally configured by the network via otherConfig.

Therefore, we propose the following:

RAN2 discuss UE’s toggling mechanism between preconfigured legacy RRM measurements configuration and inference configuration based on applicability status.

For the toggling mechanism, if the inference configuration becomes inapplicable, the UE immediately falls back to the preconfigured RRM measurement configuration and proceeds with performing legacy measurements.

For the toggling mechanism, if the inference configuration becomes applicable, the UE immediately switches to the inference configuration i.e., stop performing measurements and start performing predictions.

Sending applicability report to the network is optional i.e., if network request for it otherwise UE toggles between legacy RRM configuration and inference configurations.

## Performance Monitoring

Regarding the performance monitoring in AI for mobility, RAN2#129bis agreed that, for UE-side model, the performance monitoring in AI/ML air interface can be considered as a baseline. In the following RAN2#130 meeting, it has been agreed that

Agreements - For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. - For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

### General performance monitoring configuration and report signalling

For performance monitoring with UE-side model in functionality-based LCM, network or UE is expected to monitor the performance of inference configuration for AI functionality at the UE side, and thus majority part of performance monitoring and report configuration would be quite similar to inference and report configuration. In RAN2#129bis meeting, it has been agreed that the inference and report configuration for AI/ML mobility can be based on RRM measurement framework. It is reasonable to use RRM measurement and reporting framework as starting point for performance monitoring in RRM measurement prediction. Then the UE conducts performance monitoring according to the performance monitoring occasion in accordance with the configurations send by the network.

Existing RRM measurement configuration and reporting framework is baseline for performance monitoring for RRM measurement prediction for UE-side model.

In addition, we think the network should configure the UE with additional information to assist the UE conducting the performance monitoring in specific time windows (monitoring window) or for what percentage/portion of the predictions the UE should conduct performance monitoring. In addition we think the network should be able to configure the monitoring quantity based on which the UE performs monitoring and reports the configured quantity to the network. However, we think RAN2 should consult RAN4 for the exact performance monitoring metric as that stands out of the RAN2 competence. Therefore, we have the following proposal.

Network configures UE with performance monitoring window and/or percentage of prediction samples that UE should monitor in prediction window.

Network configures UE with performance monitoring metric/quantity based on which UE performs monitoring.

RAN2 consult RAN4 for the performance monitoring metric definition.

### Performance monitoring report (periodic/event triggered)

Performance monitoring reports could be triggered via two different approaches. One approach for performance monitoring report is to be triggered periodically, where the network configures the UE to report its performance monitoring results periodically (e.g. every nth occasion of inference occasions, or every a given time). However, such periodic reporting mechanisms is not efficient in capturing critical and context-sensitive events. This implies that an event-based reporting framework for performance monitoring report is also needed to enable UE to evaluate and report performance metrics only when specific condition(s) is(are) met, such as statistical performance metric is below or higher than identified threshold, RSRP difference is higher than a threshold, data/prediction drift, or confidence degradation. The event-based reporting enables not only uplink signaling overhead reduction but also enhances the responsiveness and precision of mobility management by delivering actionable insights when predictive model performance deteriorates. This in turn would enable the network to take actions to cope with performance degradation whenever it happens.

UE reports performance monitoring outcome in periodic or event triggered basis, according to the performance monitoring report configuration configured by the network. FFS performance monitoring report triggering events.

### Network-side performance monitoring

In intra-frequency temporal RRM measurement prediction case A and B, the UE can be configured to provide the actual measurements for a portion of the prediction instances associated to a specific inference configuration, if requested by the network. Needless to mention that the performance monitoring should be performed right at the time instances that the predictions are collected.  Therefore, we believe the performance monitoring configuration should be associated with the inference configuration for which the performance monitoring is expected by the network. So, in our understanding, the report for monitoring and inference are correlated too.

In inter-frequency RRM measurement prediction, RRM measurement prediction in one carrier frequency is based on RRM measurement results in another one or more carrier frequencies. In this case, for one inference instance, it is expected that the time instances for these RRM measurements and RRM measurement prediction in different frequencies are very close to each other. RRM measurement in one carrier frequency is filtering-based results and time-domain correlated, and moreover legacy RRM measurement and report configuration only indicates report periodicity rather than exact measurement time instances for one reported RRM measurement. Therefore, similar to intra-frequency temporal RRM measurement predication, RRM measurement and report of monitoring resources should be linked with corresponding inference configuration in inter-frequency RRM measurement prediction, especially when partial monitoring occasion is configured.

For RRM measurement prediction with UE-side model, RRM measurement and report of monitoring resources should be associated/linked with corresponding inference configuration. FFS how to link the performance monitoring configuration and the inference configuration.

For the network side performance monitoring, UE reports actual L3 measurements to the network associated to the predictions of the L3 measurements for the temporal prediction.

Figure 2: Schematic view of the content of the performance monitoring configuration. Network should configure the UE with the monitoring window/percentage, monitoring quantity, reporting configuration, and association with the inference config.

### How to calculate performance metrics for RRM measurements

In case of UE sided performance monitoring, network should configure the UE with performance monitoring configuration including the performance metrics. In our understanding the performance metric can be calculated in various forms. However, it is important to calculate the performance monitoring metric only across the cells/beams whose predictions are reported in measurement report to the network. In a normal implementation, the measurement report will include strongest cells in terms of prediction quality. Prediction accuracy of the strongest cells normally comes with a higher accuracy than the prediction of the weak cells. Given that the mobility decisions are made based on the predictions included in the report (including the strongest cells), the accuracy of the predictions of the cells which are not included in the report does not affect the mobility performance hence performance of such predictions, might mislead the UE and network on the effective performance of the model.

If a cell and its associated prediction is not listed in the report (after sorting) its prediction accuracy holds no practical value for the network.

UE calculates performance metric in a given frequency only across the cells/beams whose predictions are included in the measurement report sent to the network.

## Other

### Predicted quantities

In RAN2#129bis the following was agreed:

Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP.

This agreement indicates that RSRP is treated as baseline quantity for event predictions, which means that the RSRP is also the baseline quantity for RRM measurement predictions (since event predictions are based on RRM measurement predictions when the indirect approach is used).
However, we think that more quantities are reasonable to add. Although most of the simulations done during the Study Item were using RSRP as predicted quantity, we do not see any reason to limit the study to the RSRP quantity only since there is no indication that the results would be significantly different for other quantities than for RSRP. Therefore, other measurement quantities (e.g., RSRQ and SINR) can be considered as input/output quantity to the AI/ML model as well. From a RAN2 point of view, it may even cause additional specification impact to exclude these quantities, as they are already part of the configuration. However, the introduction of RSRQ and SINR as additional prediction quantities will require further RAN4 work since new requirements need to be specified.

Predicted RSRQ and SINR can be considered as quantities for prediction, besides predicted RSRP.

### Spatial predictions

In RAN2#130, the following was agreed:

- The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s).

- The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

And in WID [2], the following is stated:

Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI

Based on those, the remaining use case to consider is NW-side beam prediction in spatial domain.

We do not see any benefit in using NW-side spatial domain beam predictions, so this use case can be down prioritized.

NW-side spatial domain beam predictions is not included in Rel-20 WI.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In AI for PHY there is a direct association between Set A and Set B, given that Set B can be unknown to the UE if not explicitly configured.

Observation 2	Unlike AI for Beam Management, wherein upon UE decision on model/functionality becoming in-applicable, gNB-DU deactivates the model using a MAC CE, in AI for Mobility, deactivating the AI/ML model/functionality implies for the gNB-CU a more costly RRC reconfiguration.

Observation 3	If a cell and its associated prediction is not listed in the report (after sorting) its prediction accuracy holds no practical value for the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	MeasConfig IE (including the triplet MeasId, MeasObjectNR and ReportConfigNR) is the baseline for the UE side inference and reporting configuration. Relevant enhancement to be considered in WI phase.

Proposal 2	Unlike AI for PHY, for temporal and frequency domain predictions configuration, no explicit association between Set A and Set B is needed.

Proposal 3	For temporal prediction, all the cells detected by the UE can be potentially considered as Set B, and the UE performs the temporal prediction to all/some of those cells.

Proposal 4	For frequency prediction, the UE indicates via UE capability or applicability, which frequencies or frequency range are needed to measure (Set B), to perform predictions on target frequencies (Set A) and the NW configures those frequencies via the MeasObject(s). The UE selects its Set B among the configured MeasObject(s).

Proposal 5	For temporal domain prediction Case A, network configures the UE to include one or more prediction samples collected in the prediction window in the Measurement Report. The report can additionally include the current measurement results, if requested by network.

Proposal 6	For temporal domain prediction Case B UE reports latest L3 measurement/predicted RSRP values available at the time of reporting (if UE fulfils RAN4 requirements it does not matter the latest sample is prediction or measurements).

Proposal 7	The reporting for frequency domain prediction includes a list of cell/beam identifiers and their predicted measurement results.

Proposal 8	RAN2 first discuss full inference configuration (Option A), before discussing partial inference configuration (Option B).

Proposal 9	Prior to supporting partial inference configuration in WI phase, RAN2 answer the question of what are the meaningful differences (if any) between Option A and Option B?

Proposal 10	RAN2 discuss UE’s toggling mechanism between preconfigured legacy RRM measurements configuration and inference configuration based on applicability status.

Proposal 11	For the toggling mechanism, if the inference configuration becomes inapplicable, the UE immediately falls back to the preconfigured RRM measurement configuration and proceeds with performing legacy measurements.

Proposal 12	For the toggling mechanism, if the inference configuration becomes applicable, the UE immediately switches to the inference configuration i.e., stop performing measurements and start performing predictions.

Proposal 13	Sending applicability report to the network is optional i.e., if network request for it otherwise UE toggles between legacy RRM configuration and inference configurations.

Proposal 14	Existing RRM measurement configuration and reporting framework is baseline for performance monitoring for RRM measurement prediction for UE-side model.

Proposal 15	Network configures UE with performance monitoring window and/or percentage of prediction samples that UE should monitor in prediction window.

Proposal 16	Network configures UE with performance monitoring metric/quantity based on which UE performs monitoring.

Proposal 17	RAN2 consult RAN4 for the performance monitoring metric definition.

Proposal 18	UE reports performance monitoring outcome in periodic or event triggered basis, according to the performance monitoring report configuration configured by the network. FFS performance monitoring report triggering events.

Proposal 19	For RRM measurement prediction with UE-side model, RRM measurement and report of monitoring resources should be associated/linked with corresponding inference configuration. FFS how to link the performance monitoring configuration and the inference configuration.

Proposal 20	For the network side performance monitoring, UE reports actual L3 measurements to the network associated to the predictions of the L3 measurements for the temporal prediction.

Proposal 21	UE calculates performance metric in a given frequency only across the cells/beams whose predictions are included in the measurement report sent to the network.

Proposal 22	Predicted RSRQ and SINR can be considered as quantities for prediction, besides predicted RSRP.

Proposal 23	NW-side spatial domain beam predictions is not included in Rel-20 WI.

# References

R2-242393 - Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR, OPPO, RAN#105, September 2024.