3GPP TSG-RAN WG2 #131	R2-2505837

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	  8.3.2.2

Source: 	Ericsson

Title:  	UE Sided model – Functionality Management for RRM Event Prediction

Document for:	Discussion and Decision

# Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. One of the objectives included in the SID is the Life Cycle Management (LCM) of the AI/ML model:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

This contribution discusses the aspects related to the inference and report configuration for the UE sided model, as part of the LCM framework. Regarding performance monitoring in AI for mobility, RAN2#129bis agreed that, for UE side model, performance monitoring in AI/ML air interface can be studied as a baseline for AI/ML aided mobility, i.e. NW-side and UE-side performance monitoring. This contribution shares our views on different aspects of configuration, applicability and performance monitoring in AI/ML for mobility event prediction.

# Discussion

In RAN2#129bis meeting, the following agreements have been taken:

For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

## Supported use case

In general, there are two plausible approaches for indirect event predictions

- Evaluating the event triggering condition using temporal prediction Case A

- Evaluating the event triggering condition using temporal prediction Case B

Temporal prediction Case B is to reduce the measurement effort at the UE. Irrespective of how the temporal prediction case B will be captured (one could argue it can be done by UE implementation without network involvement) the predictions could be potentially used with RRM measurements to evaluate the event triggering criteria.

Temporal prediction Case A on the other side clearly requires a synergy between UE and network and network should configure the prediction window in which the UE performs prediction in such window and how many samples should be used when evaluating the report triggering event.

In light of the above we propose the following.

Temporal prediction Case A can be used in event prediction.

Temporal prediction Case B can be used in event prediction.

Designing a single solution for event prediction

In the meeting RAN2#130, it has been agreed to leave the direct vs indirect event prediction to the UE implementation.

Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

In our view, this was a positive step toward a methodology-independent RRC specification. However, it should be clearly stated that the UE-side event prediction method (direct or indirect) should not result in different standardized solutions at the RRC level. However, when it comes to the content of the report generated by the UE there might be clear differences between direct and indirect approaches. For example, in direct approach the UE may generate a report containing the probability of an event being fulfilled in a certain time window, while in indirect approach the content of the report can be potentially the predicted RRM measurements. Hence there might be fundamental differences between UE’s report to the network based on its model choices.

We also note that RAN2 already agreed to have a functionality level management and model level LCM is ruled out. And hence RAN2 should not spend time in designing solutions working for a specific model type.

In light of the above, we still believe that direct event prediction could result in a separate set of configurations and, in particular, different reporting from the UE (In our understanding in direct event prediction there is no measurement prediction to be reported), whereas the indirect approach maximizes reusability of RRM measurement predictions (Cases A and B). Indirect event prediction is simpler and improves consistent UE behavior across the network.

Direct event prediction might lead to a different report (e.g., probability of event in a future time window) than indirect event prediction (e.g., RSRP prediction in the future).

RAN2 already ruled out model level LCM and should not design solution per model type.

RAN2 agrees that UE implementation choices related to the model - e.g., using direct or indirect event prediction - should not create any impact on the specification.

RAN2 first focus on designing signaling solution for the indirect event prediction. If direct event prediction comes for free, it can be left to UE implementation, otherwise it should be ruled out (to avoid different RRC solutions for different UE implementation).

## Event prediction configuration

The prediction of an event, using indirect approach, can be seen as evaluating the entry condition of that event based on the predicted RSRP value, instead of the actual measurements. As an example, the predicted A3 event using temporal prediction is described in the following:

#### 5.5.4.4 Predicted Event A3 (Neighbour becomes offset better than SpCell in the future time instance t) The UE shall: 1> consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled; 1> consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled; 1> use the SpCell for Mp, Ofp and Ocp. NOTE 1: The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR. Inequality A3-1 (Entering condition) Mn[t] + Ofn + Ocn – Hys > Mp[t] + Ofp + Ocp + Off Inequality A3-2 (Leaving condition) #### Mn[t] + Ofn + Ocn + Hys < Mp[t] + Ofp + Ocp + Off

Here [t] denotes the time step the UE should take into account when performing the temporal RRM predictions and evaluate the A3 entry conditions and Mn[t] and Mp[t] denote the temporal prediction of the neighboring and serving cells for the future time instance [t], respectively. Once the evaluation of the A3 entry condition based on the temporal domain predictions and possibly earlier measurements of the serving and neighboring cell indicates fulfilment of the predicted A3 event for a time period of Time-To-Trigger (TTT), the UE can trigger a report to the network including the available measurements and predictions.

Currently, the events used for RRM measurements and mobility procedures are defined in ReportConfigNR IE, included in the MeasConfig. Considering that the predicted events can be defined in a similar way as the mobility events, it is proposed to use ReportConfigNR IE to configure the predicted events (e.g. pA1, pA2, …, pA6), in fields specific to the event predictions, to distinguish them from the RRM events.

Reuse the events A1 to A6 in reportConfigNR IE to configure the event that should be predicted by the UE.

Configuration parameters for event prediction based on Case A and B

The configuration of the event prediction based on Case A may include the prediction window size (how far into the future the UE should predict) and how many samples in such prediction window should be predicted. However, in case of event prediction based on temporal prediction Case B, the configuration can be different than Case A. In case B the network may need to instruct the UE to use specific skipping pattern or the measurement reduction rate or maybe both can be left to UE implementation and network just configure the UE whether it is allowed to use temporal prediction Case B for specific measurement object or not. Beside the prediction configuration it is important to pay attention to where such configuration should be configured in RRM measurement procedure.

In NR the following principle holds for the RRM measurements

- Measurement object indicates the UE the physical signal to sample/measure on (type and period)

- Quantity Config indicates what signal processing to be done on samples before reporting/event evaluation

- Report Config indicates how (periodic, event) and what quantities (RSRP, RSRQ, SINR) to report

In our understanding the temporal prediction Case A is closer to the report config as it addresses the question of how to report and what to report. In other words, it does not impact UE’s measurement and signal processing algorithm. However, the temporal prediction Case B configuration is closer to the measurement object configuration as it addresses the question how to sample/measure and also replace some of the measurement samples with the prediction. In other words, the temporal prediction Case B changes how the UE performs the measurements and applies the signal processing algorithm.

reportConfigNR is used as baseline to include the additional configurations concerning temporal prediction Case A for event prediction, including prediction window and the number of samples to be predicted.

RAN2 discuss if measObjectNR is used as baseline to include the additional configuration concerning temporal prediction Case B for event prediction, if any configuration is needed for temporal prediction Case B.

## Applicability report

Applicability report related to event prediction is in many ways similar to the applicability reporting of RRM prediction. Therefore, it is reasonable to take the agreement for applicability report procedures for the RRM measurement prediction as baseline for the applicability procedures for the RRM event prediction. However, there might be cases where differences are found, and they should then be treated.

Take applicability procedure for RRM measurement prediction as baseline for the RRM event prediction applicability procedures and only treat the differences in discussion related to RRM event prediction.

## Performance monitoring

In last meeting, agreements were made related to performance monitoring. Those agreements were made for both RRM measurement prediction and RRM event prediction together. See the following table:

Agreements (UE-side model) For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. - For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

Since the agreements in last meeting are related to both prediction modes it is reasonable to use one of them as baseline for the other to limit overlapping work in the future. We therefore propose RRM measurement prediction should be treated as a baseline for RRM measurement event prediction performance monitoring.

Performance monitoring procedures for RRM measurement prediction is the baseline for the RRM event prediction performance monitoring procedures.

However, there will be differences between them, and these should be identified and discussed. In addition, further details need to be studied, e.g. what data that is transmitted, performance monitoring at handover etc.

RAN2 should evaluate any difference between performance monitoring of RRM measurement prediction and RRM event prediction in WI phase.

### UE-side performance monitoring (UE decision)

During the last meeting, it was agreed that UE-side performance monitoring with UE-side actions remains for further study (FFS).

Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW.  FFS which use cases.

UE-side actions might refer to the UE's ability to autonomously decide on and execute actions based on performance outcomes, for example, deactivating the inference model if needed. This can be particularly beneficial for event prediction use cases. For instance, if the UE determines that the performance of the inference configuration is unsatisfactory, it can send an inapplicability indication to the network instead of proceeding with event prediction and reporting, thereby reducing unnecessary signaling overhead.

It should be noted that triggered reports based on predictions from an inapplicable model represent pure signaling overhead, as these reports do not yield trustworthy results and should not be acted upon. Therefore, event prediction is a key use case where UE-side monitoring and UE-side decision-making should be considered. The following proposals are made:

Triggered reports based of events predicted with an inapplicable model result is a signaling overhead.

UE sided decision on performance monitoring should be considered for the event prediction. If the event prediction becomes inapplicable the UE sends applicability report to the network and deactivates the event prediction function.

## Increasing robustness of event prediction

With the introduction of the predicted RRM measurements into the mobility event evaluation the event becomes more uncertain (less reliable), particularly if the performance of the model is not good enough. Hence it may not be trusted in all scenarios and performing mobility decisions based on such reports may destabilize the network load and lead to additional failures in scenarios such as e.g., handover, DC or CA setup.

Therefore, we propose that to enhance the reliability of mobility event predictions, the network should be able to configure the UE to perform multiple consecutive predictions before triggering an event report. In other words, the network can mandate the UE to repeat predictions and only trigger the prediction-based report if the predicted event is fulfilled for multiple consecutive times, as configured by the network. This mechanism enhances the robustness of the prediction and thereby increases the reliability of network decisions based on UE event predictions. This has been exemplified in the following figure.

Figure 1: UE predicts the mobility event multiple times before reporting to the network, as configured by the network. Repetitive prediction enhances the robustness of the prediction and hence increases the reliability of network decisions based on the UE event predictions.

The network can configure the UE to perform event prediction multiple times before triggering a report to the network based on the predicted event.

# Conclusion

- Direct event prediction might lead to a different report (e.g., probability of event in a future time window) than indirect event prediction (e.g., RSRP prediction in the future).

RAN2 already ruled out model level LCM and should not design solution per model type.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Temporal prediction Case A can be used in event prediction.

Proposal 2	Temporal prediction Case B can be used in event prediction.

Proposal 3	RAN2 agrees that UE implementation choices related to the model - e.g., using direct or indirect event prediction - should not create any impact on the specification.

Proposal 4	RAN2 first focus on designing signaling solution for the indirect event prediction. If direct event prediction comes for free, it can be left to UE implementation, otherwise it should be ruled out (to avoid different RRC solutions for different UE implementation).

Proposal 5	Reuse the events A1 to A6 in reportConfigNR IE to configure the event that should be predicted by the UE.

Proposal 6	reportConfigNR is used as baseline to include the additional configurations concerning temporal prediction Case A for event prediction, including prediction window and the number of samples to be predicted.

Proposal 7	RAN2 discuss if measObjectNR is used as baseline to include the additional configuration concerning temporal prediction Case B for event prediction, if any configuration is needed for temporal prediction Case B.

Proposal 8	Take applicability procedure for RRM measurement prediction as baseline for the RRM event prediction applicability procedures and only treat the differences in discussion related to RRM event prediction.

Proposal 9	Performance monitoring procedures for RRM measurement prediction is the baseline for the RRM event prediction performance monitoring procedures.

Proposal 10	RAN2 should evaluate any difference between performance monitoring of RRM measurement prediction and RRM event prediction in WI phase.

Proposal 11	UE sided decision on performance monitoring should be considered for the event prediction. If the event prediction becomes inapplicable the UE sends applicability report to the network and deactivates the event prediction function.

Proposal 12	The network can configure the UE to perform event prediction multiple times before triggering a report to the network based on the predicted event.

# References

R2-25xxxxx- Running RRC CR for L1 measurements, vivo, RAN2#131, August 2025.