3GPP TSG-RAN WG2 Meeting #131	R2-2505887

Bangalore, India, Aug 25th – Aug 29th, 2025

Agenda item:	8.3.2.1

Source:	Qualcomm Incorporated

Title:	Functionality management for RRM measurement prediction for UE-side model

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #130 meeting [1], substantial progress was achieved in the study of LCM aspects and specification impacts for AI/ML for mobility, and many agreements were made on functionality management, scenarios for specification work for AI/ML for mobility, UE configuration and reporting to support a network-side model, data collection for network-side model, and performance monitoring. Based on the progress achieved in the SI so far, in the RAN #108 plenary meeting, a WID document [2] for the Rel-20 AI/ML for mobility WI was approved.

In this contribution, we discuss the following aspects of functionality management for RRM measurement prediction for a UE-side model:

- Applicability procedure,

- Inference configuration and reporting,

- Performance monitoring.

# Applicability procedure

Figure 1 shows the applicable functionality reporting procedure for AI/ML for mobility, which is the same as that defined for the Rel-19 AI/ML for PHY WI.

Figure 1: Applicable Functionality Reporting procedure for AI/ML for mobility

It was agreed in the last RAN2 #130 meeting [1] that:

- Multiple inference configurations may be provided to the UE in Step 3.

- Option A and Option B methods of providing inference configuration information can be considered.

Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified).

For Rel-19 AI/ML for Beam Management, the following was agreed in the last RAN2 #130 meeting [1] for applicability reporting:

Below is a similar proposal for AI/ML for mobility.

Observation 1: The following proposals on the applicability procedure for Measurement Event prediction for UE-side model (see our contribution [3]) also hold for RRM measurement prediction for UE-side model.Proposal 1: For UE-side model, the UE indicates its preference to release an inference configuration (“release configuration” flag) to indicate inapplicability in Step 4 of the Applicable Functionality Reporting procedure. The RRCReconfigurationComplete or the RRC UAI message is used for this purpose.

If the UE cannot perform the inference functionality, it provides an inapplicability report to the network as well.

Proposal 2: If the UE cannot perform the inference functionality, the UE indicates this in an inapplicability report to the network using the RRCReconfigurationComplete or the RRC UAI message.

- The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain

• PW length (for temporal case A)

• Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional).  MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI

• Measured and predicted beam pattern (optional).   MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI

- Measured and predicted frequency (For inter-freq domain prediction)

In the last RAN2 #130 meeting [1], it was agreed that the following set of potential “inference parameters” can be considered for AI/ML for mobility for the Option B method.

In the following proposal, we indicate our preference for the inference parameters that should be signalled to the UE.

Observation 2: For the UE-side model, the Skipping pattern should not be signalled as an inference parameter. It should be up to UE implementation to determine the Skipping pattern to use to skip performing measurements for measurement reduction.

Observation 3: We do not support the inclusion of a “measured and predicted beam pattern” since it is not clear to us what it means and how it can be used.

Observation 4: MRRT is a metric used for tracking the performance/gains tradeoff from measuring less often, however, it is not a configuration parameter. The UE is configured with SSB/CSI-RS transmission patterns, and the UE measures some or all of these instances. If the network wants to reduce the number of those instances of SSB/CSI-RS transmissions, that is reflected in the configuration of those signals. The same logic applies to MRRS.

Proposal 3: For the UE-side model, the following set of inference parameters are considered for AI/ML for mobility, for the Option B method.

- Measured and predicted frequency (for frequency domain prediction), and

- Associated ID.

The following is a proposal for applicability reporting in case Option B is used in Step 3.

Proposal 4: If Option B is used providing multiple inference parameter sets in Step 3, the message for inapplicability reporting (RRCReconfigurationComplete or UAI) indicates the inference parameter sets along with additional information such as measurement frequencies and cells (serving and/or neighbor), that the UE supports.

# 3 Inference configuration and reporting

For RRM measurement prediction, network can reuse the legacy RRM measurement framework to configure the UE.

We support L3 beam measurement prediction for a neighbor cell. This is useful in case handover needs to be performed to the neighbor cell since the UE can provide L3 beam measurement prediction reports to the network, which the network can use to configure the UE to perform CFRA access of the neighbor cell upon handover.

The following proposal provides the configuration information regarding what the UE should predict.

Proposal 5: For RRM measurement prediction for the UE-side model, the network configures the following parameters of a serving cell or neighbor cell to predict:

- Physical layer parameters, e.g., Cell ID (PCI), Cell frequency, SSB or CSI-RS configuration,

- Cell measurement prediction (i.e., whether cell measurement prediction is performed),

- L3 beam measurement prediction (for a neighbor cell) (i.e., whether L3 beam measurement prediction is performed for a neighbor cell).

The following proposal provides the prediction reporting configuration information.

Proposal 6: For RRM measurement prediction for the UE-side model, the network provides the following configuration for reporting predictions:

- For temporal, spatial, or frequency domain prediction, the periodicity configuration in case of periodic reporting.

- For temporal, spatial, or frequency domain prediction, predicted beam and/or cell measurements of a serving cell or a neighbor cell, to include in a prediction report.

- For temporal, spatial, or frequency domain prediction, event-triggered reporting configuration (events to be specified in the WI phase).

# 4 Performance Monitoring

Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW.  FFS which use cases.

In the last RAN2 #130 meeting [1], for a UE-side model for RRM measurement prediction, network-side monitoring and UE-assisted monitoring (where network takes the LCM decision) were agreed to be supported. It was left FFS whether UE-side monitoring, where UE takes the LCM decision, is supported.

We think that UE-side monitoring, where UE takes the LCM decision, can be supported for situations where the UE, based on its computation of the performance monitoring (PM) KPIs, determines that its AI/ML model is performing poorly. In such a case, since it is obvious to the UE that there is no point in continuing to use its AI/ML model any further, it should deactivate its AI/ML model immediately. Seeking confirmation from the network before the UE deactivates its model, results in delay and unnecessary consumption of UE resources, e.g., battery power and CPU processing, to keep running the AI/ML model. The problem is exacerbated if the network is unable to successfully send the confirmation message to the UE because of radio link  conditions.

Observation 5: UE-side monitoring should be supported when the UE, based on computation of the PM KPIs, determines that its AI/ML model is performing poorly. Seeking network confirmation of model deactivation results in unnecessary delays and consumption of UE resources, e.g., battery power, CPU/NPU processing, to keep the model running, especially when delivery of the confirmation message gets delayed, e.g., due to the radio link conditions.

Observation 6: Proposal 7 below also holds for Measurement Event prediction for a UE-side model (see our contribution [3]).

Proposal 7: UE-side monitoring, where UE takes the LCM decision, should be supported based on network configuration. That is, if the UE-computed PM KPI(s) falls outside a network configured range of values, then the UE deactivates its AI/ML model, suspends prediction reporting, and informs the network of the LCM decision.

# 5 Conclusions

Based on the above discussion, we recommend that RAN2 discuss the following observations and proposals.

Applicability procedure

Observation 1: The following proposals on the applicability procedure for Measurement Event prediction for UE-side model (see our contribution [3]) also hold for RRM measurement prediction for UE-side model.

Proposal 1: For UE-side model, the UE indicates its preference to release an inference configuration (“release configuration” flag) to indicate inapplicability in Step 4 of the Applicable Functionality Reporting procedure. The RRCReconfigurationComplete or the RRC UAI message is used for this purpose.

Proposal 2: If the UE cannot perform the inference functionality, the UE indicates this in an inapplicability report to the network using the RRCReconfigurationComplete or the RRC UAI message.

Observation 2: For the UE-side model, the Skipping pattern should not be signalled as an inference parameter. It should be up to UE implementation to determine the Skipping pattern to use to skip performing measurements.

Observation 3: We do not support the inclusion of a “measured and predicted beam pattern” since it is not clear to us what it means and how it can be used.

Observation 4: MRRT is a metric used for tracking the performance/gains tradeoff from measuring less often, however, it is not a configuration parameter. The UE is configured with SSB/CSI-RS transmission patterns, and the UE measures some or all of these instances. If the network wants to reduce the number of those instances of SSB/CSI-RS transmissions, that is reflected in the configuration of those signals. The same logic applies to MRRS.

Proposal 3: For the UE-side model, the following set of inference parameters are considered for AI/ML for mobility, for the Option B method.

- Measured and predicted frequency (for frequency domain prediction), and

- Associated ID.

Proposal 4: If Option B is used providing multiple inference parameter sets in Step 3, the message for inapplicability reporting (RRCReconfigurationComplete or UAI) indicates the inference parameter sets along with additional information such as measurement frequencies and cells (serving and/or neighbor), that the UE supports.

Inference configuration and reporting

Proposal 5: For RRM measurement prediction for the UE-side model, the network configures the following parameters of a serving cell or neighbor cell to predict:

- Physical layer parameters, e.g., Cell ID (PCI), Cell frequency, SSB or CSI-RS configuration,

- Cell measurement prediction (i.e., whether cell measurement prediction is performed),

- L3 beam measurement prediction (for a neighbor cell) (i.e., whether L3 beam measurement prediction is performed for a neighbor cell).

Proposal 6: For RRM measurement prediction for the UE-side model, the network provides the following configuration for reporting predictions:

- For temporal, spatial, or frequency domain prediction, the periodicity configuration in case of periodic reporting.

- For temporal, spatial, or frequency domain prediction, predicted beam and/or cell measurements of a serving cell or a neighbor cell, to include in a prediction report.

- For temporal, spatial, or frequency domain prediction, event-triggered reporting configuration (events to be specified in the WI phase).

Performance monitoring

Observation 5: UE-side monitoring should be supported when the UE, based on computation of the PM KPIs, determines that its AI/ML model is performing poorly. Seeking network confirmation of model deactivation results in unnecessary delays and consumption of UE resources, e.g., battery power, CPU/NPU processing, to keep the model running, especially when delivery of the confirmation message gets delayed, e.g., due to the radio link conditions.

Observation 6: Proposal 7 below also holds for Measurement Event prediction for a UE-side model (see our contribution [3]).

Proposal 7: UE-side monitoring, where UE takes the LCM decision, should be supported based on network configuration. That is, if the UE-computed PM KPI(s) falls outside a network configured range of values, then the UE deactivates its AI/ML model, suspends prediction reporting, and informs the network of the LCM decision.

# References

[1] Chair notes, RAN2 #130 meeting.

[2] RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

[3] R2-2505888, RAN2 #131 contribution, Functionality management for Measurement Event prediction for UE-side model (Qualcomm Incorporated),