3GPP TSG-RAN WG2 Meeting #131	R2-2505888

Bangalore, India, Aug 25th – Aug 29th, 2025

Agenda item:	8.3.2.2

Source:	Qualcomm Incorporated

Title:	Functionality management for Measurement Event prediction for UE-side model

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #130 meeting [1], substantial progress was achieved in the study of LCM aspects and specification impacts for AI/ML for mobility, and many agreements were made on Functionality management, scenarios for specification work for AI/ML for mobility, UE configuration and reporting to support a network-side model, Data collection for network-side model, and Performance monitoring. Based on the progress achieved in the SI so far, in the RAN #108 plenary meeting, a WID document [2] for the Rel-20 AI/ML for mobility WI was approved.

In this contribution, we discuss the following aspects of functionality management for Measurement Event prediction for a UE-side model:

- Applicability procedure,

- Inference configuration and reporting,

- Performance monitoring.

# Applicability procedure

The following proposals on the applicability procedure for RRM measurement prediction for a UE-side model that we discussed in our contribution paper to this meeting [3], also hold for the applicability procedure for Measurement Event prediction for a UE-side model (Figure 1 shows the applicable functionality reporting procedure).

Observation 1: The following first 4 proposals on the applicability procedure for RRM measurement prediction for UE-side model (see our contribution [3]) also hold for Measurement Event prediction for UE-side model.

Proposal 1: For UE-side model, the UE indicates its preference to release an inference configuration (“release configuration” flag) to indicate inapplicability in Step 4 of the Applicable Functionality Reporting procedure. The RRCReconfigurationComplete or the RRC UAI message is used for this purpose.

Proposal 2: If the UE cannot perform the inference functionality, the UE indicates this in an inapplicability report to the network using the RRCReconfigurationComplete or the RRC UAI message.

Proposal 3: For the UE-side model, the following set of inference parameters are considered for AI/ML for mobility, for the Option B method.

- Measured and predicted frequency (for frequency domain prediction), and

- Associated ID.

Proposal 4: If Option B is used providing multiple inference parameter sets in Step 3, the message for inapplicability reporting (RRCReconfigurationComplete or UAI) indicates the inference parameter sets along with additional information such as measurement frequencies and cells (serving and/or neighbor), that the UE supports.

Figure 1: Applicable Functionality Reporting procedure for AI/ML for mobility

# 3 Inference configuration and reporting

For Measurement Event prediction, RAN2 agreed to support prediction for the events A1, A2, …, A6. We have the following simple proposal.

Proposal 5: For Measurement Event prediction, besides the configuration for performing RRM measurements and/or RRM measurement prediction (relevant for the case of Indirect Event prediction), the network provides the UE with a list of events to predict (consists of a subset of the events A1, A2, …, A6).

For the handover related events A3 and A5, an event prediction report can be sent by the UE if the following conditions are met:

- Predicted source cell and target cell measurements satisfy the entering condition of the event for the configured TTT duration (Indirect Event prediction).

- Entering condition of the event is met by the actual source cell and target cell measurements and is satisfied for the configured TTT duration by the predicted source cell and target cell measurements (Indirect Event prediction). This is the Handover model Option 2 in the TR 38.744 [4].

Condition 1 is a natural condition to trigger transmission of an event prediction report. Simulation studies were carried out during the SI phase for condition 2, and it was found that the performance results were satisfactory.

Accordingly, we have the following proposal on network configuration for event prediction reporting.

Proposal 6: For Event A3 or A5, the network configures the UE for transmission of an event prediction report if the following conditions are met:

- Predicted source cell and target cell measurements satisfy the entering condition of the event for the configured TTT duration.

- Entering condition of the event is met by the actual source cell and target cell measurements and is satisfied for the configured TTT duration by the predicted source cell and target cell measurements.

# 4 Performance Monitoring (PM)

We propose supporting UE-side monitoring for Measurement Event prediction, and the proposal is the same as in our contribution paper for RRM measurement prediction [3].

Observation 2: Proposal 7 below also holds for Measurement Event prediction for a UE-side model (see our contribution [3]).

Proposal 7: UE-side monitoring, where UE takes the LCM decision, should be supported based on network configuration. That is, if the UE-computed PM KPI(s) falls outside a network configured range of values, then the UE deactivates its AI/ML model, suspends prediction reporting, and informs the network of the LCM decision.

For the handover related events, A3 or A5, we think that network-side monitoring for a UE-side model for Measurement Event prediction is feasible.

UE can provide reports after the occurrence of every handover, which includes handovers that are triggered by Event prediction, and network can compute PM KPIs based on them.

Proposal 8: For Event A3 or A5, network-side monitoring for a UE-side model for Measurement Event prediction can be supported. The UE provides reports in:

- RRCReconfigurationComplete message, in case of a successful handover,

- UEInformationResponse message, containing a successful handover report,

- RRCReestablishmentRequest message, in case of a handover failure.

In the above messages, the UE can include indications that the handover was triggered based on Event prediction.

# 5 Conclusions

Based on the above discussion, we recommend that RAN2 discuss the following observations and proposals.

Applicability procedure

Observation 1: The following first 4 proposals on the applicability procedure for RRM measurement prediction for UE-side model (see our contribution [3]) also hold for Measurement Event prediction for UE-side model.

Proposal 1: For UE-side model, the UE indicates its preference to release an inference configuration (“release configuration” flag) to indicate inapplicability in Step 4 of the Applicable Functionality Reporting procedure. The RRCReconfigurationComplete or the RRC UAI message is used for this purpose.

Proposal 2: If the UE cannot perform the inference functionality, the UE indicates this in an inapplicability report to the network using the RRCReconfigurationComplete or the RRC UAI message.

Proposal 3: For the UE-side model, the following set of inference parameters are considered for AI/ML for mobility, for the Option B method.

      - Measured and predicted frequency (for frequency domain prediction), and

      - Associated ID.

Proposal 4: If Option B is used providing multiple inference parameter sets in Step 3, the message for inapplicability reporting (RRCReconfigurationComplete or UAI) indicates the inference parameter sets along with additional information such as measurement frequencies and cells (serving and/or neighbor), that the UE supports.

Inference configuration and reporting

Proposal 5: For Measurement Event prediction, besides the configuration for performing RRM measurements and/or RRM measurement prediction (relevant for the case of Indirect Event prediction), the network provides the UE with a list of events to predict (consists of a subset of the events A1, A2, …, A6).

Proposal 6: For Event A3 or A5, the network configures the UE for transmission of an event prediction report if the following conditions are met:

- Predicted source cell and target cell measurements satisfy the entering condition of the event for the configured TTT duration.

- Entering condition of the event is met by the actual source cell and target cell measurements and is satisfied for the configured TTT duration by the predicted source cell and target cell measurements.

Performance monitoring

Observation 2: Proposal 7 below also holds for Measurement Event prediction for a UE-side model (see our contribution [3]).

Proposal 7: UE-side monitoring, where UE takes the LCM decision, should be supported based on network configuration. That is, if the UE-computed PM KPI(s) falls outside a network configured range of values, then the UE deactivates its AI/ML model, suspends prediction reporting, and informs the network of the LCM decision.

Proposal 8: For Event A3 or A5, network-side monitoring for a UE-side model for Measurement Event prediction can be supported. The UE provides reports in:

- RRCReconfigurationComplete message, in case of a successful handover,

- UEInformationResponse message, containing a successful handover report,

- RRCReestablishmentRequest message, in case of a handover failure.

In the above messages, the UE can include indications that the handover was triggered based on Event prediction.

# References

[1] Chair notes, RAN2 #130 meeting.

[2] RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

[3] R2-2505887, RAN2 #131 contribution, Functionality management for RRM measurement prediction for UE-side model (Qualcomm Incorporated).

[4] R2-xxxxxxx, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR (Rel-19), 3GPP TR 38.744, v1.1.0.