3GPP TSG-RAN WG2 Meeting #131	R2-2505889

Bangalore, India, Aug 25th – Aug 29th, 2025

Agenda item:	8.3.2.3

Source:	Qualcomm Incorporated

Title:	Data collection for UE-side model

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #130 meeting [1], substantial progress was achieved in the study of LCM aspects and specification impacts for AI/ML for mobility, and many agreements were made on Functionality management, scenarios for specification work for AI/ML for mobility, UE configuration and reporting to support a network-side model, Data collection for network-side model, and Performance monitoring. Based on the progress achieved in the SI so far, in the RAN #108 plenary meeting, a WID document [2] for the Rel-20 AI/ML for mobility WI was approved.

In this short contribution, we discuss some data collection aspects for a UE-side model for RRM measurement prediction and Measurement Event prediction.

# Data collection for training of UE-side model

The Data collection functionality is used for training a UE-side AI/ML model for RRM measurement prediction or for Measurement Event prediction.

Data collection consists of UE collecting measurements for each prediction task it is configured to perform. The UE-side model can be trained at the UE or a remote server. In case the model training is performed at the remote server, the UE transmits the collected data to the server.

Observation: For configuring the UE for data collection for RRM measurement prediction or for Measurement Event prediction, the legacy RRM measurement framework can be used, with some additional parametrization.

Proposal 1: A data collection configuration for RRM measurement prediction includes:

- Measurement objects – frequencies, cells (serving and/or neighbor) – to measure,

- Associated ID,

- Indication for starting or stopping data collection.

A data collection configuration for Measurement Event prediction includes the configuration for RRM measurement prediction and the event configuration.

UE can request the network to start or stop data collection. Upon receiving such a request, the network can provide the UE with configuration(s) for data collection.

Proposal 2: The UE can request the network to start or stop data collection for data collection for RRM and event prediction. The message used to transmit the indication is left up to the WI phase.

We do not see a good motivation for UE to perform data collection in RRC_IDLE or RRC_INACTIVE.

Proposal 3: For a UE-side model for AI/ML for mobility, the UE does not perform data collection in RRC_IDLE or RRC_INACTIVE.

For a UE-side model, there may be value in UE keeping the collected data upon handover and upon experiencing RLF, but it should be left up to UE implementation whether to do so.

Proposal 4: For a UE-side model for AI/ML for mobility, it is up to UE implementation whether to keep the collected data upon handover, upon experiencing RLF, and upon transitioning to RRC_IDLE or RRC_INACTIVE.

# 3 Conclusions

Observation: For configuring the UE for data collection for RRM measurement prediction or for Measurement Event prediction, the legacy RRM measurement framework can be used, with some additional parametrization.

Proposal 1: A data collection configuration for RRM measurement prediction includes:

- Measurement objects – frequencies, cells (serving and/or neighbor) – to measure,

- Associated ID,

- Indication for starting or stopping data collection.

A data collection configuration for Measurement Event prediction includes the configuration for RRM measurement prediction and the event configuration.

Proposal 2: The UE can request the network to start or stop data collection for data collection for RRM and event prediction. The message used to transmit the indication is left up to the WI phase.

Proposal 3: For a UE-side model for AI/ML for mobility, the UE does not perform data collection in RRC_IDLE or RRC_INACTIVE.

Proposal 4: For a UE-side model for AI/ML for mobility, it is up to UE implementation whether to keep the collected data upon handover, upon experiencing RLF, and upon transitioning to RRC_IDLE or RRC_INACTIVE.

# References

[1] Chair notes, RAN2 #130 meeting.

[2] RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.