3GPP TSG-RAN WG2 Meeting #131	R2-2505902

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.12.3

Source:	Nokia

Title:	Other MIMO issues

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we share our views on some of the open issues listed in [Post130][219][MIMO_Ph5] Running CR for 38.321 (Samsung) and ([Post130][218][MIMO_Ph5] Running CR for 38.331 (Ericsson). We also raise some further points for discussion regarding the parameter definitions in the running CR for TS 38.331.

# 2	Discussion

## 2.1	MAC-related UEIBM issues

### Behaviour upon UEI Report Triggering with TAT Expired

During RAN2#130 and subsequent offline discussions ([Post130][219][MIMO_Ph5] Running CR for 38.321 (Samsung)), multiple companies supported exploring Option 3: initiating RACH with a cause indication when the UEI report is triggered but the associated resources cannot be used due to TAT expiry. We also support this direction with the addition of an explicit cause indication to the network (MAC CE).  The rationale is that UEI reporting is time-critical, and informing the network of the cause at the earliest opportunity allows timely reconfiguration of resources and avoids stale reporting.

Specifically, when this condition occurs:

- The UE initiates contention-based RACH without waiting for PDCCH-ordered RACH.

- The UE includes an indication in Msg3 (e.g., MAC CE) that the cause is “UEI report resource unavailable due to TAT expiry”.

- The same behaviour applies for both mode-A and mode-B, including the case where TAT expires after PUCCH but before PUSCH in mode-A.

Proposal 1: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and the UE includes an indication in Msg3 (e.g., MAC CE) the cause of RACH. The same behaviour applies for both mode-A and mode-B, including the case where TAT expires after PUCCH but before PUSCH in mode-A.

## 2.2	RRC-related UEIBM issues

Regarding the CSI-ReportConfig for UEIBM, we highlight two points for discussion based on the running CR.

The relevant details of the configuration are shown below (for brevity, we use "…" for details that are irrelevant).

CSI-ReportConfig ::=                SEQUENCE {

...

resourcesForChannelMeasurement       CSI-ResourceConfigId,

...

groupBasedBeamReporting              CHOICE {

enabled                                     NULL,

disabled                                    SEQUENCE {

nrofReportedRS                          ENUMERATED {n1, n2, n3, n4} OPTIONAL   -- Need S

}

},

...,

[[

...

eventTypeUE-IBR                      CHOICE {

event1-r19                                 SEQUENCE {

...

},

event2-r19                                 SEQUENCE {

...

},

event7-r19                                 SEQUENCE {

...

}

},                                     										OPTIONAL,  -- Need R

csi-ReportUE-IBR-r19                 CSI-ReportUE-IBR-r19                   OPTIONAL,  -- Cond UE-IBR

...

]]

}

CSI-ReportUE-IBR-r19             ::=     SEQUENCE {

reportTransmissionMode-r19               ...,

nrofReportedRS-UE-IBR-r19                ENUMERATED {n1, n2, n3, n4},

tci-ServCellIndex-r19				     ...                               OPTIONAL,   -- Need R

currentBeamReport-r19                    ...                               OPTIONAL,   -- Need R

conditionFulfillmentIndicator-r19        ... 							   OPTIONAL,   -- Need R

eventCountWindow-r19					 SEQUENCE {

...

}                                                                          OPTIONAL    -- Need R

pucch-Resource-r19                       SEQUENCE {

...

}                                                                          OPTIONAL    -- Need R

}

-- TAG-CSI-REPORTCONFIG-STOP

-- ASN1STOP

Currently, the UE-IBR events are configured separately from the remaining part of the UE-IBR config, i.e. eventTypeUE-IBR and csi-ReportUE-IBR are separate optional fields, where csi-ReportUE-IBR is conditional on eventTypeUE-IBR being configured. However, in our understanding, there will always be an interdependency between these fields and one cannot be configured without the other (since they both use need code “R” it is not possible to modify a CSI-ReportConfig to update one of the fields without the other one, otherwise the field not being updated would get released if it is not included in the CSI-ReportConfig). We also understand that no further UE-IBR events will be supported in Rel-19 or -20, so there is no reason to accommodate “future events” in the configuration either. Hence, it makes sense to consolidate the configurations, e.g. by moving eventTypeUE-IBR under csi-ReportUE-IBR.

Observation 1: eventTypeUE-IBR and csi-ReportUE-IBR are interdependent fields meaning CSI-ReportConfig must include both fields or neither of them (but cannot include only one).

Proposal 2: Consolidate eventTypeUE-IBR and csi-ReportUE-IBR under a single configuration within CSI-ReportConfig.

Another aspect to point out is the parameter nrofReportedRS-UE-IBR seems redundant given that groupBasedBeamReporting configuration (which includes the option to configure nrofReportedRS when group-based beam reporting is set to disabled) is already mandatory to include within CSI-ReportConfig.

Besides, regarding the actual “reference signals” that are related to nrofReportedRS-UE-IBR parameter, TS 38.214 refers to these as the “reference signals provided by the newBeamResourceSet”; but RAN2 already agreed that we would reuse resourcesForChannelMeasurement instead of introducing the parameter newBeamResourceSet. This is another reason why it seems redundant to introduce nrofReportedRS-UE-IBR.

For these reasons it seems sufficient to require the network to set groupBasedBeamReporting to disabled and include nrofReportedRS whenever csi-ReportUE-IBR is included in a CSI report configuration, instead of adding the new parameter nrofReportedRS-UE-IBR.

Observation 2: nrofReportedRS-UE-IBR seems redundant given that groupBasedBeamReporting (which includes nrofReportedRS) is already mandatory and since RAN2 already agreed to reuse resourcesForChannelMeasurement instead of the new parameter newBeamResourceSet.

Proposal 3: Remove nrofReportedRS-UE-IBR and capture in field description for groupBasedBeamReporting that whenever csi-ReportUE-IBR is included in a CSI report configuration the network shall set groupBasedBeamReporting to disabled and include the field nrofReportedRS.

## 2.3	Other RRC-related issues

In this section we raise some other issues related to the RRC configurations for CSI-CRI and CSI-CJTC that we noticed after the offline discussion ([Post130][218][MIMO_Ph5] Running CR for 38.331 (Ericsson]), which we did not comment on earlier.

### 2.3.1	CSI-CRI parameters

Regarding the CSI-CRI parameters, we think there is a mistake in the ASN.1 definitions for cri-TypeI-SinglePanelRI-Restriction and cri-TypeI-SinglePanelN1-N2-CBSR. Currently these parameters are defined by sequences of up to four bitstrings, where each bitstring corresponds to a RI/CBSR restriction on a CSI-RS resource.

CodebookConfig-r19  ::=               SEQUENCE  {

codebookType                          CHOICE {

type1                                   CHOICE {

typeI-SinglePanel-r19                  SEQUENCE {

...

cri-TypeI-SinglePanelRI-Restriction-r19   SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (8))		       OPTIONAL,    -- Need R

cri-TypeI-SinglePanelN1-N2-CBSR-r19        CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19       OPTIONAL     -- Need R

},

...

},

...

}

CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19 ::=     CHOICE {

two-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (8))},

two-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (64))},

four-one-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (16))},

three-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (96))},

six-one-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (24))},

four-two-r19           CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (128))},

eight-one-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (32))},

four-three-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (192))},

six-two-r19            CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (192))},

twelve-one-r19         CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (48))},

four-four-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (256))},

eight-two-r19          CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (256))},

sixteen-one-r19        CHOICE {no-cbsr-r19 NULL, cbsr-list-r19 SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (64))}

}

However, in accordance with clause 5.2.1.4.2 of 38.214 (relevant text below), our understanding is that for TypeI-SinglePanel codebooks, there can be up to Ks=8 CSI-RS resources, and hence cri-TypeI-SinglePanelRI-Restriction and cri-TypeI-SinglePanelN1-N2-CBSR should be able to configure up to eight restrictions (one for each CSI-RS resource):

If the UE is configured with a CSI-ReportConfig with higher layer parameter valueOfM, with reportQuantity set to 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI' and codebookType set to 'typeI-SinglePanel', or reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-r16', and Ks>1 CSI-RS resources are configured in the corresponding NZP-CSI-RS-ResourceSet for channel measurement,

-	if 2≤Ks≤4, each CSI-RS resource shall contain at most 32 CSI-RS ports. If 5≤Ks≤8, each CSI-RS resource shall contain at most 16 CSI-RS ports.

-	The CSI report contains M≤Ks CSIs, where the M sets of CSI parameters other than CRI, RI/LI (if applicable)/PMI/CQI, are independently calculated and indicated for each of the selected M CSI-RS resources. Subject to UE capability, a UE can be configured with M∈{1,…,  min4,Ks} and Ks≤8, if codebookType is set to 'typeI-SinglePanel', and with M∈{1,2} and Ks≤4, if codebookType is set to 'typeII-r16'.

<other text omitted>

-	A CSI-ReportConfig can be configured with separate RI restrictions for each of the Ks CSI-RS resources, by higher layer parameter cri-typeI-SinglePanel-ri-restriction-r19 or cri-typeII-ri-restriction-r19, for codebookType set to 'typeI-SinglePanel' or 'typeII-r16', respectively.

-	A CSI-ReportConfig can be configured with separate Codebook Subset Restrictions for each of the Ks CSI-RS resources, by higher layer parameter cri-typeI-SinglePanel-CBSR-r19 or cri-typeII-CBSR-r19, for codebookType set to 'typeI-SinglePanel' or 'typeII-r16', respectively. For codebookType set to 'typeII-r16', cri-typeII-CBSR-r19 is configured as described in Clause 5.2.2.2.5, where only the bit values '00' or '11' of Table 5.2.2.2.5-6 are configurable.

Observation 3: For TypeI-SinglePanel codebooks, there can be up to eight CSI-RS resources each with separate RI / CBSR restrictions, but right now cri-TypeI-SinglePanelRI-Restriction and cri-TypeI-SinglePanelN1-N2-CBSR only allow up to four resources restrictions to be configured.

Proposal 4: Change all instances of ‘SIZE (1..4)’ to ‘SIZE (1..8)’ within cri-TypeI-SinglePanelRI-Restriction and cri-TypeI-SinglePanelN1-N2-CBSR.

### 2.3.2	CSI-CJT parameters

Three CSI-CJT parameters, associatedSRSResourceSet, referenceAntennaPort, and triggeringScheme were provided in the RAN1 parameter list but have not yet been captured in the RRC CR.

| RAN1 parameter name | Description | Value range |
| --- | --- | --- |
| associatedSRSResourceSet | This parameter is used to indicate the SRS resource (along with resource set) associated with the CSI-RS resource used for PO measurement. Q=1 SRS resource is supported | FFS |
| referenceAntennaPort | The UE antenna port for receiving the CSI-RS configured for phase offset measurement that is same as the UE antenna port(s) for transmitting the configured ports from the associated SRS resource | up to RAN2 regarding {1,..,x}, where x is x in ‘xTyR’ when SRS usage set to antenna switching |
| triggeringScheme | Configuring the triggering scheme either separate triggering or joint triggering | {separate, joint} |

For associatedSRSResourceSet and referenceAntennaPort, we think there are already enough details in TS 38.214 to define these parameters. Clause 5.2.1.4.2 of 38.214 states the following:

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-P',

<other text omitted>

-	The UE can be configured by the higher layer parameter associatedSRSResourceSet indicating the SRS resource and corresponding SRS resource set associated with phase offset measurement, from all the y/x SRS resources and all the configured SRS resource set(s) with the higher layer parameter usage set to 'antennaSwitching' and configuration xTyR, and by the higher layer parameter referenceAntennaPort indicating which SRS port in the indicated SRS resource is associated with phase offset measurement, such that the UE antenna port used for receiving the CSI-RS resources configured for phase offset measurement is the same as the UE antenna port used for transmitting the reference SRS port in the associated SRS resource set for antenna switching. The configured associated SRS resource can be aperiodic, semi-persistent or periodic. If the configured associated SRS resource is semi-persistent or periodic, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the transmission occasions of the NTRP CSI-RS resources used for measuring the CJTC-P report.

[OPEN ISSUE: referenceAntennaPort definition]:

According to the above, associatedSRSResourceSet should point to an SRS resource (represented by SRS-ResourceId) as well as the SRS resource set (SRS-ResourceSetId) corresponding to that SRS resource. In our understanding associatedSRSResourceSet is used jointly with referenceAntennaPort, since referenceAntennaPort indicates one SRS port among the up to x SRS ports associated with the SRS resource in associatedSRSResourceSet, where the upper range network can configure for x would be determined by nrofSRS-Ports (up to 4 ports) or nrofSRS-Ports-n8 (up to 8 ports, if signalled, since up to 8T8R SRS antenna switching can be supported as of Rel-18).

Based on this, we think the parameters can be defined together as:

AssociatedSRS-ResourceSet-r19       SEQUENCE {

srs-Resource-r19                    SRS-ResourceId,

srs-ResourceSet-r19                 SRS-ResourceSetId,

referenceAntennaPort-r19            INTEGER (1..8)

}

Additionally, the field description for associatedSRS-ResourceSet-r19 should state that the network can only include the configuration when reportQuantity is set to cjtc-P.

Observation 4: [OPEN ISSUE: referenceAntennaPort definition] The relevant details for defining associatedSRSResourceSet and referenceAntennaPort are in TS 38.214 clause 5.2.1.4.2.

Proposal 5: [OPEN ISSUE: referenceAntennaPort definition] Define parameter AssociatedSRS-ResourceSet-r19 as sequence composed of SRS-ResourceId, SRS-ResourceSetId, and referenceAntennaPort-r19 (defined by INTEGER (1..8)); and capture in field description that network can only include associatedSRS-ResourceSet-r19 when reportQuantity is set to cjtc-P.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-Dd',

<other text omitted>

-	Subject to UE capability, the CSI-ReportConfig with reportQuantity set to 'cjtc-Dd' can be linked, by the higher layer parameter linkedCJTCReport, to an aperiodic CSI-ReportConfig with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' and with the corresponding CSI-RS resources for channel measurement that are aperiodic and not QCLed with the same reference signal with respect to {average delay}. The CSI-RS resource sets associated with the CJTC-Dd report are linked to the CSI-RS resources associated with the 'typeII-CJT-r18' report by a fixed correspondence between the resource set IDs and resource IDs, respectively, in sequential order of configuration in their respective Resource Setting. The UE expects that the number of CSI-RS resource sets configured in the Resource Setting for the CJTC-Dd report is the same as the number of CSI-RS resources in the Resource Setting associated with the linked 'typeII-CJT-r18' report.

-	When the two CSI reports are jointly triggered, the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the CJTC-Dd report in the same reporting instance.

-	When the two CSI reports are separately triggered, the higher layer parameter delayOffsetCompensation indicates whether the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the latest CJTC-Dd report whose reporting instance’s last symbol is before the first symbol of the DCI triggering the 'typeII-CJT-r18' report. If multiple 'typeII-CJT-r18' reports linked to CJTC-Dd report(s) are triggered in the same aperiodic trigger state and delayOffsetCompensation is configured, the UE is expected to apply delay offset compensation for all the linked 'typeII-CJT-r18' reports.

Our understanding is that the parameter triggeringScheme is also used in clause 5.2.1.4.2 (although implicitly since the parameter name is not referred to directly):

Based on this, we think that triggeringScheme can be added to the 38.331 CR and RAN2 can ask RAN1 to ensure that the parameter is referred to in TS 38.214 the same time that RAN2 informs RAN1 of the parameters that were modified in 38.331 (discussed further below).

Observation 5: TS 38.214 clause 5.2.1.4.2 already implicitly uses the parameter triggeringScheme.

Proposal 6: Define triggeringScheme in 38.331 as stated in the RAN1 parameter list and capture in field description that network can only include the field when reportQuantity is set to cjtc-Dd.

### 2.3.3	Discrepancy between parameters in RAN1 and RAN2

Then, more generally regarding the RRC CR, we observe that a lot of the parameters shared by RAN1 have been modified by RAN2 for various reasons. For example:

-	Parameter name was changed, e.g. to follow ASN.1 naming conventions and/or naming conventions established for similar parameters in earlier releases

-	Parameter was split into multiple parameters, e.g. to facilitate signalling of multiple variables associated with the same parameter

-	Multiple parameters were grouped under a single parameter, e.g. for the case of some codebook restrictions where signalling overhead could be better optimized this way

Since the Rel-19 version of the RAN1 specs have already been released using the original parameter names provided by RAN1, we think it would be good for RAN2 to highlight all of the changes we have made to the parameters so that RAN1 can update their specs accordingly. This can be communicated in an LS. We have provided a draft for the LS in Annex A.

Observation 6: RAN2 modified some RAN1 parameters in the RRC spec, for example, by renaming parameters to follow ASN.1 conventions, but also by grouping or splitting some parameters to optimize the signalling.

Proposal 7: Send an LS to RAN1 listing the changes RAN2 made to the RAN1 parameters.

# 3	Conclusion

Observation 1: eventTypeUE-IBR and csi-ReportUE-IBR are interdependent fields meaning CSI-ReportConfig must include both fields or neither of them (but cannot include only one).

Observation 2: nrofReportedRS-UE-IBR seems redundant given that groupBasedBeamReporting (which includes nrofReportedRS) is already mandatory and since RAN2 already agreed to reuse resourcesForChannelMeasurement instead of the new parameter newBeamResourceSet.

Observation 3: For TypeI-SinglePanel codebooks, there can be up to eight CSI-RS resources each with separate RI / CBSR restrictions, but right now cri-TypeI-SinglePanelRI-Restriction and cri-TypeI-SinglePanelN1-N2-CBSR only allow up to four resources restrictions to be configured.

Observation 4: [OPEN ISSUE: referenceAntennaPort definition] The relevant details for defining associatedSRSResourceSet and referenceAntennaPort are in TS 38.214 clause 5.2.1.4.2.

Observation 5: TS 38.214 clause 5.2.1.4.2 already implicitly uses the parameter triggeringScheme.

Observation 6: RAN2 modified some RAN1 parameters in the RRC spec, for example, by renaming parameters to follow ASN.1 conventions, but also by grouping or splitting some parameters to optimize the signalling.

Proposal 1: UE initiates RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired, and the UE includes an indication in Msg3 (e.g., MAC CE) the cause of RACH. The same behaviour applies for both mode-A and mode-B, including the case where TAT expires after PUCCH but before PUSCH in mode-A.

Proposal 2: Consolidate eventTypeUE-IBR and csi-ReportUE-IBR under a single configuration within CSI-ReportConfig.

Proposal 3: Remove nrofReportedRS-UE-IBR and capture in field description for groupBasedBeamReporting that whenever csi-ReportUE-IBR is included in a CSI report configuration the network shall set groupBasedBeamReporting to disabled and include the field nrofReportedRS.

Proposal 4: Change all instances of ‘SIZE (1..4)’ to ‘SIZE (1..8)’ within cri-TypeI-SinglePanelRI-Restriction and cri-TypeI-SinglePanelN1-N2-CBSR.

Proposal 5: [OPEN ISSUE: referenceAntennaPort definition] Define parameter AssociatedSRS-ResourceSet-r19 as sequence composed of SRS-ResourceId, SRS-ResourceSetId, and referenceAntennaPort-r19 (defined by INTEGER (1..8)); and capture in field description that network can only include associatedSRS-ResourceSet-r19 when reportQuantity is set to cjtc-P.

Proposal 6: Define triggeringScheme in 38.331 as stated in the RAN1 parameter list and capture in field description that network can only include the field when reportQuantity is set to cjtc-Dd.

Proposal 7: Send an LS to RAN1 listing the changes RAN2 made to the RAN1 parameters.

# Annex A:	TP for LS to RAN1 on RRC parameters

3GPP TSG-RAN WG2 Meeting #131	R2-250xxxx

Bengaluru, India, 25 – 29 August 2025

Title:	LS on RRC parameters for Rel-19 MIMO

Response to:	-

Release:	Release 19

Work Item:	NR_MIMO_Ph5-Core

Source:	TSG RAN WG2

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	First Last

E-mail Address:	first.last@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN1 for sharing the parameter list for Rel-19 NR_MIMO_Ph5. While incorporating the parameters into the RRC specification (TS 38.331), RAN2 made some changes to certain parameters. Examples of such types of changes are:

- Parameter name was changed, e.g. to follow ASN.1 naming conventions and/or naming conventions established for similar parameters in earlier releases

- Parameter was split into multiple parameters, e.g. to facilitate signalling of multiple variables associated with the same parameter

- Multiple parameters were grouped under a single parameter, e.g. for the case of some codebook restrictions where signalling overhead could be better optimized this way

RAN2 understands, however, that the Rel-19 version of the RAN1 specifications were drafted based on the original parameter list shared by RAN1, and hence, there will be a discrepancy between the RAN1 specs and the RRC spec.

The attached file lists the relevant parameter modifications made within TS 38.331 for Rel-19 MIMO. RAN2 requests that RAN1 take this information into account and update their specifications accordingly.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to update their specifications based on the attached list of parameter changes endorsed by RAN2 for TS 38.331.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#131bis	from 2025-10-13	to 2025-10-17		Prague, CZ

RAN2#132	from 2025-11-17	to 2025-11-21		Dallas, USA