3GPP TSG-RAN WG2 Meeting #131	R2-2505927

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.13.3

Source:	Nokia

Title:	On SFN-DFN offset and time sensitive applications

WID/SID:	NR_SL_relay_multihop - Release 19

Document for:	Discussion and Decision

# 1	Introduction

One open issue according to the open issue list is whether the current SFN-DFN offset mechanism defined for single-hop relay scenarios should be extended such that the connected parent Relay UE and that the SFN-DFN offset provided at the L2 U2N Relay UE or the L2 Last U2N Relay UE can be forwarded by intermediate U2N Relay UEs within the multihop relay chain.

# 2	Discussion

In the current running CR, the SFN-DFN offset mechanism is extended to be applicable to the child UE, which in our sense is a reasonable extension which should be kept.

Proposal 1: RAN2 to follow rapporteurs’ proposal and confirm SFN-DFN forwarding extension as in the current CR.

Additionally, it was proposed to discuss whether the forwarding mechanism is applicable also to positioning SIBs, which needs an exact timing. Positioning is a key element of public and emergency services, and allowing such procedure to be effectively deployed within multi-hop relay is reasonable.

Observation 1: Positioning can be a key usecase for also multi-hop relay.

If RAN2 agrees to discuss procedures for supporting multi-hop relay, in case the current procedures is not adequate to supporting positioning requirements, we believe that other SIBs should also be taken into consideration.

Accurate time synchronization has been introduced in Release-16 with the support of Time Sensitive Networking (TSN) for industrial communication purposes. With the help of accurate time synchronization, the UE will be able to acquire accurate time synchronization which can be used to timestamp or coordinate events (packets, actions, transactions, ect) and will typically be relative to a global time domain (such as Universal Coordinated Time (UTC)).

The accurate time synchronisation is enabled by a framework by identifying specific events which are then timestamped by the gNB, allowing the gNB to identify these events as the ending boundary of a reference system frame number (SFN). The timestamp is carried in a ReferenceTimeInfo-r16 IE (RTI hereafter) that can be transmitted in a SIB9 (ideal for broadcasting) or via dedicated Radio Resource Control (RRC) signalling as a part of DLInformationTransfer. In the case of RRC carried RTI, the reference SFN that the timestamp refers to is explicitly given (can be back or forth in time), but in the case of SIB9 carried SIB9, the reference SFN is implicitly given by the SFN in which the ongoing system information window (SI-window) ends [TS 38.331].

Observation 2: Since Rel-16, SIB9 can be used to achieve accurate time information by the UE as it may contain the optional ReferenceTimeInfo IE extension (RTI).

As the timing information coming from the RTI in SIB9 are relevant for time sensitive applications i.e. sidelink applicatins, we propose to discuss whether/how to ensure the remote UE is able to interpret the information from SIB9.

Proposal 2: If dedicated procedures are added to handle positioning using the connected parent relay UE RAN2 to discuss how to include handling of the forwarding of SIB9 i.e. by looking at how to interpret the SFN boundary provided in the ReferenceTimeInfo.

# 3	Conclusion

In this contribution the following observations and proposals has been made;

Proposal 1: RAN2 to follow rapporteurs proposal and confirm SFN-DFN forwarding extension as in the current CR.

Observation 1: Positioning can be a key usecase for also multi-hop relay.

Observation 2: Since Rel-16, SIB9 can be used to achieve accurate time information by the UE as it may contain the optional ReferenceTimeInfo IE extension (RTI).

Proposal 2: If dedicated procedures are added to handle positioning using the connected parent relay UE RAN2 to discuss how to include handling of the forwarding of SIB9 i.e. by looking at how to interpret the SFN boundary provided in the ReferenceTimeInfo.