3GPP TSG-RAN WG2 Meeting #131											R2-2505929

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.11.2

Source: 	Sharp

Title:  	Discussion on SBFD RA open issues

Document for: 	Discussion

# Introduction

In RAN2#130 meeting, RAN2 made the following agreements:

Agreements on MAC open issues - For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion. - Msg1 repetition number fallback can be supported within SBFD RO. - Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case. - For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations. - The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available. Other agreements - SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA. - For PRACH configuration option 1, when a feature combination is configured using preamble partitioning of the RACH-ConfigCommon, using the shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs. - For PRACH configuration option 1, same featureCombinationPreamblesList configuration is used for both Additional-ROs and Legacy-ROs.

This document discusses open issues on SBFD random access.

# Discussion

## (MAC-3) SBFD RO location derivation in CFRA

[Open Issue MAC-3-1] Do companies think any additional specification changes (in MAC and/or RRC) are essential to address the issue on how to derive the SBFD RO locations for CFRA (ZTE’s concern)? Please provide your input with the rationale. [Open Issue MAC-3-2] If so, please provide your preferred solution with the rationale, including but not limited to: - Network can indicate RO type as SBFD RO for CFRA, only if an SBFD RACH configuration is configured for CBRA (proposed by LGE).

It is not possible that CBRA uses RACH configuration option 1 and CFRA uses RACH configuration option 2. The only problematic scenario is that CBRA does not have an SBFD RACH configuration and CFRA has an SBFD RACH configuration. Since CFRA to CBRA fallback is being supported as a legacy procedure, gNB most likely uses random access in SBFD symbol for both initial CBRA and fallback CBRA. In our view, CBRA should be a baseline of SBFD random access, and CFRA can be an add-on feature. Hence, it is acceptable that SBFD CBRA can be a pre-requisite of SBFD CFRA. In this case, UE can assume that CBRA and CFRA have the same RACH configuration option.

(MAC-3) SBFD CFRA can be configured only if SBFD RACH configuration is configured for CBRA.

(MAC-3) RACH configuration for CFRA assumes the same RACH configuration of CBRA.

## (MAC-4) Parameter Initialization after RO type switching

[Open Issue MAC-4-1] Do companies think the parameter initialization is needed after RO type switching in the case of RACH configuration Option 2? Please provide your input with the rationale. [Open Issue MAC-4-2] If so, please further clarify whether all parameters in RACH-ConfigCommon should be considered for the initialization, or if certain parameters should be excluded from the initialization, along with the rationale.

In RO type switching, RACH configuration can be different. In this case, the UE should use the configuration for the switched RO type. Thus, the parameter initialization is needed as specified in subclause 5.1.1a of TS 38.321. We may consider some exceptions if there is a critical problem. For now, we do not see any big issue to allow the exceptional cases.

(MAC-4) After RO type switching, RACH parameter initialization for the switched RO type is performed as specified in subclause 5.1.1a of TS 38.321.

## (RRC-1) NT indication of RO Type

[Proposal for RRC-1] For the NT indicating RO type, use 1 bit signalling (as in the current RRC running CR).

In Rel-19 SBFD WI, RAN2 introduced multiple options to select RO type for a UE. RSRP-based RO type selection can be used to allow either only cell-center UEs or only cell-edge UEs to use SBFD RA, and the statistical ratio of SBFD RA usage is configurable based on the RSRP threshold. Also, the current RRC CR implemented a one-bit explicit indication of RO type. In other words, the current RO type selection rule already allows some flexibility. In our view, further flexibility is not necessary from both WI completion and necessity perspectives. Hence, we think the current RRC signaling design is good enough.

(RRC-1) 1-bit indication of RO Type is used.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- (MAC-3) SBFD CFRA can be configured only if SBFD RACH configuration is configured for CBRA.

- (MAC-3) RACH configuration for CFRA assumes the same RACH configuration of CBRA.

- (MAC-4) After RO type switching, RACH parameter initialization for the switched RO type is performed as specified in subclause 5.1.1a of TS 38.321.

- (RRC-1) 1-bit indication of RO Type is used.