3GPP TSG-RAN WG2 Meeting #131											R2-2505930

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.11.3

Source: 	Sharp

Title:  	Discussion on SBFD other open issue

Document for: 	Discussion

# Introduction

This document discusses an open issue on co-existence of SBFD and dual-connectivity.

# Discussion

## (RRC-7) Co-existence of SBFD and NR-DC

[Proposal for RRC-7] RAN2 understands the simultaneous configuration of SBFD and NR-DC can be supported from RAN2 perspective. Send reply LS to RAN4 (CC RAN1) with this understanding.

In RAN1#121 meeting, RAN1 made an agreement that SBFD and NR-DC can be configured simultaneously from RAN1 perspective, as follows:

Agreement - From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier. - Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly. - It is up to RAN2 to decide whether or not to support simultaneous configuration of SBFD and DC. - CC: RAN3, RAN4. Reply LS on simultaneous configuration of SBFD and DC. Final LS in R1-2504858.

This agreement has some conditions. First, it can be possible only for inter-band NR-DC. Also, the agreement considered only RAN1 perspective, and they left the final decision up to RAN2 whether to support the simultaneous configuration issue. Then, RAN2 discussion should focus on whether it can be supported from the RAN2 perspective. In other words, RAN2 should check whether there is any critical issue of the simultaneous configurations.

From RAN2 perspective, we see that the only concern may be complexity of the joint configuration from the RRC signalling perspective or procedure perspective. However, those should be independently signalled by an RRC reconfiguration message. There is no cross-functional issue. Most of expected complexities are related to physical layer and hardware support, which are RAN1 or RAN4 issue. As RAN1 already concluded that it is possible, RAN2 can agree the same from RAN2 perspective and wait for RAN4 response.

(RRC-7) From RAN2 perspective, NR-DC and SBFD can be configured simultaneously for a UE. Send a reply LS to RAN4 (CC RAN1) with this understanding.

The RAN1 agreement considers only NR-DC. It means that other MR-DC options are precluded. Thus, RAN2 does not need to consider any other MR-DC options.

(RRC-7) SBFD cannot be configured together with other MR-DC options except NR-DC.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- (RRC-7) From RAN2 perspective, NR-DC and SBFD can be configured simultaneously for a UE. Send a reply LS to RAN4 (CC RAN1) with this understanding.

(RRC-7) SBFD cannot be configured together with other MR-DC options except NR-DC.

# References

[1] RAN1#121 chairman’s note