3GPP TSG RAN WG2 #131			R2-2505931

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.1.5

Source:	BT Plc, T-Mobile, Deutsche Telekom, Turkcell, Orange, TIM, AT&T, Verizon, Docomo, Nokia, KPN

Title:	Operators views for the AI_ML model delivery options

Document for:	Discussion/Decision

# Introduction

In the Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 [1], the following is captured where RAN2 is the leading working group

- Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

Due to time constraints over the AI/ML for the air interface WID, RAN2 has had no time to properly study the model transfer. In parallel, RAN1 has been working on model transfer where they have focused their efforts on two-sided models. For one-sided models there is no consensus among companies.

In RAN2, the first discussion related to model transfer was done during RAN2#130 where the following TDocs where discussed.

R2-2504379	Discussion on AIML model transfer delivery	CMCC,China Telecom,CATT,ZTE,Apple,Samsung,Xiaomi	discussion	Rel-19	NR_AIML_air-Core

R2-2504641	Operators views for the AI_ML model delivery options	BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange	discussion	Rel-19	R2-2504635

Both documents where noted

R2-2504379 Discussion on AIML model transfer delivery CMCC,China Telecom,CATT,ZTE,Apple,Samsung,Xiaomi discussion Rel-19 NR_AIML_air-Core Proposal 1: There is no requirements on controllability or visibility for UE-sided model transfer/delivery case y, since the model trained in OTT server can be sent to the UE directly - Tmobile disagrees and thinks its important to have visibility. How would you manage something for which you have no knowledge of. CMCC is not sure how to have visibility as this is going over OTT. Qualcomm thinks that the whole monitoring procedure is sufficient to check the performance of UE. - Mediatek thinks that we shouldn’t decide whether we will have same of different tunnel. Proposal 2: It is proposed to de-prioritize case z1 for one-sided model transfer/delivery, since there is no strong requirement on visibility and controllability for UE-sided model transfer/delivery from the perspective of operator. Proposal 3: It is proposed to follow case y for UE-sided model transfer/delivery in R19, i.e. the UE-sided model is delivered over the top, which is transparent to 3GPP. Noted R2-2504635 Operators views for the AI_ML model delivery options BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange discussion Rel-19 => Revised in R2-504641 R2-2504641 Operators views for the AI_ML model delivery options BT Plc, Turkcell, T-Mobile, Deutsche Telekom, Orange discussion Rel-19 R2-2504635 Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed in RAN2 Proposal 2: One-sided dataset/model transfer solution shall follow below principles: • A1 - Size: from RAN2 point of view, aim to support various sizes of model parameter transfer (FFS on model size); • A2 - Continuity: service continuity of model transfer/delivery during UE mobility needs to be supported; • A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface; • A4 - Latency: relaxed latency requirement and infrequent update; • A5 - Visibility: dataset and model parameters to be visible and understandable by MNO; - Huawei asks which case you are referring to. BT explains is the model transfer to the UE. Qualcomm asks how do you control the equivalent of 1a over the top. BT explains that this beyond 1a. - Qualcomm asks how does it work, if the network configures you with inference config and you don’t have model you have to download. - Samsung, Apple, Xiaomi, Oppo, thinks that the discussion should focus on 2sided model and doesn’t see how the network knows whether the model is trusted. Apple explains that RAN1 already concluded to focus on 2sided. - Nokia, BT and Ericsson think controllability is important as the UE shouldn’t download a model if it is going to idle. Also it is strange how RAN1 can decide on data transfer that is not in their expertise in scope. Proposal 3: Adopt additional list of requirements for one-sided and two-sided model transfer/delivery. • Activation: once the AI/ML one-sided or two-sided model is transferred to NW and/or to UE, MNO should be capable to activate and deactivate them at any point in time • Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic. • Differentiability: model transfer/delivery traffic should be differentiated from other user traffic. • Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded. • Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one • Controllability: the NW is in control of if and when to transfer/deliver a model Noted

In this contribution, signing companies would like to progress on the model transfer study.

# Discussion

## 3GPP solution for model transfer

The solutions summarized on Table 1 in TR 38.843 [1] can be categorized into three different categories.

- RAN

- Core and LMF

- OAM

Table 1 Need to consider standardisation solutions for transferring/delivering AI/ML models

Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE. Note: The relationships between model transfer/delivery solutions and use cases can be derived from what is captured in clauses 7.3.2, 7.3.3, and 7.3.4.

Since the OTT solution has no impact on 3GPP specifications, there is no need to further discuss this solution, and we should focus the remaining time on other solutions.

Proposal 1: Solution 4a, OTT without 3GPP impact, is no longer discussed in RAN2 as a standardised solution for transferring/delivering AI/ML models

## Dataset/model parameter transfer solution for one-sided model

Before RAN2#129bis, RAN2 had an email discussion where the phase 1 had the following scope for the evaluation area and the requirements

During Rel-18 SI, we had some practices on how to evaluate different model transfer/delivery solutions among following four discussion areas: - A1: Large, no upper limit model/model parameter size, - A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs), - A3: Network controllability on model transfer/delivery (e.g., management decision at gNB), - A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

Finally, during RAN2#129bis, the following agreements were made

Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles: - A1 - Size: From RAN2 point of view, aim to define a unified solution (e.g. OTA or non-OTA) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs); - A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported; - A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server); - A4 - Latency: Relaxed latency requirement and infrequent update; - A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server). - A6: Proprietary information of the network and UE should be respected and not disclosed.

Those agreements were reached based on an email discussion in RAN2 [2] previous to RAN2#129bis meeting where questions were triggered explicitly for two-sided models.

For operators, it is required to have a standardised solution for one-sided models as well. Managing the mobile network is part of the operators’ core business. Leaving the one-sided model unspecified (OTA) may have a negative impact on network KPIs due to several reasons. It may happen that the AI/ML model is being transferred during peak hours, adding unnecessary extra congestion into the network. UEs on cell edge or with low bandwidth configured should skip the one-sided model transfer as it will have a severe impact on the power consumption. Unfortunately, that knowledge is restricted to the MNO so an OTA solution is not aware of this situation. It may happen that there is an emergency situation in the area. It is well-known that when that happens, the network will be congested. If first responders use the 3GPP network, the situation is even worse in terms of congestion leaving very limited resources to other customers. The knowledge of the emergency situation is restricted to the MNO. Therefore, trying to transfer the model to UEs inside the area will magnify the congestion issue.

Observation 1: Leaving transfer/delivery of one-sided models unspecified by 3GPP will have potentially harmful risks, including negative impacts on MNOs network KPIs

These are only a few examples to highlight the importance for operator control of the decision when the model should be transferred. Currently, the model size and the periodicity where the models will be transferred is unknown. From the RAN1 study for two-sided models, the model size may goes from few kB to hundreds of MB. Due to the high margin of uncertainty, MNOs need a safe approach and consider models will have hundreds of MB. Periodicity of the model being transferred to a UE is not provided which means, the model can be transferred daily, weekly, monthly, etc.. The criteria used to determine when a model needs to be transferred remains outside the scope of 3GPP.

In summary, the MNO cannot control the model size, the periodicity, and the criteria to transfer the model. The owner of the model cannot control the network KPIs, the congestion, the radio configuration on the UE, the radio conditions on the UE when the model is instructed to be transferred. For all of these reasons, it is required that the MNO remains in control of the one-sided model transfer.

Proposal 2: One-sided model transfer should be specified by 3GPP

For operators’ feature integration, multiple 3GPP solutions for a single problem is an undesired outcome. It fragments the market which complicates the development and the implementation. 3GPP has several examples where fragmentation has delayed solution adoption. More importantly, there is no technical reason to differentiate the principles agreed upon in RAN2#129bis for the two-sided model and not apply them to the one-sided model.

For that reason, for one-sided UE model, the aim is to agree the same principles agreed in RAN2#129bis for two-sided models.

Proposal 3: One-sided dataset/model transfer solution shall follow below principles:

- A1 - Size: from RAN2 point of view, aim to support various sizes of dataset/model parameter transfer (FFS on model size);

- A2 - Continuity: service continuity of model transfer/delivery during UE mobility needs to be supported;

- A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface;

- A4 - Latency: relaxed latency requirement and infrequent update;

- A5 - Visibility: dataset to be visible and understandable by MNO;

## Principles to transfer/delivery a model

In previous sections, we discussed how to transfer the dataset and model parameters for a one-sided model. This section outlines the basic principles for transferring or delivering the model once it has been trained by training entities. The location of these entities and who controls them are outside the scope of this TDoc.

AI/ML functionality has the potential to optimize procedures by improving key aspects such as energy consumption and signaling overhead. To achieve this, well-established and tested procedures may be replaced. For example, traditional measurements could be avoided, along with the associated work already covered by RAN4 and RAN5 to ensure expected behavior. Instead, decisions will be made based on predictions. As with any prediction, there are inherent error margins. Well-trained models can reduce these margins to a level where the expected performance is maintained. However, it is important to remember that this represents the best-case scenario.

Model training requires both data and time to improve prediction accuracy. Consequently, initial AI/ML models or models applied to new UEs may lack maturity, potentially causing issues in the network. From a 3GPP perspective, until RAN4 and RAN5 address AI/ML for the air interface, there is no guarantee of performance.

Observation 2: There are no standardised tests to validate AI/ML models

It is expected that infrastructure vendors, device vendors, and chipset vendors will create their own AI/ML models. Hence, without individual SLAs, MNOs have no control or visibility over the AI/ML model creation.

Observation 3: AI/ML models are expected to be created by infrastructure vendors, device vendors, and chipset vendors

Combining the lack of standardised tests and the lack of MNOs control over the models, it is evident that MNOs require mechanisms to activate and deactivate them. MNOs are the only ones looking for the end-to-end performance. An AI/ML model may work perfect under certain circumstances but it is hard to train the models for all of the possible situations. That makes the overall performance far short of expectations. For those situations, it is required that AI/ML models can be activated and deactivated.

Observation 4: AI/ML models may require deactivation if the end-to-end performance is impacted

In [4], a list of requirements were captured in proposal 1. Considering the challenges, it is important to adopt the following requirements to ensure optimal end-to-end performance in the network.

Proposal 4: Adopt additional requirements for both one-sided and two-sided model transfer/delivery.

- Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

- Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

- Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

- Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

- Controllability: the NW is in control of if and when to transfer/deliver a model

# Conclusions

In this TDoc, we have made the following observations:

Observation 1: Leaving transfer/delivery of one-sided models unspecified by 3GPP will have potentially harmful risks, including negative impacts on MNOs network KPIs

Observation 2: There are no standardised tests to validate AI/ML models

Observation 3: AI/ML models are expected to be created by infrastructure vendors, device vendors, and chipset vendors

Observation 4: AI/ML models may require deactivation if the end-to-end performance is impacted

For those reasons, we propose the following:

Proposal 1: Solution 4a, OTT without 3GPP impact, is no longer discussed in RAN2 as a standardised solution for transferring/delivering AI/ML models

Proposal 2: One-sided model transfer should be specified by 3GPP

Proposal 3: One-sided dataset/model transfer solution shall follow below principles:

	A1 - Size: from RAN2 point of view, aim to support various sizes of dataset/model parameter transfer (FFS on model size);

	A2 - Continuity: service continuity of model transfer/delivery during UE mobility needs to be supported;

	A3 - Controllability: NW decides on if and when to transfer/delivery over the air interface;

	A4 - Latency: relaxed latency requirement and infrequent update;

	A5 - Visibility: dataset to be visible and understandable by MNO;

Proposal 4: Adopt additional requirements for both one-sided and two-sided model transfer/delivery.

	Model transfer/delivery: traffic should be transferred at a different priority, e.g., lower than user traffic.

	Differentiability: model transfer/delivery traffic should be differentiated from other user traffic.

	Security: there should be a guarantee that models are transferred securely, in a NW-aware manner, such that untrusted models cannot be downloaded.

	Addressability: Models need to be addressable such that the UE can request the transfer/delivery of a specific one

	Controllability: the NW is in control of if and when to transfer/deliver a model

# References

- RP-243245 - New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- R2-2501939 - Report of [POST129][029][AI Phy] Model transfer (Xiaomi/Ericsson)

- TR 38.843 - Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R2-2500949 - Requirements for Model Transfer/Delivery - T-Mobile USA, Boost Mobile Network, Deutsche Telekom, Orange, Charter Communications Inc., Nokia Corporation