3GPP TSG-RAN WG2 Meeting #131	R2-2505945

Bangalore, India  Aug 25th – 29th , 2025

# Agenda item:	8.6.4

# Source:	CMCC

# Title:	Discussion on open issue of conditional LTM

# Document for:	Discussion, Decision

- Introduction

Following is the open issued identified in the email discussion of MAC and RRC Running CR for mobility [1][2]:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-26 | How to consider the TAG ID when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1 for an (C)LTM candidate cell which is configured with two TAGs in RACH-less (C)LTM_ Editor’s NOTE: In RACH-less (C)LTM, for an (C)LTM candidate cell which is configured with two TAGs, TAG ID is not considered when determining whether to set the REAMBLE_POWER_RAMPING_COUNTER to 1. Rapporteur realized the same issue exists for Rel-18 LTM, which was missing. | Issue Type: Essential How to address it: based on companies’ contribution |

| Company (Name) | Open issue | Justification | Rapporteur comment |
| --- | --- | --- | --- |
| Fujitsu, NTT DOCOMO | NW should be able to distinguish between CLTM and fast recovery after CLTM failure in order to deal with SN gap caused by SRB COUNT continuation when UE is performing fast recovery. If reordering timer (t-Reordering) on NW-side is set for a long duration, because NW must wait for the duration before receiving RRCReconfigurationComplete in RRC layer and sending contention resolution to the UE, T304 in the UE may expire, which results in fast recovery failure. In Rel-18, RAN2 agreed to deal with SN gap by NW implementation, and that can be done because LTM and fast recovery can be distinguished. However, in Rel-19, CLTM and fast recovery after CLTM failure cannot be distinguished, which is an open issue. | This issue is critical for operators which configures long reordering timer. If this is not resolved, this leads to failure of fast recovery after CLTM and interruption time longer than the duration of reordering timer. | The understanding is that this could be an issue only when the value of the reordering timer is long, but now I am wondering if we really need to fix this. Anyhow, I would encorauge the companies to bring a contribution to the next meeting and also clarify which are the solutions on the table. |

In this paper, we further discuss the open issues of conditional LTM.

- Discussion

  - MAC-14: Receiving (e)LTM CSC MAC CE when CLTM TAT timer running

Based on the agreements, CLTM and LTM can be configured to the UE simultaneously, and the UE can identify whether a LTM candidate cell is a CLTM candidate cell via the associated execution condition(s) of the candidate cell. And it’s network implementation to ensure that CLMT candidate cells belong to the same CU. Besides, RAN2 agreed that the Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used [3], in other words, the enhanced LTM cell switch command is only used to trigger an inter-CU LTM.  Therefore, the open issue should be discussed based on the cell switch command MAC CE type.

Case 1: For the case when CLTM candidate TAT timer is running, an Enhanced LTM Cell switch Command MAC CE is received

In this case, an inter-CU LTM is triggered by the  network, and the UE should follow the Enhanced LTM cell switch command MAC CE to switch to the candidate cell indicated in the LTM CSC MAC CE. And if a new TA is indicated in LTM CSC MAC CE, the UE should use the updated TA. As to the CLTM candidate cells’ configuration, especially TAT timer(s), should be released, since the related candidate cell belongs to a different CU.

Case 2: For the case when CLTM candidate TAT timer is running, an LTM Cell switch Command MAC CE is received

When an intra-CU LTM is triggered by the  network, the UE should follow the LTM cell switch command MAC CE to switch to the candidate cell indicated in the CSC MAC CE. And if new TA is indicated in LTM CSC,  the UE uses the updated TA. Different from Case 1, in this case, the UE can follow the same mechanism to CLTM to handle the related timers and TA(s), i.e., keeping the TAT(s) of candidate cells except for the target cell running and maintain the TA value(s).

Proposal 1: For the case when CLTM candidate TAT timer is running, an Enhanced LTM Cell switch Command MAC CE is received, the UE switches to the candidate cell (target cell) indicated in eLTM CSC MAC CE, uses the updated TA, if included, and releases CLTM configurations of the serving cell, especially CLTM candidate TAT timer(s).

Proposal 2: For the case when CLTM candidate TAT timer is running, an LTM Cell switch Command MAC CE is received, the UE switches to the candidate cell (target cell) indicated in LTM CSC MAC CE and uses the updated TA, if included. What’s more, the UE keeps the TAT timer(s) of CLTM candidate cell(s), and maintain the TA value(s) as in subsequent CLTM.

Furthermore, for case that TAT is running after intra-CU cell switch (including CLTM and LTM), the TA is still valid from UE perspective, there’s no need to acquire TA again for the same candidate. However, TAT is configured by UE’s previous serving cell, and the validity of one candidate’s TA is only known by the UE and the cell configured the TAT based on current agreements, while target cell has no information about this. To avoid repeated TA acquisition after one CLTM, the target cell should be indicated the information about the running TAT (and TA) of candidate cells. To achieve this, two potential solutions can be considered, one is that UE can report the valid TA related information to the target cel after access to the target cell; the other way is to exchange the TA related information via Xn interface.

Proposal 3: The valid TA related information can be indicated to the target cell via the following ways:

­	UE reports the valid TA related information after access to the target cell;

­	The source cell indicate TA related information via Xn interface.

Except for the co-existence of CLTM and LTM, the co-existence of CLTM with other mobility mechanism(s) should also be discussed.

Following are the agreements on co-existence of intra-CU LTM and other mobility mechanism:

RAN2#121bis meeting agreements: While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB. RAN2#123bis meeting agreements: It is assumed that L3 handover may happen while LTM is configured / evaluated / used. P4: RAN2 confirms that during network triggered L3 HO / PSCell change, the UE does not autonomously release the LTM configuration. P5: RAN2 confirms that the RRCReconfiguration message to execute an L3 HO or PSCell change procedure may reconfigure (setup, release) the LTM configuration. RAN2#125 meeting agreements: No restriction, LTM and conditional reconfiguration can be configured for the same candidate cell (as current TS). Assume that there will be differentiation in the RRC reconfig confirm, so that the target network can know whether CHO or LTM is used. Align with CHO text regarding CHO L3 HO, on priority of LTM cell switch cmd vs execution of CHO. The coexistence of LTM and DAPS HO is not supported in Rel-18.

Based on the agreements above, network triggered L3 HO, CHO and network triggered PScell change (SN change/SN modification) can be configured with intra-CU LTM, while the coexistence of LTM and DAPS HO is not supported in Rel-18.  therefore, we think CLTM can be configured with triggered L3 HO, CHO and network triggered PSCell change, but cannot be configured with DAPS HO.

Observation 1: Network triggered L3 HO, CHO and network triggered PScell change (SN change/SN modification) can be configured with intra-CU LTM, while the coexistence of LTM and DAPS HO is not supported in Rel-18.

Proposal 4: CLTM can coexist with network triggered L3 HO, CHO and network triggered PScell change, while the coexistence of CLTM and DAPS HO is not supported

Based on [4], if the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. In other words, if there’s HO and LTM collision, DU may refuse CU’s HO command, in other words, LTM has higher priority than L3 HO.

Observation 2: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Considering the introduction of conditional LTM, two collision cases should be discussed:

- If L3 HO command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied. In this case, UE follows L3 HO command and release all TAT timers of candidate cells.

- If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time. In this case, we think LTM may have higher priority like priority of legacy LTM if higher than L3 HO, UE performs LTM.

Proposal 5: If L3 HO command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command and release all TAT timers of candidate cells.

Proposal 6: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

  - MAC-26: PREAMBLE_POWER_RAMPING_COUNTER initial value setting

This open issue is related to UE’s PREAMBLE_POWER_RAMPING_COUNTER initial value setting during the procedure for early TA acquisition to an LTM candidate cell configured with 2 TAGs.

Based on the current description in TS 38.321, the PREAMBLE_POWER_RAMPING_COUNTER is initially set to 1, and is incremented by 1 if the PDCCH order indicates the same LTM candidate cell and the same SSB as the last Random Access Preamble transmission. In our understanding, the related description covers the scenarios that UE’s preamble transmission to different SSB of the same TRP and different  SSB of different TRP/TAGs of the same candidate cell before PREAMBLE_POWER_RAMPING_COUNTER reaches the maximum value. It seems if no TAG ID information is considered, there’s  no impact to UE’s following behaviour. Besides, based on the current LTM procedure, the UE can acquire the TA related TAG information from LTM CSC MAC CE. Therefore, we think, there’s no need to consider TAG ID in PREAMBLE_POWER_RAMPING_COUNTER initial value setting in case the candidate cell is configured with two TAGs.

Proposal 7：For RACH-less intra-/inter-CU LTM and Conditional intra-CU LTM with two TAGs, TAG ID is not considered when determining whether to set the REAM-BLE_POWER_RAMPING_COUNTER to 1.

  - CLTM fast recovery

This open issue raised by NTT DOCOMO is about the case that UE’s CLTM to cell A is failed and then UE performs fast recovery to Cell B, and since both two LTM attempts are UE autonomous, the network cannot distinguish whether UE’s attempt to Cell B is a normal CLTM or a fast recovery, and it cannot handle the SN gap caused by fast recovery. Similar issues were discussed in both CHO and LTM:

For CHO, key stream reuse is identified, and some solutions are proposed in the email discussion, while finally only the following is added to TS 38.331[5], that it’s up to UE implementation:

#### 5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: 1> ensure having valid and up to date essential system information as specified in clause 5.2.2.2; 1> stop timer T311; 1> if T390 is running: 2> stop timer T390 for all access categories; 2> perform the actions as specified in 5.3.14.4; 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, and 1> if attemptCondReconfig is configured; and 1> if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in VarConditionalReconfig: 2> apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3; NOTE 1: It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

For LTM, similar key stream reuse issue was discussed in RAN2, and to solve this issue, continue count for SRB is supported:

RAN2#125 meeting agreements [6]: Continue Count for SRB at LTM recovery (if issues are found for non-LTM-failure cases can revisit), Stage-3 impact offline in CR discussion.

As to the SN gap issue caused by continue count for SRB, RAN2 made the following agreements that it’s up to network implementation, and the note is added in TS 38.300:

RAN2#126 meeting agreements [7]: R2 assumes that network implementation can handle this. Will have a note, where and what discussed in CR discussions after meeting.

Based on the previous discussion, we think, the SN gap and t-reordering related issue caused by CLTM fast recovery can also be network implementation, while no enhanced solution is needed.

Proposal 8: How to handle the SN gap and t-reordering related issue caused by CLTM fast recovery can be up to network implementation.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: Network triggered L3 HO, CHO and network triggered PScell change (SN change/SN modification) can be configured with intra-CU LTM, while the coexistence of LTM and DAPS HO is not supported in Rel-18.

Observation 2: If L3 HO and LTM almost triggered simultaneously, DU may refuse CU’s UE CONTEXT MODIFICATION REQUEST message for mobility, which means the priority of LTM is higher than L3 HO.

Proposals:

Proposal 1: For the case when CLTM candidate TAT timer is running, an Enhanced LTM Cell switch Command MAC CE is received, the UE switches to the candidate cell (target cell) indicated in eLTM CSC MAC CE, uses the updated TA, if included, and releases the TAT timers of CLTM candidate cells.

Proposal 2: For the case when CLTM candidate TAT timer is running, an LTM Cell switch Command MAC CE is received, the UE switches to the candidate cell (target cell) indicated in LTM CSC MAC CE, uses the updated TA, if included, what’s more, the UE keeps the TAT timer(s) of CLTM candidate cell(s), and maintain the TA value(s) as in subsequent CLTM.

Proposal 3: The valid TA related information can be indicated to the target cell via the following ways:

­	UE reports the valid TA related information after access to the target cell;

­	The source cell indicate TA related information via Xn interface.

Proposal 4: CLTM can coexist with network triggered L3 HO, CHO and network triggered PScell change, while the coexistence of CLTM and DAPS HO is not supported

Proposal 5: If L3 HO command is received when UE is evaluating the execution conditions of conditional LTM or at the same time that an execution condition is satisfied, UE follows L3 HO command and release all TAT timers of candidate cells.

Proposal 6: If the execution condition of CHO and the execution condition of conditional LTM are satisfied at the same time, LTM may have higher priority than CHO, and UE performs LTM.

Proposal 7：For RACH-less intra-/inter-CU LTM and Conditional intra-CU LTM with two TAGs, TAG ID is not considered when determining whether to set the REAM-BLE_POWER_RAMPING_COUNTER to 1.

Proposal 8: How to handle the SN gap and t-reordering related issue caused by CLTM fast recovery can be up to network implementation.

- References

- Discussion summary and list of MAC open issue for Mob Ph4, vivo;

- List of open issues for mobility ph4, Ericsson;

- Report of 3GPP TSG RAN WG2 meeting #130;

- TS 38.473 F1 application protocol (F1AP) (Release 18);

- TS 38.331 Radio Resource Control (RRC) protocol specification (Release 18);

- Report of 3GPP TSG RAN WG2 meeting #125;

- Report of 3GPP TSG RAN WG2 meeting #126.