3GPP TSG RAN WG2 Meeting #131	R2-2505961

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.20.1

Source:	CMCC

Title:	Discussion on RAN4 LS on UE signaling design for NR ATG enh

Document for:	Discussion, Decision

Introduction

Based on the RAN4 R19 ATG WID scope [1], CA (Carrier Aggregation) is supported.

And in this contribution, we would like to provide some considerations on the LS [2] from RAN4 on UE signaling design for NR ATG enh and a corresponding TP is provided in annex.

Discussion

Background

In ATG network, if UE supporting antennaArrayType-r18 on the band of SCC, then when the UE perform measurement on this SCC including both serving cell and neighbour cell, it will cause scheduling restrictions and throughput loss on the SCell, due to different Rx beam directions between serving cell and neighbour cell.

However, the neighbour cell measurement on SCC is not always necessary, for the following two cases, the system performance will not be impacted without neighbour cell measurement on SCC.

Case 1: Neighbour cells has not deployed the SCC which carrier frequency are same with the serving SCC.

Case 2: PCC have larger coverage than SCC, the mobility can rely on PCC measurement.

Therefore, a neighbour cell measurement skipping scheme on SCC matches well with ATG scenario, which can enhance the throughput a lot (25% throughput gain if consider SMTC duration 5ms and SMTC periodicity 20ms), it is very important to guarantee the throughput performance for ATG network.

Observation 0: A neighbour cell measurement skipping scheme on SCC matches well with ATG scenario, which can enhance the throughput a lot (25% throughput gain if consider SMTC duration 5ms and SMTC periodicity 20ms).

Since there is no existing neighbour cell skipping scheme for particular CC, RAN4 agreed to introduce a network signaling to indicate whether an ATG UE supporting antennaArrayType-r18 on the SCC shall perform the neighbour cell measurements or not on the frequency configured by servingcellMO for SCC. And the network signaling is based on RRC. In the LS [2], RAN4 requests RAN2 to specify the detail RRC based signaling to indicate whether an ATG UE supporting antennaArrayType-r18 on the SCC shall perform the neighbour cell measurements or not on the frequency configured by servingcellMO for SCC.

ACTION: RAN4 kindly requests RAN2 to specify RRC based network signaling to indicate whether an ATG UE supporting antennaArrayType-r18 on the SCC shall perform the neighbour cell measurements or not on the frequency configured by servingcellMO for SCC.

Analysis

From RAN2 perspective, there were two solutions including explicit and implicit indication solutions could be considered to support the neighbour cell measurements skipping on the SCC.

Solution 1 is to introduce an explicit indication, for example, one bit indication (e.g. neighCellMeasofScc) could be introduced in MeasObjectNR IE, if it is configured as disabled, it indicates not to allow the UE to perform the neighbour cell measurements on the frequency configured by servingcellMO for SCC, otherwise neighbour cell measurements on the frequency configured by servingcellMO for SCC is allowed.

Solution 2 is an implicit indication solution, which is based on existing mechanism, network could implicitly indicate the UE to skip neighboring measurements by not configuring servingcellMO for SCC. However, if the servingcellMO for SCC is not configured, it may have impact on the serving cell measurements for SCC. Therefore, it is proposed to support the explicit indication (i.e. solution 1).

Observation 1: If the servingcellMO for SCC is not configured, it may have impact on the serving cell measurements for SCC.

Proposal 1: It is proposed to introduce one bit indication (e.g. neighCellMeasofScc) in MeasObjectNR IE, and if it is configured as disabled, it indicates not to allow the UE to perform the neighbour cell measurements on the frequency configured by servingcellMO for SCC.

Proposal 2: If P1 is supported, suggest RAN2 to agree the TP in annex.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions onRAN4 LS on UE signaling design for NR ATG enh and have the following proposal:

Observation 1: If the servingcellMO for SCC is not configured, it may have impact on the serving cell measurements for SCC.

Proposal 1: It is proposed to introduce one bit indication (e.g. neighCellMeasofScc) in MeasObjectNR IE, and if it is configured as disabled, it indicates not to allow the UE to perform the neighbour cell measurements on the frequency configured by servingcellMO for SCC.

Proposal 2: If P1 is supported, suggest RAN2 to agree the TP in annex.

Reference

- RP-243285 Revised WID on Enhancements for Air-to-ground network for NR

- R4-2508329 LS on UE signaling design for NR ATG enh

Annex

- TP for TS 38.331

MeasObjectNR information element

-- ASN1START

-- TAG-MEASOBJECTNR-START

MeasObjectNR ::=                    SEQUENCE {

<<omitted>>

[[

measSequence-r18                    MeasSequence-r18                                                OPTIONAL,   -- Need R

cellsToAddModListExt-v1800          CellsToAddModListExt-v1800                                      OPTIONAL    -- Need N

]],

[[

neighCellMeasofScc-r19           ENUMERATED { disabled }                                           OPTIONAL, -- Need S

]]

}

<<omitted>>

-- TAG-MEASOBJECTNR-STOP

-- ASN1STOP

MeasObjectNR field descriptions

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not provided, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

neighCellMeasofScc When this field is included in MeasObjectNR, it indicates not to allow UE to perform the neighbour cell measurements on the frequency configured by servingcellMO for SCC. If the field is absent, neighbour cell measurements on the frequency configured by servingcellMO for SCC is allowed.

nrofCSI-RS-ResourcesToAverage Indicates the maximum number of measurement results per beam based on CSI-RS resources to be averaged. The same value applies for each detected cell associated with this MeasObjectNR.