3GPP TSG-RAN WG2 Meeting #131	R2-2505965

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.3.3

Source:	CMCC

Title:	Discussion on RRM measurement prediction with NW sided model

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In last meeting RAN2#130 [1], there are discussion on configuration and report of inference input, data collection and Performance monitoring for Network side model. And the following agreements were achieved.

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

- To support cell/beam level prediction, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

Agreements For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity. - The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case. - The availability indication can be triggered due to: - full buffer being reached (if configured), (FFS if buffer is per use case) - buffer threshold being reached (if configured), or (FFS if buffer is per use case) - low power (if configured) - Upon sending the availability indication, UE indicates: - Data is available - Reason for the triggering of the availability indication (full buffer. Threshold) - Low power indication - The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling) - The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). - The UE releases the collected data upon transitioning to IDLE/INACTIVE - For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

Agreements

- RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case

- For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement

- FFS on UE awareness and preference for NW sided model

In this contribution, we provide our consideration on sub-use cases and data collection from UE for RRM measurement prediction with NW sided model.

# 2 Discussion

2.1 Sub-use cases for NW sided model

During the discussion in RAN2#129bis, RAN2 agreed that both cell level and L3 beam level measurement prediction will be studied for NW sided model.

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

For cell level and L3 beam level measurement prediction, RAN2 agreed the following sub-use cases which are specified in TR 38.744 [2].

- For cell level measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated;

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s);

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

- For beam level measurement prediction:

  - Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated;

  - Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s);

  - Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

It was argued in last RAN2 meeting that sub use case 1 and 3 should be considered as low priority because of a lot of spec impacts. But we think the following agreements achieved in last meeting make it feasible to support sub use case 1, 3, 4 and 6 in NW sided model with limited spec impacts.

Agreements

- For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

- To support cell/beam level prediction, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

There is no L3 filtering in RRM measurement by setting the filterCoefficient k to 0, that means L1 beam level measurement results (i.e., RSRP) could be reported to the network via RRM measurement reporting by using the legacy RRM measurement configuration. There is no need to introduce the RRM measurement enhancements to support L1 beam level RSRP reporting. Therefore, all the agreed sub-use cases should be considered for NW sided model to make the network have the same chance with the UE sided model to choose the sub-use case to implement RRM measurement prediction.

Observation 1: L1 beam level measurement results (i.e., RSRP) could be reported to the network via RRM measurement reporting by using the legacy RRM measurement configuration (the filterCoefficient k is set to 0). There is no need to introduce the RRM measurement enhancements to support L1 beam level RSRP reporting. It is feasible to support sub use case 1, 3, 4 and 6 in NW sided model with limited spec impacts.

Proposal 1: All the agreed sub-use cases (i.e., sub-use case 1 to 6) should be considered to support for RRM measurement prediction with NW sided model.

2.2 Data collection for NW sided model

Data collection for training

During the last meeting, there were following agreements for the collection of data for the training of a network-sided model.

UE can be configured to log, at a certain logging periodicity, one or more of the following:

-	L3 cell level measurements,

-	L3 beam level measurements,

-	L1-filtered beam level measurements (if sub-case 1 and 3 is supported)

-	Cell ID (FFS CGI of serving cell.  If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback)

-	Time info (if as agreed by AI/ML and/or if needed)

Data collection for inference

Based on the above agreements on data collection for training, if all the agreed sub-use cases could be supported by NW sided model, we understand L3 cell level measurements and/or L1 beam level measurements and/or L3 beam level measurements should be considered for AI/ML model inference in RRM measurement prediction.

Proposal 2: At least one or more of the following should be considered as the inference input of NW sided model in RRM measurement prediction:

  - L3 cell level measurements

  - L3 beam level measurements

  - L1 beam level measurements

Data collection for monitoring

As per the agreements achieved in last meeting, for performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. And then, the network could calculate KPIs of RRM measurement prediction (e.g., RSRP difference) to evaluate prediction accuracy. The actual measurement results from UE could be L3 cell level measurements, L3 beam level measurements and L1 beam level measurements. And we think the existing periodical RRM measurement reporting can be reused to report the actual measurement for performance monitoring of NW-side model.

Proposal 3: At least one or more of the following should be considered for performance monitoring of NW-side model in RRM measurement prediction:

  - L3 cell level measurements

  - L3 beam level measurements

  - L1 beam level measurements

Proposal 4: The existing periodical RRM measurement reporting can be reused to report the actual measurement for performance monitoring of NW-side model.

# 3 Conclusion

In this paper, we provide our consideration on sub-use cases and data collection from UE for RRM measurement prediction with NW sided model. We have the following observation and proposals:

Sub-use cases for NW sided model

Observation 1: L1 beam level measurement results (i.e., RSRP) could be reported to the network via RRM measurement reporting by using the legacy RRM measurement configuration (the filterCoefficient k is set to 0). There is no need to introduce the RRM measurement enhancements to support L1 beam level RSRP reporting. It is feasible to support sub use case 1, 3, 4 and 6 in NW sided model with limited spec impacts.

Proposal 1: All the agreed sub-use cases (i.e., sub-use case 1 to 6) should be considered to support for RRM measurement prediction with NW sided model.

Data collection for inference

Proposal 2: At least one or more of the following should be considered as the inference input of NW sided model in RRM measurement prediction:

  - L3 cell level measurements

  - L3 beam level measurements

  - L1 beam level measurements

Data collection for monitoring

Proposal 3: At least one or more of the following should be considered for performance monitoring of NW-side model in RRM measurement prediction:

  - L3 cell level measurements

  - L3 beam level measurements

  - L1 beam level measurements

Proposal 4: The existing periodical RRM measurement reporting can be reused to report the actual measurement for performance monitoring of NW-side model.

# Reference

- Draft_RAN2_130_Meeting_Report_v3, May 2025.

- TR 38.744 "Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR".

- TR 38.843 "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".