3GPP TSG-RAN2 Meeting #131			R2-2505969

Bangalore, India, Aug. 25th-29th, 2025

Agenda item:	8.2.3

Title:	Remaining open issues on random access for Ambient IoT

Source:	CMCC

Document for:	Discussion

Introduction

In the last RAN2 meeting [1], we have the following agreement and some FFS.

Agreements 1 For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952). Capture a NOTE that other implementations are allowed. X, Y will be signalled by paging message 2 The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages. R2D trigger message is not sent in CFRA procedure. Come back if RAN1/4 sees any issues. Send LS to RAN1/RAN4 3 FFS R2D byte alignment dependent on TBS size discussion - Agreements on RA - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. Agreements on NACK reception: - After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify. Agreements on RN16/AS ID maintenance: - Confirm a device is not expected to maintain both AS ID and RN16. After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader. This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission. How to capture device behaviour is FFS

As it’s very close to the WI endorsement, this contribution offers some potential solution for the open issues.

Discussion

  - Overall

Since the time left before the WI endorsement is limited, and there are still some issues lacking consensus, e.g., the indication of the collided Random ID in A-IoT Msg2 and how the device determines access failure, it is proposed that, for open issues with divergence, priority should be given to ensuring the feasibility of the current technical solution. Any potential enhancements should be considered in the next release.

  - Access Trigger message

It is noted that there is no variable part in the R2D Access Trigger message. The parameters related to access occasions, such as X and Y, and scheduling-related information can be indicated through the A-IoT paging message. Therefore, no any other information should be captured in the Access Trigger message. For reader collision, maybe a reader identifier should be captured in the Access Trigger message. However, it’s far beyond the RAN2’s scope for now.

Proposal 1a: RAN2 confirms that the Access Trigger message has fixed length, i.e., there will be no TBS indication.

As such, the R2D trigger message should only contain a message header to identify itself. And the message can have a short and predefined size. Therefore, it is not necessary to enforce byte alignment at least for the Access Trigger message, which would only add redundant padding bits without functional benefits.

Observation 1a: Any access occasion related parameter is indicated via the A-IoT paging message.

Thus, the Access Trigger message may only carry a minimal indication, such as the message type. This allows the message to be represented using as few as 3 or 4 bits. Compared to a byte-aligned format, this can result in over 50% reduction in transmitted bits, improving spectral efficiency and reducing unnecessary overhead. Please note that the access occasion should be the most frequently sent message.

Observation 1b: It’s beneficial to keep the Access Trigger message as short as possible as it is the most frequently sent message for A-IoT reader.

In scenarios where the Access Trigger message is lost, the reader can simply transmit additional R2D trigger messages as compensation by its implementation. For device counter that reaches 0x0000, it can flip to 0xFFFF and continues to count down. No any other significant problem is observed so far.

Hence, we suggest that the Access Trigger message is not byte-alignment by mandatory.

Proposal 1b: The Access Trigger message is not byte-aligned by mandatory for simplicity.

In scenarios where the Access Trigger message is lost, the reader can simply transmit additional R2D trigger messages as compensation by its implementation. For device counter that reaches 0x0000, it can flip to 0xFFFF and continues to count down. No any other significant problem is observed so far.

  - Failure determination of Random ID message

In order for the device to determine whether the transmission of the Random ID message, i.e., A-IoT Msg1, has failed or not, a clear boundary is required after which the device should consider the contention-based access attempt as failure.

Based on the discussion at the last RAN2 meeting, one simple and efficient approach is to define this boundary as either the reception of the next R2D trigger message or the subsequent paging message. This method allows the device to make a failure determination based solely on the type of the received message, without needing to maintain a counter of how many R2D trigger messages have been received. This simplifies the device-side implementation and minimizes complexity.

Alternatively, the boundary could also be defined as the reception of the Kth R2D trigger message following device sending A-IoT Msg1. However, this approach introduces additional signalling overhead if the value of K is dynamically indicated in the paging message. On the other hand, if K is pre-configured, the solution sacrifices flexibility, particularly in dynamic or variable deployment scenarios.

Observation 2a: Using the 1st trigger message as the boundary for failure determination is simpler for device as it doesn’t need to maintain another counter.

For the Kth R2D trigger message as boundary, we admit that there will be some gain in the system efficiency, as the spare time interval among commands can be utilized. However, it would be hard to argue the appropriate value of K, and it may have impact on the paging message design, e.g., whether to indicate K in the paging message.

Observation 2b: Using the Kth trigger message as the boundary for failure determination may have some gain in system efficiency, but the range of K, whether K should be configured in the A-IoT paging message are still not discussed yet.

As the time limit for the WI endorsement, although we are open to both alternatives, the simple and feasible option should be prioritized.

Proposal 2: For simplicity, RAN2 should use the 1st trigger message as the boundary. If the Kth trigger message were agreed, K should be a pre-defined value, i.e., K is fixed and there is no impact on the A-IoT paging message.

  - Collision in Random ID response message

The intention to introduce any time or frequency indication in Random ID response message (A-IoT Msg2) is to address the collision caused by multiplexing multiple echoed random IDs.

However, from our perspective, with regrading to the current MAC CR, any device whose random ID collides in the A-IoT Msg2 would naturally fails in the Msg3 transmission and reader can initiate re-access for those devices. Any time or frequency indication in A-IoT Msg2 is an enhancement rather than a necessity.

Observation 3a: As re-access can always address the echoed random ID collision in A-IoT Msg2, introducing any frequency domain indication is an enhancement.

Considering there isn’t much time left before the WI endorsement and there is no solid consensus on the details of frequency indication, e.g., using index or bitmap. If companies still cannot reach any agreement, we suggest to postpone this enhancement to the next release.

For now, the maximum number of time and frequency domain occasion, namely X and Y, is 2 and 8 respectively. As device lacks the capability of accurate timing and the time to send A-IoT Msg3 can be long enough for device to lose synchronization, there must be two A-IoT Msg2 separately if X = 2, which also means explicit time domain indication is not necessary.

Observation 3b: TDMed Msg3 transmission is impossible as device cannot maintain synchronization after the transmission of the first group of A-IoT Msg3.

Regarding the frequency index, maximum 8 bits bitmap per message and 3 bits frequency domain index per collided random ID are both feasible based on RAN2’s agreement.

However, how many collided random ID will be in the A-IoT Msg2 is random, and it’s impossible for reader to know whether any failed random ID happens to be same with any echoed random ID. Hence, reader has to signal every

Observation 3c: Reader has to indicate the frequency domain index for every echoed random ID because any failed random ID may be the same with the echoed random ID.

With above, using the frequency domain index is a bit complicated and may introduce at least uncertain overhead. Bitmap is simpler and can follow some legacy MAC CE format.

Proposal 3a: To address random ID collision in A-IoT Msg2, A-IoT Msg2 should contain a bitmap as frequency domain indication.

There is a natural limitation on the number of echoed Random ID in A-IoT Msg2, i.e., at most Y device(s) can be echoed in the A-IoT Msg2. When the number of successful random IDs is larger than Y, there must be 2 A-IoT Msg2. For example, when Y = 4 and there are 5 echoed random IDs, a second A-IoT Msg2 is necessary.

Proposal 3b: RAN2 agrees that the value of Y is not changed within a paging round.

Another issue is whether to indicate the number of echoed random IDs in the A-IoT Msg2. It’s definitely unnecessary with the bitmap indication as bitmap itself is kind of a quantity indication. If bitmap is not present, the message length or message size can also help device.

Proposal 4: The number of echoed random IDs is not included in A-IoT Msg2.

Conclusion

Based on the above, we propose that:

Access Trigger message:

Proposal 1a: RAN2 confirms that the Access Trigger message has fixed length, i.e., there will be no TBS indication.

Observation 1a: Any access occasion related parameter is indicated via the A-IoT paging message.

Observation 1b: It’s beneficial to keep the Access Trigger message as short as possible as it is the most frequently sent message for A-IoT reader.

Proposal 1b: The Access Trigger message is not byte-aligned by mandatory for simplicity.

Boundary for failure determination:

Observation 2a: Using the 1st trigger message as the boundary for failure determination is simpler for device as it doesn’t need to maintain another counter.

Observation 2b: Using the Kth trigger message as the boundary for failure determination may have some gain in system efficiency, but the range of K, whether K should be configured in the A-IoT paging message are still not discussed yet.

Proposal 2: For simplicity, RAN2 should use the 1st trigger message as the boundary. If the Kth trigger message were agreed, K should be a pre-defined value, i.e., K is fixed and there is no impact on the A-IoT paging message.

Frequency domain indication in A-IoT Msg2:

Observation 3a: As re-access can always address the echoed random ID collision in A-IoT Msg2, introducing any frequency domain indication is an enhancement.

Observation 3b: TDMed Msg3 transmission is impossible as device cannot maintain synchronization after the transmission of the first group of A-IoT Msg3.

Observation 3c: Reader has to indicate the frequency domain index for every echoed random ID because any failed random ID may be the same with the echoed random ID.

Proposal 3a: To address random ID collision in A-IoT Msg2, RAN2 should take the bitmap as frequency domain indication.

Proposal 3b: RAN2 agrees that the value of Y is not changed within a paging round.

Proposal 4: The number of echoed random IDs is not included in A-IoT Msg2.

References

[1]	R2_130_Chair_minutes_2025-05-23_final