3GPP TSG-RAN2 Meeting #131			R2-2505971

Bangalore, India, Aug. 25th-29th, 2025

Agenda item:	8.7.6

Title:	Remaining open issues on rate control for XR

Source:	CMCC

Document for:	Discussion

Introduction

In the last RAN2 meeting [1], RAN2 has reached following agreements on rate control and left a few FFS.

Agreements for XR rate control - (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. - (MAC-04) We rule out explicit signalling of PDU session ID + QFI - (MAC-04) We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping - (RRC-1) The granularity of bit rate query prohibit timer is QoS flow. - FFS The value of the prohibit timer is the same for all flows - (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow. - (MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit. - (MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate. - (MAC-09) No optimization is needed to address DC case for XR rate control MAC CE - (MAC-09) We capture in stage-2 that XR rate indication is for application and not for MAC entity. - (MAC-11) RAN2 will not specify DL rate control and DL rate control query

This contribution focuses on the remaining issues of scheduling enhancement, and propose several solutions for them.

Discussion

  - Indication of QoS flow in UL Rate Control MAC CE

Since multiple QoS flows may be associated with a single radio bearer, the QoS flow indication based on bitmap would require a number of bits, i.e., 63 bits, proportional to the number of possible QoS flows, which can become excessive and lead to unnecessary overhead, especially when only a single QoS flow needs to be indicated in the MAC CE.

Therefore, indication based on index is preferred, and there are two possible index schemes. One is relative index, i.e., numbering the QoS flows in ascending order within a radio bearer; the other is absolute index i.e., using a unique identifier to each QoS flow (e.g., 6 bits for up to 63 flows).

While the relative index has the advantage of less overhead, it requires both gNB and UE to maintain an up-to-date mapping of the current QoS flow order within each DRB, which can introduce complexity and ambiguity during reconfiguration. Besides, it’s also hard to predict how many QoS flows there will be in the radio bearer, it would cause some difficulties in MAC CE design.

In contrast, the absolute index directly corresponds to the configured QFI and avoids dependency on dynamic ordering. Given that the number of QoS flows per radio bearer is not always predictable and may vary with service configuration, using the absolute index ensures unambiguous identification and minimizes coordination overhead, while still keeping the signaling size bounded.

For indicating the QoS flow in the UL Rate Control MAC CE, the combination of DRB ID and QFI is supported, with the use of an index rather than a bitmap.

Proposal 1: Explicit DRB and QFI Index is utilized to indicate the QoS flow in UL Rate Control MAC CE.

  - Prohibit timer for UL rate control

If the prohibit timer were configured per QoS flow, the MAC entity would need to maintain a large number of independent timers—potentially up to the maximum number of configured QoS flows—which would introduce substantial complexity and lots of overhead for MAC entity.

In addition, since the MAC entity does not have direct visibility of per-QoS-flow delay information, per-flow timers would offer little practical benefit for timer optimization.

From our point of view, configuring the prohibit timer per MAC entity is preferred, because the UL Rate Control MAC CE related procedure is aligned with existing MAC CE procedures, e.g., BSR, DSR and PHR. The implantation complexity is also reduced, by avoiding the need for fine-grained per-QoS-flow timer management. It would also offer a predictable behavior at both UE and gNB side, which means that the UL Rate Control MAC CE are subject to a single, coherent timing constraint, simplifying both UE and gNB behavior.

Proposal 2: The prohibit timer is the same for all QoS flows.

  - UL Rate Control MAC CE

The UL Rate Control MAC CE shall include fields to indicate DRB ID, QFI ID, and the bit rate. Specifically, the DRB-Identity ranges from 1 to 32, max QFI is 63, therefore their indication are 5 and 6 bits separately.

For bit rate indication, it would be better if it’s 8bits, to our understanding, the rate control is not very precise hence finer granularity is not always welcome. Besides, 16 bits bit rate would have an all zero byte for smaller bit rate, which is more common in case of congestion.

The DRB ID and QFI entries within the MAC CE can be ordered in ascending order of their indexes. This ordering ensures unambiguous interpretation of the MAC CE content and avoids the need for additional sequence identifiers or mapping information.

Proposal 3a: The UL Rate Control MAC CE should include a 5-bit DRB ID, a 6-bit QFI and an 8-bit target bit rate field and a 1-bit E field to indicate whether the QFI is corresponding to the former DRB.

The priority of the newly defined UL Rate Control MAC CE shall be identical to that of the legacy UL Rate Control MAC CE, thereby ensuring backward compatibility and preventing unintended changes in scheduling behavior when both legacy and enhanced MAC CEs are present in the system.

Proposal 3b: The priority of the UL Rate Control MAC CE is the same as the legacy MAC CE.

  - UE behavior if UL grant cannot accommodate UL Rate Control MAC CE

The UL Rate Control MAC CE shall follow the same transmission scheme as the DSR and other legacy MAC CEs except padding BSR, which means it does not support truncated format either.

Specifically, if the available uplink grant is insufficient to accommodate the entire UL Rate Control MAC CE, the MAC CE shall remain pending in the UE buffer until sufficient uplink resources are granted. This approach avoids the introduction of a truncated MAC CE variant, which would increase both signalling complexity and interpretation ambiguity at the gNB. It also ensures that the complete set of intended parameters (e.g., DRB ID, QFI, bit rate) is delivered in a single, coherent transmission, thereby preventing partial or inconsistent rate control updates.

Proposal 4: If UL grant cannot accommodate the UL Rate Control MAC CE, the MAC CE should remain pending and waits for next UL grant until it is cancelled.

  - UE capability on support of UL Rate Control

According to the liaison from SA2, the CN can indicate whether a specific QoS flow is subject (no capable) to uplink rate control. This is distinct from indicating whether the QoS flow supports uplink rate control. The CN can make this determination based on knowledge of the QoS flow characteristics, such as the 5QI configuration or through deep packet inspection, and thus is aware of whether the QoS flow inherently supports uplink rate control.

It is noted that rate adaptation is largely associated with the transport protocol or application layer—for example, most modern TCP/UDP/QUIC implementations incorporate congestion control mechanisms, enabling rate adaptation based on packet loss or delay. From a RAN perspective, the focus should be on rate adaptation capability within the AS layer rather than relying on UE indications to the CN regarding application-level rate adaptability.

Consequently, explicit UE indication to the CN on whether a QoS flow is rate-adaptable is considered unnecessary. Additionally, multiplexing QoS flows with and without rate adaptation in the same DRB is not a critical concern, as the UL Rate Control MAC CE is defined on a per-QoS-flow basis.

Proposal 5: UE indication on rate adaptability to the CN is not necessary.

To avoid gNB indicate too many QoS flows for rate control that UE cannot handle, the UE shall support a minimum number of QoS flows that can be subject to rate control, the proposed value can be 2 or 4. This baseline ensures that common service configurations, including XR services and typical multimedia sessions, can be supported without excessive signaling.

Proposal 6a: A UE that supports UL rate control shall also support rate control operation for a minimum number of QoS flows simultaneously. FFS it’s value, e.g., 2 or 4.

For UEs capable of supporting more than the baseline number of rate-controlled QoS flows, an additional UE capability indication may be provided to report the exact maximum number of QoS flows subject to rate control. This allows the gNB to optimize MAC CE construction and scheduling, particularly in scenarios with large-scale QoS flow usage such as multi-camera streaming or complex enterprise applications. By clearly defining both the baseline and extended capability, interoperability is maintained while enabling optimization for high-capability devices.

Proposal 6b: If UE supports rate control for more QoS flows, UE can indicate the exact value in the UE capability message to RAN.

Conclusion

Based on the above, we propose that:

QoS flow indication:

Proposal 1: Explicit DRB and QFI Index is utilized to indicate the QoS flow in UL Rate Control MAC CE.

Prohibit timer:

Proposal 2: The prohibit timer is the same for all QoS flows.

MAC CE:

Proposal 3a: The UL Rate Control MAC CE should include a 5-bit DRB ID, a 6-bit QFI and an 8-bit target bit rate field and a 1-bit E field to indicate whether the QFI is corresponding to the former DRB.

Proposal 3b: The priority of the UL Rate Control MAC CE is the same as the legacy MAC CE.

Proposal 4: If UL grant cannot accommodate the UL Rate Control MAC CE, the MAC CE should remain pending and waits for next UL grant until it is cancelled.

UE capability:

Proposal 5: UE indication on rate adaptability to the CN is not necessary.

Proposal 6a: A UE that supports UL rate control shall also support rate control operation for a minimum number of QoS flows simultaneously. FFS it’s value, e.g., 2 or 4.

Proposal 6b: If UE supports rate control for more QoS flows, UE can indicate the exact value in the UE capability message to RAN.

References

[1]	R2_130 R19 XR and LTE Broadcast notes (Dawid) 2025-05-23_EOM_rev1