3GPP TSG-RAN WG2 Meeting #131			         R2-2505978

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.5.3

Source: 	LG Electronics Inc.

Title:         	Remaining issues on OD-SIB1 operation

Document for: 	Discussion and Decision

Introduction

In this contribution we discuss following open issues listed in the summary of [POST130][107][NES] Running RRC CR (Ericsson)

- FFS whether to support OD-SIB1 request/reception via SUL

- Issue 1 (Q12): Can “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1” be added to the field description of si-BroadcastStatus to address the FFS?

- Issue 2 (vivo003): How to receive OD-SIB1 upon receiving an SI change notification

Discussion

FFS whether to support OD-SIB1 request/reception via SUL

RAN2 agreed to support OD-SIB1 even for cell selection while T311 is running. The cell selection aims to quickly find a suitable cell on which the UE can camp. When performing cell selection, if there is a legacy cell available for camping, the UE will not attempt to camp on an OD-SIB1 cell. If the UE requests OD-SIB1 while T311 is running, it is likely because there is no legacy cell nearby that it can camp on. Therefore, in the absence of alternatives, if the UE is located within SUL coverage, it should be able to request SIB1 via SUL.

Supporting SUL does not require further work in RAN1, as was the cast for on-demand SI request via SUL. From the RAN2 perspective, since the need to support OD-SIB1 request/reception via SUL is clear, we propose to agree on supporting OD-SIB1 request/reception via SUL and to inform RAN1 accordingly.

Proposal 1	Support OD-SIB1 request/reception via SUL.

Proposal 2	Include information required for UL carrier selection in the WUS configuration.

Issue1 (Q12): Can “The UE supporting OD-SIB1 in RRC_CONNECTED considers the stored SIB1 as the latest SIB1” be added to the field description of si-BroadcastStatus to address the FFS?

- Agreement in RAN2#130:

Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

According to the agreement, the UE behaviour is already captured in 5.2.2.3.5 as follows:

5.2.2.3.5 Acquisition of SIB(s) or posSIB(s) in RRC_CONNECTED The UE shall: (omitted) 2> a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one.

Therefore, there is no need to duplicate the same description in the field description of si-BroadcastStatus.

Proposal 3	[Q12] No further clarification in the field description of si-BroadcastStatus. (It is already clear in 5.2.2.3.5)

However, this UE behaviour should only apply when the serving cell is transmitting SIB1 in on-demand manner. In the current CR, the restriction corresponding to “in NES Cell” in the agreement is missing. In addition, this UE behaviour should be specified before the UE behaviour that uses si-BroadcastStatus. Therefore, it should be revised as follows:

5.2.2.3.5 Acquisition of SIB(s) or posSIB(s) in RRC_CONNECTED The UE shall: (omitted) 1> else if the UE is in RRC_CONNECTED with an active BWP configured with common search space with the field searchSpaceOtherSystemInformation and the UE has not stored a valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) or posSIB(s) in accordance with clause 5.2.2.1: 2> if ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> a UE supporting OD-SIB1 considers the si-BroadcastStatus in the stored SIB1 as the latest one; 2> for the SI message(s) that, according to the si-SchedulingInfo in the stored SIB1, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting: 3> acquire the SI message(s) as defined in clause 5.2.2.3.2;

Proposal 4	[Q12] Add the condition “if ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell” and change the position as described above.

Issue2 (vivo002): UE behavior after receiving SI change notification from OD-SIB1 cell

Upon reception of the SI change indication, the UE acquires MIB and SIB1. According to current CR, the UE will request on-demand SIB1 transmission, if UE considers SIB1 acquisition is required:

1> if the UE is in RRC_IDLE or in RRC_INACTIVE; or (omitted) 2> else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell: 3> if the UE has a stored valid version of od-SIB1-Config for this cell: 4> perform the actions as specified in clause 5.2.2.3.3x;

Therefore, it is already clear that UE requests on-demand SIB1 upon reception of the SI change indication from OD-SIB1 cell, and no further discussion/decision on this case is needed.

Proposal 5	Upon receiving the SI change indication from an OD-SIB1 cell, the UE requests on-demand SIB1 transmission from the OD-SIB1 cell. (No CR change).

Issue3 (HW006): ra-SearchSpace-r19

In the current running CR, only a SearchSpaceId is signaled without SearchSpace itself to configure the search space for random access procedure, i.e. ra-SearchSpace-r19, for serving cell and neighbor cells. For SIB1 request from serving cell the SearchSpaceId in SIB1-RequestConfig-r19 is sufficient since the corresponding SearchSpace configuration indicated by the SearchSpaceId is present in PDCCH-ConfigCommon or PDCCH-Config received from the serving cell. However, since the UE does not yet have the PDCCH configuration of neighbor cell, it cannot derive the search space configuration solely from the search space ID. Therefore, search space configuration, rather than the search space ID, should be provided.

Proposal 6	For ra-SearchSpace-r19, use CHOICE structure consisting of SearchSpaceId (for serving cell) and SearchSpace (i.e. for neighbor cell).

Conclusion

Proposal 1	[SUL issue] Support OD-SIB1 request/reception via SUL.

Proposal 2	[SUL issue] Include information required for UL carrier selection in the WUS configuration.

Proposal 3	[Q12] No further clarification in the field description of si-BroadcastStatus. (It is already clear in 5.2.2.3.5)

Proposal 4	[Q12] Add the condition “if ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell” and change the position as described above.

Proposal 5	[vivo 002] Upon receiving the SI change indication from a OD-SIB1 cell, the UE requests on-demand SIB1 transmission from the OD-SIB1 cell. (No CR change).

Proposal 6	[HW006] For ra-SearchSpace-r19, use CHOICE structure consisting of SearchSpaceId (for serving cell) and SearchSpace (i.e. for neighbor cell).