3GPP RAN WG2 Meeting #131	R2-2505986

Bengaluru, India, Aug 25th – 29th 2025

Agenda Item:	8.8.6

Source:	InterDigital

Title:	Discussion on LTE to NR NTN mobility

Document for:	Discussion, Decision

# Introduction

.In RAN2#130 meeting [1], dedicated priority for LTE to NR NTN mobility was discussed but RAN2 couldn’t reach any consensus.

This contribution discusses about the dedicated priority for LTE to NR NTN mobility.

# Discussion

A contribution [2] for the dedicated priority was treated in RAN2#130 but RAN2 postponed to discuss it.

Proposal 1	Similar to redirection, include the satellite ID information within the FreqPriorityNR IE to help UE know whether it should prioritize the NR NTN cells or NR TN cells.

Can come back in the next meeting

According to 38.331 CR for LTE to NR NTN mobility [3], the following changes were applied in the ASN.1 definition of RRCConnectionRelease message for the support of redirection.

RRCConnectionRelease-v1650-IEs ::=	SEQUENCE {

mpsPriorityIndication-r16				ENUMERATED {true}	OPTIONAL, -- Cond Redirection2

nonCriticalExtension					RRCConnectionRelease-v19xy-IEs		OPTIONAL

}

RRCConnectionRelease-v19xy-IEs ::=	SEQUENCE {

ntn-RedirectedCarrierInfoNR-r19		NTN-RedirectedCarrierInfoNR-r19	OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}		OPTIONAL

}

NTN-RedirectedCarrierInfoNR-r19 ::=	SEQUENCE {

satAssistanceInfoList-r19			SEQUENCE (SIZE(1..maxSat-r17)) OF SatelliteId-r18	OPTIONAL,	-- Need ON

redirectedCarrierInfo-r19			CarrierInfoNR-r15

}

Observation 1: Satellite ID list is present per redirectedCarrierInfo-r19 to clarify that the destination is NR NTN and provide necessary assistance information for the UE.

For the case of the redirection from LTE to NR NTN, an NR NTN specific redirection IE was defined so that the network can provide the satellite ID list for a destination NR carrier frequency.

The addition of the satellite ID list was necessary to avoid any confusion at the UE for the case that NR NTN frequencyies and NR TN frequencies are overlapping.

The same principle should be applied for the dedicated priority signalling to address the NR NTN and NR TN frequency overlapping case.

Proposal 1:	The satellite ID list is present for each entry of FreqPriorityListNR-r15

# Conclusion

In this contribution the following observation and proposal are made for dedicated priority signalling for LTE to NR NTN mobility:

Observation 1: Satellite ID list is present per redirectedCarrierInfo-r19 to clarify that the destination is NR NTN and provide necessary assistance information for the UE.

Proposal 1:	The satellite ID list is present for each entry of FreqPriorityListNR-r15

# References

R2-2504673	Report from session on NR NTN and IoT NTN	Session chair (ZTE), RAN2#130, St. Julians, Malta, May 2025

R2-2504007	Discussion on dedicated priority via RRCConnectionRelease	OPPO, RAN2#130, St. Julians, Malta, May 2025

R2-2501418, Introduction of LTE TN or NR NTN IDLE mode mobility, CATT, RAN2#129, Athens, Greece, February 2025