3GPP TSG-RAN WG2 Meeting #131											R2-2505990

Bengaluru, India, 25th– 29th August 2025

Agenda item:		8.5.3

Title: 	Remaining issues of on-demand SIB1

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some consideration on the remaining issues for on demand SIB1, with the latest higher parameter list [1] from RAN1 taken into consideration.

- Discussion

  - Whether to support SUL for OD-SIB1 request

There has been discussion last meeting on whether to support SUL for OD-SIB1 request with no conclusion reached [1]:

----------------------------------------------

R2-2503391	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1a: SIB 1 request is supported for both SUL and NUL.

Proposal 1b: RSRP threshold for UL carrier selection is included in SIB1 request configuration.

[ZTE]: Probably no much impact on RAN2, but it was excluded in RAN1 decision. [Huawei]: Want to follow RAN1 discussion and decision. [Samsung]: Understand RAN1 actually has never discussed specific SUL aspect. [ZTE]: Understand RAN1 clearly excluded SUL. [IDC]: Support the proposal. [Xiaomi]: Share the view with Samsung. [LG]: Would like to have more time to check.

- Revisit it next meeting.

----------------------------------------------

Per our understanding, there are two issues here need to be clarified:

- Issue 1: Whether to support UL-WUS configuration, i.e. on demand SIB1 request configuration, in SUL for the OD-SIB1 cell?

- Issue 2: Whether to support SUL in an OD-SIB1 cell?

For issue 2, it is clear that once an OD-SIB1 cell start to broadcast SIB1, it can have both NUL and SUL and UE will follow the existing procedure when selecting the UL for random access.

Proposal 1: For an OD-SIB1 cell, once it starts to broadcast SIB1, it is up to NW implementation to broadcast NUL configuration with or without SUL in SIB1.

While for issue 1, the views are split as shown in the RAN2 discussion history copied above.

From RAN2’s perspective, the spec effort to support SUL for on demand SIB1 request is predictable, e.g. add sib1-RequestConfigSUL in SIBXX, while the major concern is whether the need for supporting SUL is consistent among RAN WGs.

After double checking the RAN1 discussion history, we found the following conclusion from RAN1#120 [2]:

----------------------------------------------

RAN1#120

Conclusion (amended as shown in red in Friday session)

- The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system.

- RA-RNTI is not included in the UL-WUS configuration for FDD system.

- Cell barring information is not included in the UL-WUS configuration for FDD system.

- Cell selection information is not included in the UL-WUS configuration for FDD system.

- Accessible UE types is not included in the UL-WUS configuration for FDD system.

- CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration for FDD system.

- InitialUplinkBWP is not included in the UL-WUS configuration for FDD system.

- TotalNumberOfRA-Preambles is not included in the UL-WUS configuration for FDD system.

- SIB1-RequestResourceRedCap is not included in the UL-WUS configuration for FDD system.

- SupplementaryUL is not included in the UL-WUS configuration

-----------------------------------------------

Proposal 2: Follow RAN1 conclusion on whether to support SUL for on demand SIB1 request configuration, i.e. SupplementaryUL is not included in the UL-WUS configuration. LS should be sent to RAN1 for confirmation if RAN2 make different agreement.

  - Whether to support Msg1 repetition for OD-SIB1 request

For on demand SI request via Msg1, we have the following configuration for RedCap and Msg1 repetition [3]:

si-RequestConfigMSG1-Repetition-r18           SI-RequestConfigRepetition-r18          OPTIONAL,  -- Cond MSG-1

si-RequestConfigRedCap-MSG1-Repetition-r18    SI-RequestConfigRepetition-r18          OPTIONAL,  -- Cond REDCAP-MSG-1

si-RequestConfigSUL-MSG1-Repetition-r18       SI-RequestConfigRepetition-r18          OPTIONAL   -- Cond SUL-MSG-1

-- ASN1START

-- TAG-SI-REQUESTCONFIGREPETITION-START

SI-RequestConfigRepetition-r18 ::=     SEQUENCE {

rach-OccasionsSI-r18                   SEQUENCE {

rach-ConfigSI-r18                      RACH-ConfigGeneric,

ssb-perRACH-Occasion-r18               ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                               OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum2-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum4-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

si-RequestResourcesRepetitionNum8-r18  SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResourcesRepetition-r18  OPTIONAL,  -- Need R

...

}

SI-RequestResourcesRepetition-r18 ::=  SEQUENCE {

ra-PreambleStartIndex-r18              INTEGER (0..63)

}

-- TAG-SI-REQUESTCONFIGREPETITION-STOP

-- ASN1STOP

Similarly, we need to consider whether to support Msg1 repetition for on demand SIB1 request procedure. Per our understanding that the Msg1 repetition for PRACH coverage enhancement is mainly applied for UL heavy traffic while on demand SIB1 for network energy saving will be mostly deployment when the traffic load is relatively low. Thus, we understand there is no strong motivation to support Msg1 repetition for on demand SIB1 request.

Proposal 3: Msg1 repetition is not supported for on demand SIB1 request.

  - OD-SIB1 specific reselection priority

We reached the following agreement for OD-SIB1 specific reselection priority [4]:

----------------------------------------------

RAN2#128:

New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency

----------------------------------------------

And the following parameters have been introduced in SIB2 and SIB4 for the serving frequency and neighbor frequencies:

odsib1-CellReselectionPriority-r19             CellReselectionPriority              OPTIONAL, Need R

odsib1-CellReselectionSubPriority-r19          CellReselectionSubPriority           OPTIONAL  Need R

And it has been described in 38.304 that these priorities will override the common priorities broadcast as copied below [5]:

------------------38.304 CR----------------------------

If dedicated frequency priority parameters (odsib1-CellReselectionPriority, odsib1-CellReselectionSubPriority) are provided in system information, the UE supporting OD-SIB1 ignores the cellReselectionPriority and cellReselectionSubPriority in the system information and applies the dedicated ones to determine frequency prioritization. If intraFreqODSIB1-ExcludedCellList is provided in system information, the UE supporting OD-SIB1 ignores intraFreqExcludedCellList and does not consider the cell(s) (if any) in intraFreqODSIB1-ExcludedCellList as candidates for cell reselection. If interFreqODSIB1-ExcludedCellList is provided in system information, the UE supporting OD-SIB1 ignores interFreqExcludedCellList and does not consider the cell(s) (if any) in interFreqODSIB1-ExcludedCellList as candidates for cell reselection.

------------------38.304 CR----------------------------

Observation 1: The OD-SIB1 specific cell reselection priorities override the common broadcast reselection priorities.

Apart from these OD-SIB1 specific reselection priorities, currently we have reselection priorities broadcast via system information and also dedicated reselection priorities provided via dedicated signaling as copied below:

CellReselectionPriorities ::=       SEQUENCE {

freqPriorityListEUTRA               FreqPriorityListEUTRA                   OPTIONAL, Need M

freqPriorityListNR                  FreqPriorityListNR                      OPTIONAL, Need M

t320                                ENUMERATED {min5, min10, min20, min30, min60, min120, min180, spare1} OPTIONAL,     -- Need R

...,

[[

freqPriorityListDedicatedSlicing-r17 FreqPriorityListDedicatedSlicing-r17    OPTIONAL Need M

]]

}

FreqPriorityListEUTRA ::=           SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=              SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR

FreqPriorityEUTRA ::=               SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                OPTIONAL Need R

}

FreqPriorityNR ::=                  SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                OPTIONAL Need R

}

Usually, the dedicated priorities override the broadcast priorities as stated in 38.304 [6]:

------------------38.304---------------------

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If any fields with cellReselectionPriority or nsag-CellReselectionPriority are provided in dedicated signalling, the UE shall ignore any fields with cellReselectionPriority and nsag-CellReselectionPriority provided in system information.

------------------38.304---------------------

Then we need to consider how to handle the case when both OD-SIB1 specific reselection priorities and dedicated reselection priorities are configured for the same frequency.

Per our understanding, the following options can be considered:

- Option 1: Introduce odsib1-CellReselectionPriority and odsib1-CellReselectionSubPriority also in dedicated signalling and UE supporting OD-SIB1 and not supporting OD-SIB1 will follow the corresponding reselection priorities configured. If odsib1-CellReselectionPriority and odsib1-CellReselectionSubPriority is not provided via dedicated signaling, the reselection priority provided via dedicated signaling override the cell reselection priorities in system information.

- Option 2: Follow the existing rule that the dedicated priorities override any priorities provided via system information, including the OD-SIB1 specific reselection priorities. Since the dedicated reselection priorities are configured per UE, NW should be able to configure suitable UE specific reselection priorities with UE capability for OD-SIB1 considered. This requires capability for OD-SIB1, which is now defined as optional without capability signaling by RAN1.

Option 2 is preferred from our side for simplicity.

Proposal 4: Follow the existing rule that the reselection priorities provided via dedicated signaling override any reselection priorities provided via system information, including the OD-SIB1 specific reselection priorities.

Proposal 5: The OD-SIB1 capability is defined as optional with capability signaling to assist the dedicated reselection priority configuration at network side.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: For an OD-SIB1 cell, once it starts to broadcast SIB1, it is up to NW implementation to broadcast NUL configuration with or without SUL in SIB1.

Proposal 2: Follow RAN1 conclusion on whether to support SUL for on demand SIB1 request configuration, i.e. SupplementaryUL is not included in the UL-WUS configuration. LS should be sent to RAN1 for confirmation if RAN2 make different agreement.

Proposal 3: Msg1 repetition is not supported for on demand SIB1 request.

Observation 1: The OD-SIB1 specific cell reselection priorities override the common broadcast reselection priorities.

Proposal 4: Follow the existing rule that the reselection priorities provided via dedicated signaling override any reselection priorities provided via system information, including the OD-SIB1 specific reselection priorities.

Proposal 5: The OD-SIB1 capability is defined as optional with capability signaling to assist the dedicated reselection priority configuration at network side.

- Reference

- Draft_RAN2_130_Meeting_Report_v3

[2] Final_Minutes_report_RAN1#120_v100

[3] TS 38.331 V18.6.0

[4] Draft_RAN2_128_Meeting_Report_v4

[5] R2-250xxxx - draftCR_TS38.304 NES_RunningCR-v02_Rapp (clean)

[6] TS 38.304 V18.4.0