3GPP TSG-RAN WG2 Meeting #131											R2-2505991

Bengaluru, India, 25th– 29th August 2025

Agenda item:		8.5.4

Title: 	Remaining issues of common signal/channel adaption

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the remaining issues of common signal/channel adaption.

- Discussion

  - Paging occasion adaptation

The following agreements have been made in the recent meeting on paging adaptation:

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

If we introduce N with new value T/32 and Ns with new value 8, the following new parameters are needed for firstPDCCH-MonitoringOccasionOfPO and firstPDCCH-MonitoringOccasionOfPEI-O for paging adaption as we prefer to keep the current granularity of symbol level:

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT											SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT											 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..143359)

Proposal 1: The above new values for firstPDCCH-MonitoringOccasionOfPO and firstPDCCH-MonitoringOccasionOfPEI-O should be introduced for paging adaption.

  - SSB adaptation

The following parameters have been agreed by RAN1 [1] but not included in the latest RRC CR:

| addl-ssb-Periodicity | Additional SSB burst periodicity for the Scell. | same as ssb-periodicity X≤2 additional SSB burst periodicities from SSB burst periodicities in ssb-periodicity |
| --- | --- | --- |
| ssbPeriodicityIndication-RNTI | The RNTI value for scrambling CRC of DCI format 2_9 for SSB burst periodicity switching for Scell(s). | RNTI-Value |
| SearchSpace | The search space for monitoring DCI format 2_9 for SSB burst periodicity switching. | Existing searchSpace configuration with addition of dci-Format2-9 |
| sizeDCI-2-9-r18 | The size of DCI format 2_9 for SSB burst periodicity switching(see TS 38.212, subclause 7.3.1.3.10). | same as for existing sizeDCI-2-9-r18 |
| positionInDCI-ssbPeriodicityIndicationForScell | The starting bit position of an information block of DCI format 2_9 for SSB burst periodicity switching of this serving cell (see TS 38.212, subclause 7.3.1.3.10). | 0..maxDCI-2-9-Size-1-r18 |
| addl-ssb-Offset | Indicate SFN offset from the SFN which satisfies (SFN index *10) modulo (addl-SSB periodicity) = 0 | INTEGER (0..15) |
| addl-ssb-halfFrameIndex | Indicate whether SSB according to the addl-SSB-periodicity is in the first half or the second half of the frame. | [0, 1] |

Proposal 2: Based on the latest input from RAN1, the following parameters should be included for SSB adaption:

- addl-ssb-Periodicity

- ssbPeriodicityIndication-RNTI

- SearchSpace

- sizeDCI-2-9-r18

- positionInDCI-ssbPeriodicityIndicationForScell

- addl-ssb-Offset

- addl-ssb-halfFrameIndex

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT											SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT											 SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..35839),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..143359)

Proposal 1: The above new values for firstPDCCH-MonitoringOccasionOfPO and firstPDCCH-MonitoringOccasionOfPEI-O should be introduced for paging adaption.

Proposal 2: Based on the latest input from RAN1, the following parameters should be included for SSB adaption:

- addl-ssb-Periodicity

- ssbPeriodicityIndication-RNTI

- SearchSpace

- sizeDCI-2-9-r18

- positionInDCI-ssbPeriodicityIndicationForScell

- addl-ssb-Offset

- addl-ssb-halfFrameIndex

- Reference

- R1-2503243_Consolidated Rel-19 higher layers parameters list Post RAN1#121