3GPP TSG-RAN WG2 Meeting #131										  R2-2505993

Bangalore, India Aug 25th – 29th, 2025						Revision of R2-2503764

Agenda item:		8.4.4

Title: 	Procedures for LP-WUS in RRC_CONNECTED

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this contribution, we will further discuss the RAN2 related issues on procedure of LP-WUS for RRC_CONNECTED state and give our proposals.

- Discussion

2.1 Activation/deactivation for LP-WUS monitoring in RRC_CONNECTED state

It has been agreed that the LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. Considering that the coverage of LP-WUS may be less than or equal to the cell coverage, even the LP-WUS related configuration is provided, the LP-WUS may not be monitored, e.g. when the UE is at the cell edge. The LP-WUS monitoring activation/deactivation should be considered.

Usually, there are two solutions for LP-WUS monitoring activation/deactivation:

- Solution 1: similar as RRC_IDLE/RRC_INACTIVE state, a cell edge condition (e.g. RSRP threshold) is configured to UE, and UE monitors LP-WUS in RRC_CONNECTED state when it is not at the cell edge. For this solution, Network cannot know whether the UE is monitoring the LP-WUS and should always send LP-WUS, whichever the UE monitors it or not. In this case, for Option 1-1, Network couldn’t know whether UE starts the drx-onDurationTimer.

- Solution 2: Network activate or deactivate the LP-WUS explicitly, e.g. with RRC message, DCI or MAC CE. For this solution, UE need monitor LP-WUS only when it is activated by network, and network can activate or deactivate the LP-WUS based on the CQI info or UE assistance information reported by UE. So that network can determine whether and when to activate or deactivate the LP-WUS monitoring in RRC_CONNECTED state, LP-WUS monitoring entry and exit condition (e.g. RSRP and optional RSRQ threshold) should be configured to UE (e.g. by RRCReconfiguration message), and UE can report LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message .

To keep alignment between UE LP-WUS monitoring and gNB LP-WUS transmission, solution 2 is preferred. For light signaling load and not impact RAN1 on DCI design, MAC CE is suggested to be used for network to activate or deactivate the LP-WUS explicitly.

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

## 2.2 LP-WUS monitoring schemes with C-DRX configured

IN RAN2#130 meeting [1], the following working assumption has been made:

RAN2#130 progress - Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

In the current specification, secondary DRX Group can be configured for UE power saving when both FR1 and FR2 cells are configured in one cell group, in which case the UE power consumption can be reduced if a separate drx-InactivityTimer and drx-onDurationTimer in secondary DRX Group is configured for the FR2 cells enabling FR2 to go to sleep more quickly.In RAN1#120bis meeting [2], the following conclusion has been made:

Conclusion For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19. - Alt1 - Basic capability - LP-WUS can be configured only on PCell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Advanced capability - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has prerequisite of the above basic capability - Alt2 - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

The main concern for option 1-1 to configure LP-WUS per DRX group is that: the C-DRX cycle is common for dual DRX group, only drx-onDurationTimer and drx-InactivityTimer can be configured for DRX group and secondary DRX group separately, and the drx-onDurationTimer is always started simultaneously for the two DRX group.

Based on the LS R2-2505020_R1-2504888[4], RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19.

Thus, it is not possible to support LP-WUS per DRX group and wake up each DRX group separately because if one DRX group is activated by LP-WUS with MR operating, the UE cannot monitor LP-WUS for the other DRX group at the same time.

Observation 1: Per the RAN1 input that UE is not able to operate LR and MR simultaneously in Rel-19, it is not possible to support LP-WUS per DRX group and wake up each DRX group separately as if one DRX group is activated by LP-WUS with MR operating, the UE cannot monitor LP-WUS for the other DRX group at the same time.

Since both secondary DRX Group and LP-WUS are used for UE power saving, with the same mechanism as drx-onDurationTimer starting, one LP_WUS subgroup can be configured per cell group, which can always wakes up both DRX groups simultaneously.

Proposal 2: Confirm the following working assumption to support LP-WUS with dual DRX group for both option 1-1 and 1-2, and when LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

For Option 1-2, as ‘UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)’ in RAN1, UE can determine the LP-WUS monitoring occasion using the similar formula as that to start legacy drx-onDurationTimer. Considering that LP-WUS monitoring consumes much less UE power than PDCCH monitoring, and smaller LP-WUS cycle can decrease service delay, one LP-WUS cycle with smaller value can be configured, which does not differentiate long LP-WUS cycle and short LP-WUS cycle.

Proposal 3: For Option 1-2, RAN2 confirms that one single LP-WUS cycle is defined to determine the LP-WUS monitoring occasion.

In Rel-18, non-integer C-DRX cycle is defined to match the non-integer traffic periodicity (e.g. XR periodicity), considering the low power consumption, small granularity of LP-WUS cycle (e.g. value of 1ms) can be used to match the non-integer C-DRX periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 4: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

For option1-2, when LP-WUS is configured with secondary DRX Group simultaneously, one LP-WUS monitoring configuration (e.g. LP-WUS monitoring cycle, time offset) is configured and UE monitors one LP-WUS, LP-WUS subgroup(s) are used to indicate for which DRX group the lpwus_PDCCHMonitoringTimer starts.

Similar as drx-onDurationTimer, lpwus_PDCCHMonitoringTimer can be configured per DRX group.

Proposal 5: For option1-2, when LP-WUS is configured with secondary DRX Group simultaneously, one LP-WUS monitoring configuration (LP-WUS cycle, timeOffset) is configured per cell group, LP-WUS subgroup number and lpwus_PDCCHMonitoringTimer is configured per DRX group.

## 2.3 About empty UAI on LP-WUS offset

There is the following option issue that is still not concluded yet.

| RRC-5 | whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2 Editor’s NOTE: There is no conclusion on whether it is allowed to report an empty UAI on offset for LP-WUS monitoring for both option 1-1 and option 1-2. | Issue Type: not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

In the TS 38.331 running CR, SetupRelease{} structure is used to define the lpwus-OffsetPreferenceConfig-r19 value range as follows:

OtherConfig-v19xx ::= SEQUENCE { lpwus-OffsetPreferenceConfig-r19 SetupRelease {LPWUS-OffsetPreferenceConfig-r19} OPTIONAL -- Need M }

Thus, the preferred LP-WUS offset can be canceled if release is indicated in SetupRelease{}, i.e. no input in in SetupRelease{}. Thus, empty UAI on LP-WUS offset is not necessary.

Proposal 6(RRC-5): The preferred LP-WUS offset can be canceled if release is indicated in SetupRelease{ } for lpwus-OffsetPreferenceConfig-r19. Empty UAI on LP-WUS offset is not necessary

- Conclusion

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CEl.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

Observation 1: Per the RAN1 input that UE is not able to operate LR and MR simultaneously in Rel-19,   it is not possible to support LP-WUS per DRX group and wake up each DRX group separately as if one DRX group is activated by LP-WUS with MR operating, the UE cannot monitor LP-WUS for the other DRX group at the same time.

Proposal 2(RRC-3): Confirm the following working assumption to support LP-WUS with dual DRX group for both option 1-1 and 1-2, and when LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

Proposal 3(RRC-3): For Option 1-2, RAN2 confirms that one single LP-WUS cycle is defined to determine the LP-WUS monitoring occasion.

Proposal 4(RRC-3): For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 5(RRC-3): For option1-2, when LP-WUS is configured with secondary DRX Group simultaneously, one LP-WUS monitoring configuration (LP-WUS cycle, timeOffset) is configured per cell group, LP-WUS subgroup number and lpwus_PDCCHMonitoringTimer is configured per DRX group.

Proposal 6(RRC-5): The preferred LP-WUS offset can be canceled if release is indicated in SetupRelease{ } for lpwus-OffsetPreferenceConfig-r19. Empty UAI on LP-WUS offset is not necessary

- References

- Draft_RAN2_130_Meeting_Report_v1

- Draft_Minutes_report_RAN1#120b_v010

- 3GPP TS 38.321 V18.4.0 (2024-12),NR; Medium Access Control (MAC) protocol specification