3GPP TSG-RAN WG2 Meeting #131	R2-2506005

Bangalore, India, Aug 25th – 29rd, 2025

Agenda Item		:	8.2.2 (Ambient_IoT_solutions)

Source		:	LG Electronics Inc.

Title		:	Discussion on A-IoT paging

Document for		:	Discussion and Decision

1.	Introduction

In this contribution, we discuss our views on ambient IoT (A-IoT) paging, considering the WID [1], the current progress in RAN2, and the ongoing open issue discussion of [POST130][027][AIoT] MAC Running CR (Huawei).

2. Discussion

2.1 Transaction ID-related issues

During the RAN2#129 meeting [2] and RAN2#130 meeting [3], RAN2 reached the following agreements regarding the transaction ID. There are still several FFS points that we need to discuss.

Agreements – RAN2#129 Simultaneous Paging Procedures - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). Which entity generates the transaction ID - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID Transaction ID Size - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. Multiple Reader Scenario - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. Agreements – RAN2#130 Agreements on parallel service request - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in stage 2 specification. - The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in stage 3 specification. - Send LS to RAN3 to notify them of agreements 1 and 2 - Parallel service request for overlapping reader scenario can be addressed in Rel-20

We discussed how reader generates transaction ID. As agreed in the RAN2#129 meeting, parallel service requests will not be supported by the same reader. Additionally, it was agreed in the RAN2#130 meeting that, in Rel-19, parallel service requests from multiple readers will not come through network implementation. Therefore, at any given time, an A-IoT device may not need to distinguish between different service requests using transaction ID (i.e., there would not be a situation where different service requests are parallelly overlapped in Rel-19). Consequently, at least for Rel-19, it seems unnecessary to specify how the reader generates the transaction ID, and this can be left to the reader’s implementation.

However, we anticipate that in Rel-20 or future releases, a multiple-reader scenario may be considered. Currently, RAN2 agreed in the RAN2#129 meeting to rely on the transaction ID to handle the multi-reader scenario. Considering that the simultaneous service request can be supported in the multi-reader scenario, the transaction ID may be used to differentiate between neighbouring readers.

Additionally, the intermediate UE reader for topology 2 will be specified in Rel-20. Since this intermediate UE reader can be freely moved within A-IOT coverage, differentiation between the legacy reader and the intermediate UE reader may be necessary.

The number of bits required for future releases will depend on future discussion. However, to ensure distinct transaction IDs are assigned between neighbouring readers and to distinguish between the intermediate UE reader and the legacy reader, additional transaction ID bits may be needed. Specifically:

- At least two bits are required to assign unique transaction IDs to neighboring readers.

- At least one additional bit is needed to differentiate the intermediate UE reader from the legacy reader.

In addition, considering that two or three bits are already required to distinguish between a new paging request and subsequent paging from the same reader, we propose that the transaction ID should be allocated at least five or six bits to accommodate these requirements.

Proposal 1: The length of the transaction ID should be at least five bits or six bits considering the future release.

3.	Conclusion

This contribution makes the following proposal:

Proposal 1: The length of the transaction ID should be at least five bits or six bits considering the future release.

4.	References

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- R2-2501702, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece

- Draft Report of 3GPP TSG RAN WG2 meeting #130