3GPP TSG-RAN WG2#131		      					R2-2506010

Bengaluru, India, August 25 – 29, 2025

Agenda item:	8.6.4

Source: 	Kyocera

Title: 	Remaining issues of Conditional intra-CU LTM

Document for:	Discussion

Introduction

The following statements were agreed in RAN2#130 for Conditional intra-CU LTM [1]:

Agreements on C-LTM 1. (MAC-9) If there is at least one beam associated with the configured uplink grant with RSRP above configured RSRP threshold, the UE selects a beam among the beams and CG-based RACH-less CLTM is performed. If the UE finds out multiple beams above threshold, it is up to UE implementation to select which one. If there is no beam associated with the configured uplink grant with RSRP above configured RSRP threshold, RACH-based CLTM is performed. 2. For L3 based CLTM, the UE performs RACH-based CLTM if there is no configured grant. 3. Upon CLTM execution, if TA is valid (i.e. TAT timer is running) at the first available CG occasion, and there is CG resource configured, RACH-less CLTM will be performed. 4. During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM. 5. In case CG resource is only associated with SSB, if the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS. 6. RAN2 will not do anything for the separate TCI mode optimization.

It is necessary to resolve the remaining issue on CLTM, to timely complete the work item. So, this paper discusses the handling of TA values for the coexistence of NW-triggered LTM and CLTM.

Discussion

Open issue MAC-14

According to the list of MAC open issues for mobility enhancements phase 4 [2], there is an FFS for the case when ltm-TimeAlignmentTimer is running (i.e., the TA value of the candidate cell acquired in the early TA procedure is valid) and an (Enhanced) LTM Cell Switch Command MAC CE (LTM CSC MAC CE) is received. The exact issue is how to handle the TA value in this case.

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |

According to TS38.321 [3] below, the procedure is divided into the two cases as follows:

Case 1: LTM Cell Switch Command MAC CE with a TA value; and,

Case 2: LTM Cell Switch Command MAC CE without a TA value (i.e., the Timing Advance Command field is set to ‘FFF’).

5.18.35 LTM Cell Switch Command The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; […] 3> else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the indicated LTM target configuration: 4> process the measured Timing Advance (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing. […]

Case 1: LTM Cell Switch Command MAC CE with a TA value

In Rel-18, if the UE receives an LTM CSC MAC CE with a TA value, the UE applies this TA value and considers the RACH-less LTM cell switch to be ongoing. This means that regardless of whether the UE has successfully measured a TA value by UE-based TA measurement, the UE always applies the TA value received in the LTM CSC MAC CE.

In Rel-18, the UE prioritizes the TA value provided in an LTM CSC MAC CE over the measured TA value (via UE-based TA measurement).

In Rel-19 CLTM, the UE acquires and stores a TA value in advance through the early TA procedure, which is separated from the cell switch execution (i.e., via LTM Candidate Timing Advance Command MAC CE). The validity of this TA value is managed by ltm-TimeAlignmentTimer. The issue is which TA value the UE should apply for RACH-less LTM, if it has the valid stored TA value when it receives the (enhanced) LTM CSC MAC CE without a TA value.

When the UE receives an (Enhanced) LTM CSC MAC CE with a TA value, it should apply the TA value received in the (Enhanced) LTM CSC MAC CE, even if it has the valid stored TA value (i.e., ltm-TimeAlignmentTimer is running). This is because the TA value included in the (Enhanced) LTM CSC MAC CE is the most recent (thus, considered as the most accurate) TA value since it’s provided at the timing of the cell switch execution.

If the UE receives an (Enhanced) LTM CSC MAC CE with a TA value, the UE should always apply the TA value provided in the (Enhanced) LTM CSC MAC CE, even if a stored TA value (via Early TA) is valid.

According to RAN2#129-bis agreements [4], ltm-TimeAlignmentTimer for the target cell is not stopped upon CLTM cell switch execution to the target cell.

C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

To support subsequent CLTM, ltm-TimeAlignmentTimer for the target cell should also not be stopped upon LTM cell switch execution triggered by an (Enhanced) LTM CSC MAC CE. Therefore, if the UE receives an (Enhanced) LTM CSC MAC CE with a TA value, the UE should store this TA value for the target cell and start or restart ltm-TimeAlignmentTimer of the target cell.

If the UE receives an (Enhanced) LTM CSC MAC CE with a TA value, the UE should store this TA value for the target cell and start or restart ltm-TimeAlignmentTimer.

Case 2: LTM Cell Switch Command MAC CE without a TA value (i.e., the TAC field is set to ‘FFF’)

In Rel-18, if the UE receives an LTM CSC MAC CE without a TA value and has successfully measured the TA value, the UE applies the measured TA value and considers the RACH-less LTM cell switch to be ongoing. This means that the UE is allowed to execute the RACH-less LTM cell switch applying the measured TA value (i.e., the TA value measured by UE-based TA measurement).

In Rel-18, if the UE receives an LTM CSC MAC CE without a TA value, the UE is allowed to execute the RACH-less LTM cell switch applying the measured TA value (via UE-based TA measurement).

For Rel-19, the issue is whether the UE should apply the valid stored TA value for RACH-less LTM, when it receives the (enhanced) LTM CSC MAC CE without a TA value.

Considering Observation 2 in Rel-18 behaviour, for Rel-19 behaviour, it can also be executed without any problem by applying the stored TA value acquired through the early TA procedure, as long as ltm-TimeAlignmentTimer is running.

If the UE receives an (enhanced) LTM CSC MAC CE without a TA value, the UE applies the stored TA value for the target cell, as long as the corresponding ltm-TimeAlignmentTimer is running and executes the RACH-less LTM cell switch to the target cell.

According to the RAN2#129-bis agreements [4], when the CLTM execution is triggered, if a TA value for the target cell acquired by early TA procedure is valid (i.e., ltm-TimeAlignmentTimer of the target cell is running), the UE applies this TA value, otherwise if the UE has successfully measured a TA value of the target cell, the UE applies the measured TA value by UE-based TA measurement. This means that the UE prioritizes the TA value with a running ltm-TimeAlignmentTimer acquired by the early TA procedure over the measured TA value (i.e., UE-based TA measurement).

Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

In Rel-19 CLTM, the UE prioritizes the stored TA value with a running ltm-TimeAlignmentTimer over the TA value measured by UE (i.e., the stored TA value > UE-based TA measurement).

RAN2 should clarify that the UE prioritizes TA application upon (C)LTM execution in order as follows:

- - The TA value provided in the LTM CSC MAC CE

- - The stored TA value with a running ltm-TimeAlignmentTimer

- - The measured TA value (i.e., UE-based TA measurement)

Conclusion

In this contribution, the issues for supporting conditional intra-CU LTM are discussed. RAN2 is kindly asked to take into account the observations and proposals below:

Observation 1	In Rel-18, the UE prioritizes the TA value provided in an LTM CSC MAC CE over the measured TA value (via UE-based TA measurement).

Proposal 1	If the UE receives an (Enhanced) LTM CSC MAC CE with a TA value, the UE should always apply the TA value provided in the (Enhanced) LTM CSC MAC CE, even if a stored TA value (via Early TA) is valid.

Proposal 2	If the UE receives an (Enhanced) LTM CSC MAC CE with a TA value, the UE should store this TA value for the target cell and start or restart ltm-TimeAlignmentTimer.

Observation 2	In Rel-18, if the UE receives an LTM CSC MAC CE without a TA value, the UE is allowed to execute the RACH-less LTM cell switch applying the measured TA value (via UE-based TA measurement).

Proposal 3	If the UE receives an (enhanced) LTM CSC MAC CE without a TA value, the UE applies the stored TA value for the target cell, as long as the corresponding ltm-TimeAlignmentTimer is running and executes the RACH-less LTM cell switch to the target cell.

Observation 3	In Rel-19 CLTM, the UE prioritizes the stored TA value with a running ltm-TimeAlignmentTimer over the TA value measured by UE (i.e., the stored TA value > UE-based TA measurement).

Proposal 4	RAN2 should clarify that the UE prioritizes TA application upon (C)LTM execution in order as follows:

- The TA value provided in the LTM CSC MAC CE

- The stored TA value with a running ltm-TimeAlignmentTimer

- The measured TA value (i.e., UE-based TA measurement)

References

- R2-2504671, “Report from session on R18 SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#130

- R2-2505398, “Discussion summary and list of MAC open issue for Mob Ph4”, vivo, RAN2#131

- TS38.321, “Medium Access Control (MAC) protocol specification”, 3GPP

- R2-2502981, “Report from session on R18 SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#129bis