3GPP TSG-RAN WG2 Meeting #131		R2-2506016

Bengaluru, India, 25th – 29th Aug. 2025

Source:	vivo

Title:	Discussion on SA2 reply LS on paging capability loss issue

Agenda Item:	8.19.2

Document for:	Discussion

- Introduction

In this contribution, we discuss SA2 reply LS on paging capability loss issue in [1].

- Discussion

In SA2#166 meeting, SA2 sent an LS to RAN2 and RAN3 in [2] about how UE to report the UE capability on LP-WUS supporting in case the initial Registration is performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS).

Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

In this case, the legacy gNB does not understand the LP-WUS capability and thus cannot include this capability in the UERadioPagingInformation inter-node message to CN or other gNBs for paging, i.e. the LP-WUS capability reported by the UE is not maintained by the network, which results in inconsistent paging operations between the UE and the network and causes paging issues.

RAN2 has discussed this issue firstly in RAN2#129 meeting with the following agreement and sent an LS to SA2[3]:

Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

Furthermore, RAN2 further discussed this issue in RAN2#129bis meeting and reached the following agreements with a new reply LS to SA2 [4]:

Agreements - Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for a gNB that supports this Rel-19 “container” - We will not change legacy UE capabilities and rely on network implementation - Send LS to SA2

Based on the above, it could be observed that RAN2 common understanding is that:

- For the new Rel-19 and later gNB, it could support the “container” for UE Radio Paging Capabilities for paging naturally, and there is no such paging missing issue when UE reports the UE capability during the initial registration or TAU procedure on such gNB.

- For the legacy gNB, since it couldn’t understand Rel-19 “container” for paging capabilities, nothing won’t be introduced and we leave it to network implementation if UE just camps on legacy gNB when it reports UE capability to AMF.

However, according to the latest reply LS from SA2[1], it seems that SA2 misunderstood our intention that:

In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

In the reply LS from SA2, SA2 mentioned that:

The 5GS design means that the UE Radio Capability for Paging is loaded into the AMF only in the cell where the UE attaches (does “initial registration”) or first moves from LTE to NR. After that the UE can move throughout the whole PLMN and normally there is no event that triggers any update of the UE Radio Capability for Paging information stored in the AMF. Hence the “certain area” mentioned by RAN 2 is the whole NR coverage of that PLMN. It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution.

Observation 1: SA2 may misunderstand RAN2 intention that: if Rel-19 LP-WUS feature is deployed, all the gNBs belonging to a certain area are required to support the Rel-19 “container”, which SA2 considers to be either infeasible or impractical in commercial networks.

Actually, based on the discussion in RAN2, this “container” is newly introduced from Rel-19. All the gNBs before Rel-19 cannot support this “container”, which means that there may be paging capability miss issue.

- RAN2 common understanding is that it relies on network implementation to handle it (e.g. upgraded to support the ASN.1 from Rel-19) or not to address it.

Regarding the network from Rel-19, if operators want to support the related features in one area, e.g. LP-WUS, new version of gNBs from Rel-19 in this area should support this “container”; while for old version of gNB before Rel-19 (including Rel-19 gNB not supporting Rel-19 “container”) in this area:

- If they are not upgraded to support Rel-19 “container”, there may be paging capability miss issue in this area only when the UE camps on this kind of gNB during the initial registration or TAU procedure and reports capability via this gNB.

- If they are upgraded to support Rel-19 “container” (by implementation, only need to be upgraded to support the ASN.1 from Rel-19), there will be no paging capability miss issue.

With this, whether to upgrade all gNBs in the area when supporting the Rel-19 related feature, e.g. LP-WUS, is totally up to operators’ deployment.

Regarding the solution investigated in SA2:

SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF.

We understand it is not feasible for legacy gNB (not supporting Rel-19 “container”) to use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features, which still needs the gNB to be upgraded to understand the Rel-19 ASN.1.

Proposal 1: Reply LS to SA2 to clarify the RAN2 discussion situation:

- The “container” is newly introduced from Rel-19. All the gNB before Rel-19 cannot support this “container”, which means that there may be paging capability miss issue.

  - RAN2 common understanding is to rely on network implementation to handle it (e.g. upgraded to support the ASN.1 from Rel-19) or not to address it.

- Regarding the network from Rel-19, if operators want to support the related features in one area, e.g. LP-WUS, new version of gNBs from Rel-19 in this area should support this “container”; while for old version of gNB before Rel-19 (including Rel-19 gNB not supporting Rel-19 “container”) in this area:

  - If they are not upgraded to support Rel-19 “container”, there may be paging capability miss issue in this area only when the UE camps on this kind of gNB during the initial registration or TAU procedure and reports capability via this gNB.

  - If they are upgraded to support Rel-19 “container” (by implementation, only need to be upgraded to support the ASN.1 from Rel-19), there will be no paging capability miss issue.

A draft reply LS is provided in Annex.

Proposal 2: RAN2 to consider the draft reply LS in Annex.

- Conclusions and Proposals

In this contribution, we discussed the SA2 reply LS on paging capability loss issue. Based on the discussion, we have the following observations and proposals.

Observation 1: SA2 may misunderstand RAN2 intention that: if Rel-19 LP-WUS feature is deployed, all the gNBs belonging to a certain area are required to support the Rel-19 “container”, which SA2 considers to be either infeasible or impractical in commercial networks.

Proposal 1: Reply LS to SA2 to clarify the RAN2 discussion situation:

- The “container” is newly introduced from Rel-19. All the gNBs before Rel-19 cannot support this “container”, which means that there may be paging capability miss issue.

  - RAN2 common understanding is to rely on network implementation to handle it (e.g. upgraded to support the ASN.1 from Rel-19) or not to address it.

- Regarding the network from Rel-19, if operators want to support the related features in one area, e.g. LP-WUS, new version of gNBs from Rel-19 in this area should support this “container”; while for old version of gNB before Rel-19 (including Rel-19 gNB not supporting Rel-19 “container”) in this area:

  - if they are not upgraded to support Rel-19 “container”, there may be paging capability miss issue in this area only when the UE camps on this kind of gNB during the initial registration or TAU procedure and reports capability via this gNB.

  - if they are upgraded to support Rel-19 “container” (by implementation, only need to be upgraded to support the ASN.1 from Rel-19), there will no paging capability miss issue.

Proposal 2: RAN2 to consider the draft reply LS in Annex.

- References

R2-2505063, Response to Reply LS on paging capability loss issue, SA2/Vodafone

S2-2412876, 	LS Reply on LP-WUS subgrouping

R2-2501388, Discussion on SA2 LS on LP-WUS subgrouping

R2-2503172, Reply LS on paging capability loss issue

Annex – [draft] Reply LS on paging capability loss issue

Title:	[Draft] Reply LS on paging capability loss issue

Response to:	R2-2505063/ S2-2506087

Release:	Rel-19

Work Item:	NR_LPWUS-Core, TEI

Source:	vivo [to be RAN2]

To:	SA2

Contact Person:

Name:	              Chenli

E-mail Address:	 Chenli5g@vivo.com

1. Overall Description:

RAN2 would like to thank SA2 for the response to Reply LS on paging capability loss issue in [R2-2505063/ S2-2506087].

RAN2 wants to further clarify our intention on the method of UE radio paging capability loss mentioned in the previous LS [S2-2412876]: [To be updated based on the online discussion]

- The “container” is newly introduced from Rel-19. All the gNBs before Rel-19 cannot support this “container”, which means that there may be paging capability miss issue if UE reports the UE paging capability during the initial registration or TAU procedure via such gNB.

  - RAN2 common understanding is to rely on network implementation to handle it (e.g. upgraded to support the ASN.1 from Rel-19) or not to address it.

- Regarding the network from Rel-19, if operators want to support the related features in one area, e.g. LP-WUS, new version of gNBs from Rel-19 in this area should support this “container”; while for old version of gNB before Rel-19 (including Rel-19 gNB not supporting Rel-19 “container”) in this area:

  - If they are not upgraded to support Rel-19 “container”, there may be paging capability miss issue in this area only when the UE camps on this kind of gNB during the initial registration or TAU procedure and reports capability via this gNB.

  - If they are upgraded to support Rel-19 “container” (by implementation, only need to be upgraded to support the ASN.1 from Rel-19), there will be no paging capability miss issue.

Regarding the solution investigated in SA2:

SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF.

RAN2 understand: it is not feasible for legacy gNB (not supporting Rel-19 “container”) to use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features, which still needs the gNB to be upgraded to understand the Rel-19 ASN.1.

2. Actions:

To RAN SA2:

RAN2 kindly request SA2 to take the above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #131bis      13th Oct. – 17th Oct. 2025	Prague, Czech Republic

TSG RAN WG2 Meeting #132           17th Nov. – 21st Nov. 2025	Dallas，USA