3GPP TSG-RAN-WG2 Meeting #131	R2-2506018

Bangaluru, India  Aug 25th – 29th, 2025

Agenda item	:		8.8.2

Source	: 	Sharp

Title	: 	Discussion on DL coverage enhancement

Document for	:	Discussion and Decision

Introduction

In the RAN2 #130 meeting, RAN2 made some agreements/conclusions as follows [1].

Agreements:

1.	the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

2.	To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

4.	No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

Working Assumption:

1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

In this paper, we further discuss remaining issues for SMTC configuration.

Discussion

SSB-index based SMTC Selection

For idle/inactive UE, it was agreed to introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbour cells, and it is FFS whether SSB-index based SMTC selection is supported.

We do not support the introduction of SSB-index based SMTC selection. A location-based SMTC selection procedure was already agreed in the previous meeting and provides a robust framework. Introducing an additional mechanism may lead to unnecessary complexity and signalling overhead. Moreover, the effectiveness of SSB-index based selection is dependent on the number of SSB indices used in the cell. If the number of SSB index is not enough to consider the optimal SMTC configuration, NW needs to use the location-based approach. Therefore, we think it is not needed to support SSB-index based approach.

Proposal.1: For SMTC selection, SSB-index based approach is not introduced.

SMTC Configuration in Connected Mode

For connected UE, it is WA that a mechanism to assist the NW to configure the SMTCs in connected mode is introduced, and there are two following alternatives.

Alt 1: UE provides the closest N reference locations/neighbour cells to the network.

Alt 2: UE reports the selected SMTCs from the configured SMTC set to the network.

For Alt 1, this method allows the network to make more informed decisions based on UE-reported proximity to reference locations, which can lead to more efficient SMTC configuration and improved measurement performance. In addition, Alt 1 provides flexibility for the network to adapt configurations dynamically based on UE location and mobility patterns. So, reference location can be configured independently from neighbour cell location.

Proposal.2: RRC Connected UE provides the closest N reference locations to the network.

Proposal.3: NW configures the parameter N and reference locations to UE by RRC signalling.

Conclusion

In this paper, we made following proposals;

Proposal.1: For SMTC selection, SSB-index based approach is not introduced.

Proposal.2: RRC Connected UE provides the closest N reference locations to the network.

Proposal.3: NW configures the parameter N and reference locations to UE by RRC signalling.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #130