3GPP TSG-RAN-WG2 Meeting #131	R2-2506019

Bengaluru, India, 25th– 29th Aug, 2025

Agenda item	:		8.13.2

Source	: 	Sharp

Title	: 	Remaining issues and solutions on Relay discovery and (re)selection for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#130 meeting, RAN2 reached following agreements [1]:

Agreements: - The notification message is used for the L2 child UE in case of upstream relay (re)selection triggered by AS layer failure cases (Uu/PC5 link failure/release, using legacy cause value) where the upper layer does not trigger link release towards the child. For reselection triggered by upper layers we rely on the upper-layer notification/release mechanism already defined. - Remote UE handling of the notification is up to upper layer implementation. - Further handling of any inter-layer issues on this topic can be done in maintenance phase. - LS to SA2 to notify them of these agreements. - [Chair’s note: The second agreement above was clarified in a later session as indicated below, such that the remote UE handling of “the notification” in the second agreement refers to the upper-layer notification.] - Leaving remote UE handling of the notification to implementation applies to the upper-layer notification for remote UEs in RRC_IDLE/RRC_INACTIVE. UEs in RRC_CONNECTED follow the later agreement to trigger RRC reestablishment. No case is expected where a UE in RRC_CONNECTED receives the upper-layer notification. - Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism. FFS new cause values. - In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed. FFS which parameters are related. This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified. - FFS if notification behaviour can be optimized for intermediate relay UEs in RRC_CONNECTED. - Upon receiving the notification message or upon PC5 link release, remote UE in RRC_CONNECTED needs to initiate RRC Reestablishment as legacy. - When the intermediate Relay UE in RRC_CONNECTED receives the notification message from its parent intermediate Relay UE/last Relay UE or when the PC5 link is released, it initiates RRC Reestablishment.

In this paper, we discuss how to introduce multi-hop relay.

Discussion

  - Discovery related issues

- RRC container in discovery message

RAN2 already agreed that the multi-path topology should not be supported in multi-hop relay. In that case, remote UEs and intermediate relay UEs should use the SIBs broadcasted on the last relay UE’s serving cell. In such a case, to prevent mis-configuration, the last relay UE's child UEs need to know the last relay UE's serving cell. In Rel-17 relay discovery, the RRC container is included in the discovery message, and the cell-related information is included in the RRC container. In multi-hop relay, based on the discovery message received from the last relay UE, the corresponding cell-related information needs to be included in the discovery message to be transmitted.

- UE sets the access information to the UE’s access information in the current spec, but UE may not have valid access information in multi-hop relay system.

- When an intermediate relay UE transmits discovery message, the UE includes the cell access info received in parent UE’s discovery message.

- FFS whether the case that there is no established PC5 link between the last relay UE and the intermediate relay UE before discovery model B transmission can occur and whether the intermediate relay UE always establishes itself as a remote UE first.

If the UE follows legacy behaviour, the UE establishes Uu RRC connection upon receiving RRCSetup message from a child UE. Furthermore, during discovery phase, the remote UE initiating the discovery is selecting the path. Thus, the intermediate relay UE might not be selected. So, RRC setup/resume should be initiated by an intermediate relay UE after the remote UE selects the path, i.e. the case can occur. Therefore, the intermediate relay UE always establishes RRC connection when receives RRC setup message from a child UE if there is no established PC5 link between the last relay UE and the intermediate relay UE.

- (RRC-6) RAN2 confirms that an intermediate relay UE always establishes RRC connection when receives RRC setup message from a child UE if there is no established PC5 link between a last relay UE and the intermediate relay UE.

  - Relay (re)selection related issues

Triggering condition of relay reselection

- FFS whether the conditions for L2 multi-hop relay can be applied L3 multi-hop relay.

We don’t see any issues to apply the condition for L2 multi-hop relay of relay reselection to L3 multi-hop relay. However, it should be made clear that these conditions do not directly trigger relay reselection but are instead a notification to higher layers.

- The agreed trigger conditions for relay reselection can be applied to L3 multi-hop relay but should only be for an indication to higher layers.

Notification message related FFS

RAN2 needs to discuss the following remaining issues.

- FFS the content in the notification (copy or regeneration)

If a remote UE in RRC_IDLE/INACTIVE receives notification indicating RC5-RLF from its parent UE, the remote UE should consider cell/relay reselection occurs. Then, the remote UE finds new relay UE and establishes new RRC connection via the new relay UE. However, in case of the parent UE-initiated relay reselection, the remote UE could keep the PC5 connection to its parent UE. Thus, a notification message from the parent UE should indicate appropriate indication type. If the content of the notification message is a copy of original notification message, e.g., indication type is PC5-RLF, the remote UE performs relay reselection. So, when a child UE receives a notification message from its parent UE and performs relay reselection, the child UE should regenerate a notification message indicating relay reselection occurs.

- (RRC-5) When a child UE receives a notification message from its parent UE and performs relay reselection, the child UE should regenerate a notification message indicating relay reselection occurs.

- FFS which parameters are AS visible parameters that could affect relay reselection

RAN2 made following agreement in RAN2#130 meeting:

In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed.  FFS which parameters are related. This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified.

Here, we consider which the AS-visible parameters are. For cell ID (cell access information), it is forwarded by RRC container in discovery message. Thus, cell ID is AS-visible parameter. For hop count, it is upper layer parameter. However, RAN2 agreed that RRC Timers are multiplied by hop count in RAN2#130 meeting. Thus, hop count should be AS-visible parameter.

- (RRC-11) When the hop-count or cell ID (cell access information) change as a result of relay reselection, an intermediate relay UE in RRC_IDLE/INACTIVE sends a notification message to its child UE(s).

RRC Resume/Setup triggered by a remote UE while relay reselection performed by an intermediate relay UE

In RAN2#130, RAN2 made the following agreements:

- When an intermediate relay UE is in RRC_CONNECTED, it should notify the failure or release PC5 link

- When an intermediate relay UE is NOT in RRC_CONNECTED and AS-visible parameters are not changed, not AS notification is sent.

Then, while an intermediate relay UE is not in RRC_CONNECTED:

- in a case that an upper layer of the intermediate relay UE performs relay reselection, the upper layer sends upper layer notification (notifying path change) to its child UE(s) after the relay reselection.

- in a case that an upper layer of the intermediate relay UE doesn’t perform relay reselection, the upper layer sends upper layer notification (forcing its child UE(s) to initiate relay reselection) to its child UE(s) immediately.

Here, in the former case, when the child UE initiates RRC Setup Request or RRC Resume Request and the AS-visible parameters changes due to relay reselection performed by the intermediate relay UE, Remote needs to wait until the extended Timer (T300 or T319) expires and initiate relay reselection procedure based on the notification message from is parent UE. This is because RRC processing is performed sequentially.

- Remote UE cannot initiate relay reselection triggered by a notification message until the extended timer (T300 or T319) which is multiplied by hop count expires.

The extended timer increases the waiting time of the remote UE, which means that the remote UE cannot quickly return to the RRC connected state when, for example, the UE receives paging message, or its own data is generated. Therefore, Remote UE should consider T300/T319 as expires and initiates relay reselection immediately if it receives a notification message from its parent UE.

- Remote UE should consider T300/T319 as expires and initiates relay reselection immediately if it receives a notification message from its parent UE.

- FFS other conditions or timing to transmit the notification message

Considering the above cases, to deliver appropriate information to child UE at the right time, an intermediate relay UE should transmit the notification message in the following cases while performing relay reselection:

- When an intermediate relay UE receives paging message for its child UE

- When an intermediate relay UE receives SRB0 (e.g. RRCSetupRequest, RRCResumeRequest) message from its child UE

These conditions ensure that the notification message is transmitted in response to key events that directly impact the communication between the intermediate relay UE and its child UE. By aligning the timing of message delivery with these triggers, the remote (child) UE can establish RRC connection at the right time.

- RAN2 considers the following conditions as additional timing to transmit the notification messages.

  - When an intermediate relay UE receives paging message for its child UE

  - When an intermediate relay UE receives SRB0 (e.g. RRCSetupRequest, RRCResumeRequest) message from its child UE

Conclusion

In this paper, we provided following observations and proposals:

- UE sets the access information to the UE’s access information in the current spec, but UE may not have valid access information in multi-hop relay system.

- When an intermediate relay UE transmits discovery message, the UE includes the cell access info received in parent UE’s discovery message.

- (RRC-6) RAN2 confirms that an intermediate relay UE always establishes RRC connection when receives RRC setup message from a child UE if there is no established PC5 link between a last relay UE and the intermediate relay UE.

- The agreed trigger conditions for relay reselection can be applied to L3 multi-hop relay but should only be for an indication to higher layers.

- (RRC-5) When a child UE receives a notification message from its parent UE and performs relay reselection, the child UE should regenerate a notification message indicating relay reselection occurs.

- (RRC-11) When the hop-count or cell ID (cell access information) change as a result of relay reselection, an intermediate relay UE in RRC_IDLE/INACTIVE sends a notification message to its child UE(s).

- Remote UE cannot initiate relay reselection triggered by a notification message until the extended timer (T300 or T319) which is multiplied by hop count expires.

- Remote UE should consider T300/T319 as expires and initiates relay reselection immediately if it receives a notification message from its parent UE.

- RAN2 considers the following conditions as additional timing to transmit the notification messages.

  - When an intermediate relay UE receives paging message for its child UE

  - When an intermediate relay UE receives SRB0 (e.g. RRCSetupRequest, RRCResumeRequest) message from its child UE

Reference

- R2-2502984, Report from session on positioning and sidelink relay