3GPP TSG-RAN-WG2 Meeting #131	R2-2506021

Bengaluru, India, 25th– 29th Aug, 2025

Agenda item	:		8.13.4

Source	: 	Sharp

Title	: 	Remaining issues and solutions on service continuity for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#130 meeting, RAN2 reached the agreement as below [1];

- In scenarios A/B, the Intermediate Relay UE could be configured to perform the measurement on adjacent upstream relay link, Uu link and candidate relay link. - In scenarios A/B, the measurement report from Intermediate relay UE will also include at least serving intermediate/last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result. - Upon Intermediate Relay UE (including first relay UE) receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release) or sends NotificationMessageSidelink message to the downstream intermediate Relay/Remote UE. The choice is implementation-based. - Measurement event X2 could be applied to scenario B with multi-hop indirect to single-hop indirect path switching. - reconfigurationWithSync without sl-PathSwitchConfig can be reused to trigger Case A (Intra-gNB multi-hop indirect to direct path switching). - reconfigurationWithSync with sl-PathSwitchConfig can be reused to trigger Case B (Intra-gNB multi-hop indirect to single-hop indirect path switching). - RAN2 will not address cases of service continuity towards a relay path with intermediate relays in idle/inactive under the current WID.

In this paper, we discuss some potential issues and solutions for it.

Discussion

  - Path switching mechanisms

Scenario A and B are below;

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

- Whether the UE not in RRC_CONNECTED is selected as a target relay UE (multi-hop to single-hop specific issue)

In scenario B, i.e. intra-gNB multi-hop indirect to single-hop indirect path switching, the target relay UE is always potential last relay UE. The role of last relay UE is very similar with Rel-17 U2N relay UE. And RAN2 already supported the i2i path switching procedure with service continuity in Rel-18. In Rel-18, the UE not in RRC_CONNECTED is reported to the gNB and can be selected as a target U2N relay UE. Therefore, there shouldn’t be any issues to support the case the UE not in RRC_CONNECTED is selected as a target relay UE in intra-gNB multi-hop indirect to single-hop indirect path switching scenario (scenario B).

- In scenario B, RAN2 confirms that the UE not in RRC_CONNECTED can be selected as a target relay UE without any efforts in RAN2.

- Whether/when the each PC5 link on source path should be released

On the source path, after remote UE switches from the source path to the target path, PC5 link(s) and related configuration for the remote UE are not needed. In Rel-17/18, RAN2 already discussed service continuity issue. Based on the discussion,

- For DL, based on remote UE’s PDCP status report, NW should guarantee lossless path switching.

- For UL, remote UE only performs PDCP recovery-based re-transmission, and remaining packet at the source relay UE should be sent to the (source) gNB. After the gNB receives all remaining packets, the gNB will configure the source relay UE to release a configuration for the remote UE.

These principles can be applied to the scenario A and B. In other words, from the DL point of view, the source path can be released if remote UE switches, and from the UL point of view, at least the connection between the relay and remote can be released if remote UE switches. The exact timing can be up to the remote UE.

- The PC5 connection between the remote UE and the first relay UE can be released after the remote UE switches from the multi-hop indirect path to direct/single-hop indirect path. The exact timing is up to the remote UE.

For the configurations for the remote UE in the first relay UE and the last relay UE on the source path and the PC5 connection between the first relay UE and the last relay UE, for lossless path switching for UL data, it should be up to gNB whether/when to be released. Of course, if the last relay UE is acting as a remote UE, its PC5 connection may not be released (if same RSC is used for single-hop/multi-hop relaying).

- For lossless path switching for UL data, it is up to gNB whether/when the configurations for the remote UE in the first relay UE and the last relay UE on the source path and the PC5 connection between the first relay UE and the last relay UE are released.

- Path switching with a child UE

RAN2 discusses to support relay reselection performed by an intermediate relay UE while keeping PC5-link to a child UE, according to the conclusion in SA. We assume that this mechanism is introduced for reducing wasteful relay reselection performed by relay UEs (to reduce signalling, communication outage time). By applying the same mechanism to reconfiguration with sync, the number of cases where the same benefits can be enjoyed with less effort will increase. However, to support service continuity for a child UE, more workloads are expected. So, RAN2 should consider if the mechanism that relay reselection performed by an intermediate relay UE while keeping PC5-link to a child UE can be applied to the reconfiguration with sync, without supporting service continuity.

- RAN2 considers if the mechanism that relay reselection performed by an intermediate relay UE while keeping PC5-link to a child UE can be applied to the reconfiguration with sync, without supporting service continuity.

  - . Contents of candidate relay UE(s) report

In Rel-18 U2N relay, (candidate) remote UE sends measurement report to send candidate relay UEs information to the gNB. The measurement report includes following contents;

SL-MeasResultListRelay-r17 ::= SEQUENCE (SIZE (1..maxNrofRelayMeas-r17)) OF SL-MeasResultRelay-r17

SL-MeasResultRelay-r17 ::= SEQUENCE {

cellIdentity-r17			CellAccessRelatedInfo,

sl-RelayUE-Identity-r17	SL-SourceIdentity-r17,

sl-MeasResult-r17	SL-MeasResult-r16,

...,

[[

sl-MeasQuantity-r18	ENUMERATED { sl-rsrp, sd-rsrp }	OPTIONAL,

sl-RelayIndicationMP-r18	SL-RelayIndicationMP-r18		OPTIONAL

]]

}

For the cell access information, the gNB needs to find whether the candidate first relay UE reaches the same gNB. In legacy, remote UE reports the serving cell information of the candidate relay UE to the gNB. And the remote UE considers the serving cell of the relay UE as PCell. Same as the legacy, in multi-hop system, remote UE should report the serving cell information of last relay UE connecting with the candidate first relay UE.

- gNB can decide whether the reported first relay UE can reach the gNB since first relay UE contains cellAccessRelatedInfo of serving cell of last relay UE in RRC container included in discovery message.

- The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

RAN2 already agreed that cell info, source ID and measurement result are included in the report. But there is no agreement on information indicating whether the measurement result is SL-RSRP or SD-RSRP. From our view, the legacy IE can be reused.

- Information indicating whether the measurement result is SL-RSRP or SD-RSRP can be included in the measurement report to the gNB as with legacy.

We also consider what information is needed for gNB to select the appropriate PC5 links. We assume the AS layer evaluation metrics that the gNB uses to select optimal PC5 links are PC5 link quality of each PC5 link, number of hops, and QoS profile for the (potential) remote UE. This allows the gNB to select a path with low latency and/or robustness (i.e., less likely to be disconnected) depending on the QoS and radio resource management of the remote UE. According to the current SA2 discussion, the discovery message may include hop counter, ID of last relay UE and accumulated QoS for the PC5 links (i.e. Achievable PDB) in addition to the legacy contents. Therefore, the remote UE knows these metrics. And, the gNB knows that the reported first relay UE is connecting with the same gNB or not, the number of hops of the PC5 links to which the reported candidate first relay UE belongs, and the QoS profile for the (potential) remote UE. So, for selecting appropriate path (first relay UE), UE should send the information of PC5 link quality of the path the first relay UE belongs. Furthermore, as the number of hops increases, the quality of each hop affects the delay and the efficiency of radio resource utilization. So, we consider that the UE needs to report to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

- In order to select appropriate path, from the perspective of QoS and radio resource management, gNB needs to know PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE.

- gNB already knows number of hops, and QoS profile for the (potential) remote UE.

- The greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization.

- Potential remote UE reports to the gNB the PC5 link qualities of each hop of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Conclusion

In this paper, we made following observations and proposals:

- In scenario B, RAN2 confirms that the UE not in RRC_CONNECTED can be selected as a target relay UE without any effort in RAN2.

- The PC5 connection between the remote UE and the first relay UE can be released after the remote UE switches from the multi-hop indirect path to direct/single-hop indirect path. The exact timing is up to the remote UE.

- For lossless path switching for UL data, it is up to gNB whether/when the configurations for the remote UE in the first relay UE and the last relay UE on the source path and the PC5 connection between the first relay UE and the last relay UE are released.

- RAN2 considers if the mechanism that relay reselection performed by an intermediate relay UE while keeping PC5-link to a child UE can be applied to the reconfiguration with sync, without supporting service continuity.

- gNB can decide whether the reported first relay UE can reach the gNB since first relay UE contains cellAccessRelatedInfo of serving cell of last relay UE in RRC container included in discovery message.

- The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

- Information indicating whether the measurement result is SL-RSRP or SD-RSRP can be included in the measurement report to the gNB as with legacy.

- In order to select appropriate path, from the perspective of QoS and radio resource management, gNB needs to know PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE.

- gNB already knows number of hops, and QoS profile for the (potential) remote UE.

- The greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization.

- Potential remote UE reports to the gNB the PC5 link qualities of each hop of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Reference

- R2-2502984, Report from session on positioning and sidelink relay