3GPP TSG-RAN-WG2 Meeting #131	R2-2506022

Bengaluru, India, 25th– 29th August, 2025

Agenda item	:		8.6.2

Source	: 	Sharp

Title	: 	Remaining issues and solutions on inter-CU LTM

Document for	:	Discussion and Decision

Introduction

WID [1] for mobility enhancement was approved and RAN2 started discussion on the WI. In RAN2#130, RAN2 made agreements as follows [2];

Agreements on inter-CU LTM:

- (RRC-2) The appliedLTM-CandidateId is included in both MN and SN RRCReconfigurationComplete message (Rel18 specification should be okay).

- (RRC-6) In inter-MN LTM failure (LTM cell switch failure) the UE fall back to the previous source PCell configuration before selecting a cell (same as legacy procedure).

- (RRC-7) The legacy sk-counter is used for inter-MN LTM (this means sk-counter value can be re-used).

- RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

- If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

- RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE.

- RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

- If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

- RAN2 not support CSI-RS based CFRA resources indicated via LTM cell switch command MAC CE.

- The Enhanced LTM CSC MAC CE always contains an NCC value field. The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

In this paper, we discuss inter-CU LTM related issues.

Discussion

Inter-CU LTM configuration

In the CR discussion [2], it is clarified that an ltm-Config for SCG and an ltm-ConfigNRDC cannot be configured simultaneously. Then, RAN2 needs to discuss how to treat the ltm-Config for SCG when the ltm-ConfigNRDC is configured and how to treat the ltm-ConfigNRDC when the ltm-Config for SCG is configured. According to the current RRC CR and some agreements, inter-CU SCG LTM candidate configurations are provided by MCG and a LTM CSC MAC CE indicating inter-SN LTM cell switch is provided by SCG. If SN provides intra-CU SCG LTM candidate configuration via RRC signalling for the UE, UE holds ltm-Config for SCG and UE variables associated with ltm-Config for SCG. Thus, UE needs to release or replace the configuration and UE variables. When UE doesn’t release the LTM configuration (ltm-Config for SCG or ltm-ConfigNRDC), LTM-related misconfigurations can occur, e.g., non-negotiated R18 ID settings are mixed in. Here, two implementation methods to avoid such a situation can be considered. The first one is that NW (SCG) sends RRC signalling to the UE to release the configurations and variables, and the second one is that UE autonomously releases the configurations and variables. It seems wasteful if UE releases it by explicit signalling from the SCG from the perspective of radio resource efficiency. That is, the UE needs to autonomously release the contents of ltm-Config for SCG.

- It is wasteful if UE releases ltm-Config for SCG by explicit signalling from SN before ltm-ConfigNRDC is configured by MN, from the perspective of radio resource efficiency.

- UE autonomously releases the contents of old ltm-Config for SCG/ltm-ConfigNRDC when the ltm-ConfigNRDC/ltm-Config for SCG is configured.

RAN2 agreed that UE will re-establish an PDCP entity if Rel-19 Set ID of target cell is different from Rel-19 Set ID of serving cell and if the termination point of the DRB is changed. From this, KeyToUse might be changed during LTM cell switch procedure in Rel-19. However, in Rel-18, there is no termination point change during LTM cell switch. This principle should be observed even if Rel-19 Set ID of target cell is same with Rel-19 Set ID of serving cell. This is because changing KeyToUse even in the case of intra-CU not only eliminates the benefits of Rel-18LTM (short interruption time due to less L2 processing), but also has an impact on the specifications.

- If Rel-19 Set ID of target cell is same with Rel-19 Set ID of serving cell, KeyToUse will not be changed in any DRB.

Inter-CU SCG LTM execution

Here, the procedure introduced in the CR discussion [2] is not aligned with the agreement:

Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

⦁	UE performs PDCP re-establishment to all radio bearers.

Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

⦁	For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

⦁	For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

⦁	If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

However, the handling of the SRB remains an open topic that has not yet been fully discussed. As of now, the following agreements regarding SRB have been formed in RAN2:

- For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

That is, the cases listed below remain undiscussed:

  - For inter-CU MCG LTM, how to handle PDCP entity of SRB3.

  - For inter-CU SCG LTM, how to handle PDCP entity of SRB1/2.

Regarding the first point, similar to SRB1/2, when the Rel-19 ID of the candidate cell is different from that of the serving cell, the UE should perform PDCP re-establishment. This is because SRB3 uses the S-KgNB (K_RRCint/K_RRCenc) derived from K_gNB, and without PDCP re-establishment, proper key updates will not be performed. On the other hand, when the Rel-19 ID of the candidate cell is the same as that of the serving cell, the UE does not need to perform PDCP SDU discard. This is because SRB3 only has an SCG RLC leg, and the SCG RLC entity is not affected by intra-CU MCG LTM, similar to the behavior in Rel-18.

- For inter-CU MCG LTM, the UE performs PDCP re-establishment for SRB3.

Regarding the second point, similar to DRB, the UE doesn’t need to perform PDCP re-establishment for split SRB1/2 since the uplink data can be transferred via MCG leg. Also, data recovery is not needed for split SRB1/2 since the data recovery is only applied for AM DRB. However, for the RLC entities associated with split SRB1/2, the UE is required to perform RLC re-establishment, as these entities may be affected by the configuration change due to the LTM cell switch. Naturally, non-split SRB1/2 are not impacted by the SCG key change, and therefore, no action is needed.

- For inter-CU SCG LTM, the UE performs RLC re-establishment for split SRB1/2.

- If above Proposal 3/4 agreed, RAN2 considers TP1 as baseline for CR drafting.

Co-existence issues

LTM and L3 HO

In RAN2#127, it was agreed that L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE while inter-CU LTM is being configured (w/o DC). It was also agreed that when performing L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations. That is, the UE simultaneously keeps the RRC configuration of L3 mobility (HO and CHO) and inter-CU LTM. Depending on the triggering timing, the UE performs either L3 mobility or inter-CU LTM. However, it is the MAC entity of the UE that receives the LTM CSC MAC CE from the source DU to trigger the inter-CU LTM, while the RRC layer of the UE receives the L3 HO command from the source CU to trigger L3 mobility. Based on Rel-18 intra-CU LTM, when the intra-CU LTM is triggered, the MAC entity of the UE subsequently notifies the RRC layer of the UE. Therefore, we propose that when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE further notifies the RRC layer of the UE, by following the rationale in Rel-18. Afterwards, RAN2 further discuss the cases when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and the execution priority of L3 HO and inter-CU LTM.

- By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

- RAN2 further discuss:

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution priority of L3 HO and inter-CU LTM.

Conclusion

In this paper, we made the following observations and proposals:

- It is wasteful if UE releases ltm-Config for SCG by explicit signalling from SN before ltm-ConfigNRDC is configured by MN, from the perspective of radio resource efficiency.

- UE autonomously releases the contents of old ltm-Config for SCG/ltm-ConfigNRDC when the ltm-ConfigNRDC/ltm-Config for SCG is configured.

- According to the current CR, when the Rel-19 Set ID of candidate cell is different from serving cell, all PDCP/RLC entities are re-established. This procedure doesn’t align with the agreements.

- If Rel-19 ID of target cell is same with Rel-19 ID of serving cell, KeyToUse will not be changed in any DRB.

- For inter-CU MCG LTM, the UE performs PDCP re-establishment for SRB3.

- For inter-CU SCG LTM, the UE performs RLC re-establishment for split SRB1/2.

- If above Proposal 3/4 agreed, RAN2 considers TP1 as baseline for CR drafting.

- By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

- RAN2 further discuss:

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution priority of L3 HO and inter-CU LTM.

Reference

- R2-250xxxx, Report from session on V2X/SL, R19 NES and MOB

- R2-250xxxx, [POST129bis][115][MOB] RRC inter-CU and CLTM (Ericsson)

Annex. Text Proposal 1 for TS 38.331-5.3.5.18.6

- 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

<omit>

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

<omit>

2>	at the end of the procedure, for each srb-Identity value that is part of the current UE configuration:

3>	if the LTM cell switch is triggered on the SCG:

4>	if this SRB is SRB3:

5>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KgNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

5>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

4>	if the srb-Identity of this SRB is included in any RLC-BearerConfig in the CellGroupConfig associated with the SCG (i.e. split SRB1, split SRB2 and SRB3):

5>	re-establish the corresponding RLC entity for the SCG as specified in TS 38.322 [4];

3>	else:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KgNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

4>	re-establish the corresponding RLC entity as specified in TS 38.322 [4];